EPON OLT Products User Manual

FD1204S/FD1208S/FD1216S/FD8000-L116

---Command Line Operation

Version：V1.2

Company Address: 6th floor, Building 4 of South Area 2, Honghualing Industrial Park, Xili Avenue, Nanshan District, Shenzhen, China

Factory Address: 1st floor, Building B, Wentao Industrial Park, Yingrenshi Community, Shiyan Avenue, Baoan District, Shenzhen, China

Telephone: 0755-26014509/26014710/26014711

Fax: 0755-26014506

Email: Marketing@cdatatec.com

Website: www.cdatatec.com

www.cdatatec.com.cn

Company Address: 6th floor, Building 4 of South Area 2, Honghualing Industrial Park, Xili Avenue, Nanshan District, Shenzhen, China

Factory Address: 1st floor, Building B, Wentao Industrial Park, Yingrenshi Community, Shiyan Avenue, Baoan District, Shenzhen, China

Telephone: 0755-26014509/26014710/26014711

Fax: 0755-26014506

Email: Marketing@cdatatec.com

Website: www.cdatatec.com

www.cdatatec.com.cn

©Statement

Copyright ©2017 Shenzhen C-Data Technology Co., Ltd. All Rights Reserved. No part of this manual may be reproduced or transmitted in any form or by any means without the prior written consent.

The information in this document may be update at any time due to productversion upgrade or any other reasons. Unless otherwise agreed, this manual

is for reference only. All statements, information and recommendations in

this manual do not constitute the warranty of any kind, express or implied. Please contact your suppliers to get the latest version of this document.

About This Manual

This manual is applicable to C-Data FD1204、FD1208S、FD1216S、FD8000-L116 EPON OLT products cli command operation, Is the user through cli command config EPON OLT equipment should read the information before guidelines.

The related documents for EPON OLT device are:
《FD1204S/FD1208S/FD1216S/FD8000-L116 User Manual-Device Install Guide》

《FD1204S/FD1208S/FD1216S/FD8000-L116 User Manual-Quick Configuration Guide》

《FD1204S/FD1208S/FD1216S/FD8000-L116 User Manual-EMS Configuration Guide》

目录

1. Read Instructions	1

Document Scope	1

Revision History	1

Conventions for Command Line	1

Conventions for Keyboard Operation	2

Conventions for symbols	2

Conventions for Words	2

Precautions	2

2. Command Line Interface View Introduce	3

2.1. Command Line View Overview	4

2.1.1. Enter Enable View	5

2.1.2. Enter Config View	5

2.1.3. Enter Interface View	5

2.1.4. Enter ACL View	6

2.1.5. Enter Multicast-vlan View	6

2.1.6. Exit Arbitrary View	7

3. OLT Device Upgrade Management	7

3.1. Upgrade OLT software	7

3.2. Check OLT Software and Hardware Version	8

3.3. Show the progress of current load/copy/backup in olt	8

4. OLT Device Management	9

4.1. OLT Reboot	9

4.2. Config OLT Outband Manager IP	9

4.3. Config Vlanif Interface IP Address(Inband)	10

4.4. Show MGMT IP Address（Inband）	11

4.5. Show Vlanif IP Address（Outband）	11

4.6. Show OLT Detail Information	12

4.7. Config OLT User Logout Time	13

4.8. Show OLT User Logout Time	13

4.9. Logout System	13

4.10. Exit OLT Current View Mode	14

4.11. Config OLT DNS server IP Address	14

4.12. Delete OLT DNS Server IP Address	14

4.13. Show OLT DNS Server IP Address	15

4.14. Config OLT Hostname	15

5. OLT Status Monitor	15

5.1. Show OLT Fan Working Status	15

5.2. Show OLT Working Temperature	16

5.3. Show OLT Memory Usage	16

5.4. Config OLT System Time	17

5.5. Show OLT System Time	17

5.6. Show OLT Boot Time and Running Time	17

5.7. Config OLT NTP Time and Server	18

5.8. Delete NTP Time and Server	18

5.9. Show NTP Server Session Information	18

5.10. Config OLT Systme Timezone	19

5.11. Show OLT System Current Timezone	19

5.12. Show OLT Local MAC Address Information	20

5.13. Show OLT CPU Usage	20

5.14. Show OLT History Input Command	21

6. OLT Configuration Manage	21

6.1. Backup OLT Configuration	21

6.2. Download OLT Configuration	22

6.3. Show OLT Current Configuration	23

6.4. Save OLT Configuration	23

6.5. Erase OLT Configuration	24

6.6. Show OLT Saved-config	24

7. OLT Login User Manage	25

7.1. Add OLT New Login Username and Password	25

7.2. Delete OLT Login User	25

7.3. Change OLT Login User Password	26

7.4. Show OLT Exist Username	26

7.5. Show OLT Login User Information	27

7.6. Kill OLT Online User	27

8. OLT SNMP Config	27

8.1. SNMP Enable and Disable	27

8.2. Show OLT Snmp Agent Status	28

8.3. Config OLT SNMP Community	28

8.4. Show OLT SNMP Read Community	29

8.5. Config OLT SNMP Write Community	29

8.6. Show SNMP Write Community	29

8.7. Config OLT SNMP Community Group with Authentication	30

8.8. Config OLT SNMP Community Group with Unauth	30

8.9. Config OLT SNMP Community Group with Privacy	31

8.10. Config OLT SNMP Agent Description	32

8.11. Configure SNMP Agent Location Information	32

8.12. Configure SNMP Agent Contact Infomation	32

8.13. Configure Snmp Agent System Name	33

8.14. Show SNMP Agent System Info	33

8.15. Config SNMP Trap IP	34

8.16. Show SNMP Trap IP Infor	34

8.17. Config SNMP Access User&Auth Mode &Password	35

8.18. Show SNMP Access User	35

9. OLT Uplink Port Configuration	36

9.1. OLT Uplink Basic Function Configuration	36

9.1.1. Disable Uplink Port	36

9.1.2. Enable Uplink Port	37

9.1.3. Config Uplink Port Name	37

9.1.4. Delete Uplink Port Name	38

9.1.5. Config Uplink Electric Port Auto-negotiation	38

9.1.6. Config Uplink Electric Port Duplex Mode	39

9.1.7. Config Uplink Electric Port Speed	39

9.1.8. Config Uplink Port MTU	40

9.1.9. Delete Uplink Port MTU	40

9.1.10. Config Uplink Port Flow-control Function	40

9.1.11. Config Uplink Port MAC Address Learning Function	41

9.1.12. Show OLT Uplink Port Property and Statuss	41

9.1.13. Show OLT Uplink Port Optical Power Information	43

9.2. Uplink Port Mirror Manage	43

9.2.1. Config Uplink Port Mirror Function	43

9.2.2. Delete Uplink Port Mirror Function	44

9.2.3. Show Uplink Port Mirror Configuration	44

9.3. Uplink Port Performance Statistics Function	45

9.3.1. Config Uplink Port Performance Statistics Threshold	45

9.3.2. Show Uplink Port Performance Statistics Threshold	46

9.3.3. Clear Uplink Port Performance Statistics Infor	46

9.3.4. Config Uplink Port Performance Statistics Period	47

9.3.5. Config Uplink Port Performance Statistics Period 24H	47

9.3.6. Show Uplink Port Current 15min Performance Statistics	48

9.3.7. Show Uplink Port Current 24Hs Performance Statistics	49

9.3.8. Show Uplink Port History 15min Performance Statistics	50

9.3.9. Show Uplink Port History 24H Performance Statistics	51

9.4. Uplink Port Storm Control Function	52

9.4.1. Config Uplink Port Broadcast Storm Control Function	52

9.4.2. Config Uplink Port Unknown Multicast Storm Control Function	52

9.4.3. Config Uplink Port Unknown Unicast Storm Control	53

9.5. Uplink Port Rate Limit Function	53

9.5.1. Config Uplink Port Upstream and Downstream Rate Limit	53

9.5.2. Delete Uplink Port Upstream and Downstream Rate Limit	54

9.5.3. Show Uplink Port Upstream and Downstream Rate Limit Configuration	54

9.6. Uplink Port Isolate Function	55

9.6.1. Config Uplink Port Isolate	55

9.6.2. Show Uplink Port Isolation Configuration Infor	55

9.7. Uplink Port RSTP Function	56

9.7.1. Config Uplink Port RSTP Cost	56

9.7.2. Config Uplink Port RSTP Edged-port	56

9.7.3. Config Uplink Port RSTP Mcheck Property	57

9.7.4. Config Uplink Port RSTP Point-to-Point Link Function	57

9.7.5. Config Uplink Port RSTP Priority	58

9.7.6. Show Uplink Port RSTP Configuration	58

9.8. OLT Uplink Port VLAN Config	59

9.8.1. Config Uplink Port VLAN Mode	59

9.8.2. Config Uplink Port Native-vlan	60

9.8.3. Config Uplink Port Native-vlan Priority	60

9.8.4. Config Uplink Port Access Mode VLAN	60

9.8.5. Config Uplink Port Hybrid Mode VLAN	61

9.8.6. Delete Uplink Port Hybrid Mode VLAN	62

9.8.7. Config Uplink Port Trunk Mode VLAN	62

9.8.8. Delete Uplink Port Trunk Mode VLAN	62

9.8.9. Config Uplink Port Translate Mode VLAN	63

9.8.10. Delete Uplink Port Translate Mode VLAN	63

9.8.11. Config Uplink Port Protocol VLAN	64

9.8.12. Show OLT Uplink Port VLAN Configuration	64

10. OLT PON Port Configuration	65

10.1. OLT PON Port Property Config	65

10.1.1. Disable PON Port	65

10.1.2. Enable PON Port	65

10.1.3. Config PON Port Name	66

10.1.4. Delete PON Port Name	66

10.1.5. Config PON Port MTU	67

10.1.6. Delete PON Port MTU	67

10.1.7. Config PON Port Encrypt Mode and Key-exchange-time	67

10.1.8. Config PON Port Flow-control Function	68

10.1.9. Config Pon Port MAC Address Learning Function	68

10.1.10. Show OLT PON Port Property and Status	69

10.1.11. Show OLT PON Port Optical Power Information	70

10.2. Config OLT PON Port Detect ONU Long Laser Function	71

10.2.1. Config Auto Detect ONU Long Laser Function	71

10.2.2. Config Manual Detect ONU Long Laser Function	71

10.2.3. Show Detect ONU Long Laser Configuration	72

10.3. Config OLT PON Port Mirror Function	72

10.3.1. Config Pon Port Mirror Function	72

10.3.2. Delete PON　Port Mirror Function	73

10.3.3. Show PON Port Mirror Configuration	74

10.4. OLT PON Port Performance Statistics Function	74

10.4.1. Config PON Port Performance Statistics Period 15min	74

10.4.2. Config PON Port Performance Statistics Period 15H	74

10.4.3. Config PON Port Performance Statistics Threshold	75

10.4.4. Clear PON Port Performance Statistics Infor	76

10.4.5. Show PON Port Current 15min Performance Statistics	77

10.4.6. Show PON Port Current 24Hs Performance Statistics	78

10.4.7. Show PON Port History 15min Performance Statistics	79

10.4.8. Show PON Port History 24H Performance Statistics	80

10.4.9. Show PON Port Performance Statistics Threshold Configuration	81

10.5. OLT PON Port Storm Control Function	81

10.5.1. Config PON Port Broadcast Storm Control Function	81

10.5.2. Config PON Port Unknown Multicast Storm Control Function	82

10.5.3. Config PON Port Unknown Unicast Storm Control	82

10.6. OLT PON Port Rate Limit Function	83

10.6.1. Config PON Port Egress and Ingress Rate Limit	83

10.6.2. Delete PON Port Egress and Ingress Rate Limit	83

10.6.3. Show PON Port Egress and Ingress Rate Limit Configuration	84

10.7. OLT PON Port Isolate Function	84

10.7.1. Config PON Port Isolate	84

10.7.2. Config PON Port Ont-isolate Function	85

10.7.3. Show PON Port Isolation Configuration Info	85

10.8. OLT PON Port VLAN Config	86

10.8.1. Config PON Port VLAN Mode	86

10.8.2. Config PON Port Native-vlan	86

10.8.3. Config PON Port Native-vlan Priority	87

10.8.4. Config PON Port Access Mode VLAN	87

10.8.5. Config PON Port Hybrid Mode VLAN	88

10.8.6. Delete PON Port Hybrid Mode VLAN	88

10.8.7. Config PON Port Trunk Mode VLAN	89

10.8.8. Delete PON Port Trunk Mode VLAN	89

10.8.9. Config PON Port Translate Mode VLAN	90

10.8.10. Delete PON Port Translate Mode VLAN	90

10.8.11. Config PON Port QinQ VLAN	91

10.8.12. Delete PON Port QinQ VLAN	91

10.8.13. Config PON Port Aggregation VLAN	92

10.8.14. Config PON Port Protocol VLAN	92

10.8.15. Show OLT PON Port VLAN Configuration	93

10.8.16. Show PON Port Translate VLAN Configuration	93

10.8.17. Show PON Port Aggregation VLAN Configuration	94

11. OLT MAC Address Table Manage	94

11.1. Config OLT MAC-address Black-hole	94

11.2. Delete MAC-Address Black-hole	95

11.3. Config OLT Mac Address Entries limit	95

11.4. Add Static MAC Address Bind Function	96

11.5. Delete Static MAC Address bind	97

11.6. Config OLT MAC Address Aging Time	98

11.7. Clear OLT MAC Address Table	98

11.8. Clear OLT Port MAC Address Table	99

11.9. According OLT VLAN Clear MAC Address Table	99

11.10. Show OLT MAC Address Table	100

11.11. Show OLT MAC Address Black Hole	100

11.12. Show OLT Dynamic MAC Address Table	101

11.13. Show MAC Address Table From ONU	101

11.14. Show MAC Address Table From PON Port	102

11.15. Show MAC Address Table From GE Port	103

11.16. Show MAC Address Table From XGE Port	104

11.17. Show MAC Address Table From Aggregation Group	104

11.18. Show OLT Static MAC Address Tables	104

11.19. Show OLT MAC Address Aging Time Configuration	105

11.20. Show MAC Address Table From Specified Vlan	105

12. OLT Global VLAN Configurationss	106

12.1. OLT VLAN Basic Configuration	106

12.1.1. Create OLT VLAN or VLAN List	106

12.1.2. Detele OLT VLAN or VLAN List	106

12.1.3. Config OLT VLAN or VLAN List Name	107

12.1.4. Delete VLAN or VLAN List Name	107

12.1.5. Show OLT VLAN Configuration	108

12.1.6. Show OLT VLAN Translate Configuraiton	109

12.2. Vlanif Configuration	109

12.2.1. Create or Delete Vlanif Interface	109

12.2.2. Create or Delete Vlanif IP Address	110

12.2.3. Config Vlanif Interface Description	110

12.2.4. Show Vlanif Interface Detail Information	111

12.3. VLAN Policy Configuration	112

12.3.1. Add VLAN Policy Based On Mac address	112

12.3.2. Show MAC-VLAN Entry	112

12.3.3. Add VLAN Policy Based On IP Address	113

12.3.4. Show IP-Subnet-VLAN Entry	113

12.3.5. Add VLAN Policy Based On Protocol	114

12.3.6. Show Protocol-vlan Entry	115

13. OLT IGMP Configuration	115

13.1. Config IGMP Fast-leave Function	115

13.2. Config IGMP Mode	116

13.3. Config IGMP Proxy Parameter	117

13.4. Config IGMP Forwarding Policy	119

13.5. Show IGMP Configuration	119

13.6. Show IGMP Forwarding Table	120

13.7. Config Multicast VLAN	121

13.8. Config IGMP Match Group	121

13.9. Show IGMP Match Group	121

13.10. Add IGMP Program	122

13.11. Add Batch IGMP Program	122

13.12. Delete IGMP Program	123

13.13. Config IGMP Router-port	123

13.14. Show IGMP Router-port	124

13.15. Configure Unknow-multicast Forwarding Policy	124

13.16. Show Unknow-multicast Forwarding Policy	124

13.17. Add IGMP User（Member） in Multicast-vlan	125

13.18. Show Multicast-vlan Member Configuration	125

13.19. Config IGMP User Bind Rights Profile	126

14. STP Configuration	126

14.1. Global STP Config	126

14.2. Show STP Configuraiton	126

14.3. Config STP Priority	128

14.4. Config STP Bridge Forward-delay Time	128

14.5. Config STP Bridge Hello Message Send Period Time	129

14.6. Config STP Bridge Max-age Time	129

14.7. Config STP BPDU TX Hold Count	130

14.8. Config OLT Port STP Cost	130

14.9. Config OLT Port STP Edged-port	130

14.10. Config OLT Port STP Mcheck Property	131

14.11. Config OLT Port STP  Point-to-Poing Function	131

14.12. Config OLT Port STP Priority	132

14.13. Show OLT Port STP Configuration	133

14.14. Show OLT Link-aggregation Group STP Configuration	133

15. ACL Configuration Manage	134

15.1. ACL Apply Time-range Cofiguration	134

15.1.1. Config ACL Apply Relative Time-range	134

15.1.2. Config ACL Apply Absolute Time-range	135

15.1.3. Delete ACL Apply Time-range	135

15.1.4. Show ACL Apply Time-range Configuration	135

15.2. Config OLT Basic ACL Function	136

15.3. Config OLT Advanced ACL Function	137

15.4. Config OLT Link ACL Function	139

15.5. Config OLT PON ACL Function	140

15.6. Config ONU ACL Function	141

15.7. Show OLT ACL Configuration	142

15.8. Modify OLT ACL Rule ID	143

16. OLT QOS Configuraiton	144

16.1. Config OLT Global QOS mode	144

16.2. Config Traffic Control Based On ACL Rule	144

16.2.1. Config Packet Filter Based On ACL Rulesss	144

16.2.2. Changes Traffic DSCP Based On ACL Rule	145

16.2.3. Config Traffic Mirror Based On ACL Rule	146

16.2.4. Config Traffic Limit Based On ACL Rule	146

16.2.5. Add Traffic Outer VLAN Based On ACL Rule	147

16.2.6. Translate Traffic VLAN Based On ACL Rule	148

16.2.7. Modify Traffic VLAN Priority Based On ACL Rule	148

16.2.8. Config Traffic Redirect Based On ACL Rule	149

16.2.9. Modify Traffic TOS Value Based On ACL Rule	149

16.2.10. Config ACL Action and Bind ACL to PON Port	150

16.2.11. Config ACL Action and Bind ACL to ONU	151

16.2.12. Show ACL Bind Configuration	152

16.2.13. Show Port Packet-filter Policy Configuration	153

16.2.14. Show Port QOS Configuration Information	153

16.3. Config OLT QOS Queue	154

16.3.1. Config OLT QOS Queue Mapping Mode	154

16.3.2. Config QOS Queue Mapping Mode Based On 802.1p	154

16.3.3. Config OLT QOS Queue Scheduled Mode	155

16.3.4. Show OLT QOS Queue Schedule Mode	157

16.4. OLT Port QOS Queue Configuration	158

16.4.1. Config OLT Port QOS Queue Mapping Mode	158

16.4.2. Config OLT Port QOS Queue Mapping Mode Based On 802.1p	158

16.4.3. Config OLT Port QOS Queue Schedule Mode	159

16.4.4. Show OLT Port QOS Queue Schedule Mode	161

17. OLT DHCP Function Configuration	162

17.1. OLT DHCP-Snooping Function Configuration	162

17.1.1. Enable or Disable DHCP-Snooping Function	162

17.1.2. Show DHCP-Snooping Configuration	162

17.1.3. Config DHCP-Snooping Monitor VLAN	164

17.1.4. Config DHCP-Snooping Trust Port	164

17.1.5. Enable or Disable OLT DHCP-Snooping Source MAC Address Detect	165

17.1.6. Config DHCP-Snooping Request Message Rate Limit	165

17.1.7. Enable or Disable DHCP-Snooping Opton82 Function	166

17.1.8. Config DHCP-Snooping Option82 Forwarding Policy	166

17.1.9. Config DHCP-Snooping option82 Format	167

17.1.10. Config DHCP-Snooping Port Binding Policy	167

17.1.11. Delete DHCP-Snooping Bind-table	168

17.1.12. Config DHCP-Snooping Bind-table Write-delay Time	168

17.1.13. Config DHCP-Snooping Bind-table Delete-time	169

17.1.14. Config DHCP-Snooping Bind-table Write-to-flash	169

17.1.15. Save DHCP-Snooping Bind-table to Server	170

17.1.16. Show DHCP-Snooping Bind-table Entry	170

17.1.17. Enable or Disable DHCP-Snooping arp-reply-fast	171

17.1.18. Enable or Disable DHCP-Snooping arp-detect Function	171

17.2. OLT DHCP-Relay Function Configuration	171

17.2.1. Enable or Disable DHCP-Relay Function	172

17.2.2. Config DHCP-Relay Vlanif Interface and Server	172

17.2.3. Show DHCP-Relay Configuration	172

17.3. OLT DHCP-Client Function Configuration	173

17.3.1. Enable or Disable DHCP-Client Function	173

17.3.2. Config DHCP-Client Manual Renew IP Address	173

17.3.3. Config DHCP-Client Manual Release IP Address	173

17.3.4. Config DHCP-Client Option60	174

17.3.5. Show DHCP-Client Configuration	174

17.3.6. Show DHCP-Client Option60 Configuration	175

18. OLT Link Aggregation Function Configuration	175

18.1. OLT Link-Aggregation Group Basic Function Config	175

18.1.1. Add or Delete Link-Aggregation Group Member	175

18.1.2. Enable or Disable Link-Aggregation Group Flow-control Function	176

18.1.3. Config Link-Aggregation Group System Priority	176

18.1.4. Set Link-Aggregation Group Port Priority	177

18.1.5. Show Link-Aggregation Group System Priority	177

18.1.6. Show Link-Aggregation Group Priority	177

18.1.7. Config Link-Aggregation Group MTU	178

18.1.8. Config Link-Aggregation Group Unicast Load Balance Mode	178

18.1.9. Config Link-Aggregation Group Non-unicast Load BalanceMode	179

18.1.10. Config Link-Aggregation Group Name	179

18.1.11. Clear Link-Aggregation Group Statistics	180

18.1.12. Enable or Disable Link-Aggregation Group	180

18.2. Link-Aggregation Group STP Configuration	181

18.2.1. Config Link-Aggregation Group STP Cost	181

18.2.2. Enable or Disable Link-Aggregation Group STP Edged-port	181

18.2.3. Config Link-Aggregation Group STP Mcheck Property	182

18.2.4. Config Link-Aggregation Group STP Point-to-Point Function	182

18.2.5. Config Link-Aggregation Group STP Priority	183

18.3. Link-Aggregation Group VLAN Configuration	183

18.3.1. Config Link-Aggregation Group VLAN Mode	183

18.3.2. Config Link-Aggregation Group Native-vlan(access)	184

18.3.3. Config Link-Aggregation Group Native-vlan Priority	184

18.3.4. Config Link-Aggregation Group Access Mode VLAN	185

18.3.5. Config Link-Aggregation Group Hybrid Mode VLAN	185

18.3.6. Delete Link-Aggregation Group Hybrid Mode VLAN	186

18.3.7. Config Link-Aggregation Group Trunk Mode VLAN	187

18.3.8. Delete Link-Aggregation Group Trunk Mode VLAN	187

18.4. Show Link-Aggregation Group Information	187

18.4.1. Show Link-Aggregation Group VLAN Information	188

18.4.2. Show Link-Aggregation Group STP Information	188

18.4.3. Show Link-Aggregation Group Port Status	189

18.4.4. Show Link-Aggregation Group STP Information Statistics Information	189

18.4.5. Show Link-Aggregation Group Config Information	190

19. OLT Routing Function	191

19.1. Add or Delete Static Route	191

19.2. Show IP Route Information	191

19.3. Show IP Routing-table Information	192

20. OLT Profile Configuration	192

20.1. OLT DBA Profile Configurations	192

20.1.1. Create DBA Profile	192

20.1.2. Config DBA Profile Bandwith	193

20.1.3. Commit DBA Profile Config	195

20.1.4. Show OLT DBA Profile Information	195

20.2. ont-lineprofile Configuraton	196

20.2.1. Create ont-lineprofile	196

20.2.2. Bind DBA Profile in ont-lineprofile	196

20.2.3. Config FEC Function in ont-lineprofile	196

20.2.4. Config Data Encryption in ont-lineprofile	197

20.2.5. Config ONU-Car in ont-lineprofile	197

20.2.6. Commit ont-lineprofile Configuration	197

20.2.7. Show OLT ont-lineprofile Information	198

20.3. ont-srvprofile Configuration	199

20.3.1. Create ont-srvprofile	199

20.3.2. Config ONU Capability in ont-srvprofile	199

20.3.3. Config ONU Port Native-vlan in ont-lineprofile	200

20.3.4. Config ONU Port VLAN in ont-srvprofile	200

20.3.5. Config ONU VLAN Mode in ont-lineprofile	201

20.3.6. Config ONU Port Multicast VLAN in ont-lineprofile	201

20.3.7. Config ONU Port Downstream Rate limit in ont-lineprofile	202

20.3.8. Config ONU Port Upstream Rate limit in ont-lineprofile	202

20.3.9. Config ONU Port MAC Count Limit in ont-lineprofile	202

20.3.10. Config ONU Port IGMP Number in ont-lineprofile	203

20.3.11. Config ONU Port IGMP VLAN Process Mode in ont-lineprofile	203

20.3.12. Config ONU IGMP Fast-leave Function in ont-lineprofile	204

20.3.13. Enable or Disable ONU Loop Detect Function in ont-lineprofile	204

20.3.14. Show ont-srvprofile Current Configuration	205

20.3.15. Commit ont-srvprofile Configuration	206

20.4. OLT traffic-profile Configuration	206

20.4.1. Create traffic-profile	206

20.4.2. Modify traffic-profile	207

20.4.3. Show OLT traffic-profile Information	208

20.5. OLT VLAN Pool Configuration	208

20.5.1. Create VLAN Pool	208

20.5.2. Modify VLAN Pool	209

20.5.3. Delete VLAN Pool	209

20.5.4. Show OLT VLAN Pool Information	209

21. ONU Management	210

21.1. ONU Authentication Management	210

21.1.1. Enable or Disable ONU Autofind Function	210

21.1.2. Show Autofind ONU	210

21.1.3. MAC Address Method Authenticated ONU And Bind Profile	211

21.1.4. Loid Method Authenticated ONU and Bind Profile	212

21.1.5. Loid+Password Method Authenticated ONU and Bind Profile	213

21.1.6. Config ONU Authmode	213

21.1.7. Change ONU Bind Profile	214

21.1.8. Config Pre-binding ONU Profile	215

21.1.9. Confirm Autofind ONU	215

21.1.10. Delete Authentication ONU	216

21.1.11. Cancel Autofind ONU	217

21.1.12. Re-register ONU	217

21.1.13. Modify ONU Authmode	218

21.1.14. Delete Blacklist Auth ONU	219

21.1.15. Show ONU BlackList Authenticated Config	219

21.1.16. Show ONU Registered Status and Information	220

21.2. Policy-auth ONU and Batch Delivery Configuration Manage	221

21.2.1. Enable or Disable Global Policy-auth ONU	221

21.2.2. Enable or Disable Policy-auth ONU Under PON Port	221

21.2.3. Config Policy-auth ONU Mode	222

21.2.4. Config Policy-auth ONU Mode and Batch Delivery Configuration Match Mode	222

21.2.5. Show Policy-auth ONT Configuration Information	224

21.3. ONU Basic Function Management	224

21.3.1. Active ONU	224

21.3.2. Deactive ONU	225

21.3.3. Reboot ONU	225

21.3.4. ONU Remote Manage IP Function Config	226

21.3.5. ONU Upstream Rate Limit Function Config	226

21.3.6. ONU Downstream Rate Limit Function Config	227

21.3.7. ONU MAC Aging Time Config	227

21.3.8. Add ONU Description Information	228

21.3.9. ONU Encryption Function Config	228

21.3.10. Enable or Disable ONU FEC Function	229

21.3.11. ONU Service-sla Function Config	229

21.3.12. ONU SNMP Function Config	230

21.3.13. Enable or Disable ONU Tx Optical Power Supply	231

21.3.14. Show ONU ipconfig(Remote Manage IP Address)	231

21.3.15. Show ONU SNMP Configuration	232

21.3.16. Show ONU Capability Information	232

21.3.17. Show ONU Configured Capability	233

21.3.18. Show ONU Opitcal Power Information	234

21.3.19. Show ONU Firmware Version	235

21.4. ONU Port VLAN Configuration(Non-template Mode)	235

21.4.1. ONU Port Native-vlan(access) Configs	236

21.4.2. ONU Port Vlan Config	236

21.4.3. ONU Port Translation Mode VLAN Config	237

21.4.4. ONU Port Transparent Mode VLAN Config	237

21.4.5. Show ONU Port VLAN Configuration	238

21.5. ONU Port Management	238

21.5.1. Enable or Disable ONU Port	239

21.5.2. ONU Port Isolate Function Config	239

21.5.3. ONU PON Port Performance Statistics Config	240

21.5.4. ONU ETH Port Performance Statistics Config	240

21.5.5. ONU CATV Port Enable or Disable	241

21.5.6. ONU Port Auto-negotiation Function Config	241

21.5.7. ONU Port Downstream Rate Limit Config	242

21.5.8. ONU Port Upstream Rate Limit Config	242

21.5.9. Enable or Disable ONU Port Flow Control Function	243

21.5.10. Clear ONU Port Learned MAC	244

21.5.11. ONU Port MAC Count Limit Function Config	244

21.5.12. ONU Port Multicast Group Number Limit Config	245

21.5.13. ONU Port Multicast VLAN Process Mode Config	245

21.5.14. ONU Port Speed and Duplex Mode Config	246

21.5.15. ONU Port Storm-ctrl Function Config	246

21.5.16. Enable or Disable ONU Port Loop Detect Function	247

21.5.17. Enable or Disable ONU Port Loop Detected	248

21.5.18. ONU Port Upstream classification/mapping/priority-mark Rule Config	248

21.5.19. Show ONU Port Performance Statistics	249

21.5.20. Show ONU Port Attribute(Configuration)	250

21.5.21. Show ONU CATV Port Status	251

21.5.22. Show ONU Port Learned MAC Address Information	251

21.5.23. Show ONU PON Port Performance Statistics	252

21.6. ONU IGMP Function Configuration(Non-template Mode)	252

21.6.1. ONU IGMP Mode Config	253

21.6.2. ONU Fsast-leave Function Config	253

21.6.3. Show ONU Multicast-group Record	253

21.6.4. Show ONU IGMP Mode	254

21.6.5. Show ONU Port IGMP VLAN Tagstrip Mode	254

21.7. ONU Upgrade Management	255

21.7.1. Transfer ONU Firmware to OLT	255

21.7.2. Batch Upgrade ONU	256

21.7.3. Start or Stop Batch Upgrade ONU	256

21.7.4. Show ONU Upgrade Configuration Information	257

21.7.5. Show ONU Upgrade Progress	258

21.7.6. Separately Upgrade ONU Under PON Port	258

21.7.7. Active and Commit ONU New Firmware	259

21.8. ONU Alarm Management	259

21.8.1. ONU Opitcal Bias Current Alarm Function Config	259

21.8.2. ONU Optical Voltage Alarm Function Config	260

21.8.3. ONU Opitcal Power Alarm Function Config	261

21.8.4. ONU Opitcal Temperature Alarm Function Config	262

21.8.5. ONU Port CRC Checkout Alarm Function Config	263

21.8.6. ONU Port Dropping Message Alarm Function Config	264

21.8.7. ONU Port Dropevents Alarm Function Config	265

21.8.8. ONU Port Error Message Alarm Function Config	266

21.8.9. ONU Port Message Fragment Alarm Function Config	266

21.8.10. ONU Port Message Jabber(time out) Alarm Function Config	267

21.8.11. ONU Port Oversize Message Alarm Function Config	268

21.8.12. ONU Port Ultra Short Message Alarm Function Config	269

21.8.13. Enable or Disable ONU Dying-gasp Alarm Function	270

21.8.14. Bind Alarm-config to ONU	271

21.8.15. Bind Optical-alarm-config to ONU	271

21.8.16. Show ONU Dying-gasp Alarm Config Status	272

21.8.17. Show ONU Optical power Alarm Config Status	273

21.8.18. Show ONU ETH Port Alarm Config Status	274

21.8.19. Show ONU PON Port Alarm Config Status	275

21.9. ONU Port VLAN Pool Function Config	276

22. OLT Log Management and Query	277

22.1. Add Syslog Record Server	277

22.2. Delete Syslog Record Server	277

22.3. Enable or Disable Syslog Record to Server	278

22.4. Enable or Disable Alarmlog Record to Server	278

22.5. Active Syslog Record server	278

22.6. Deactive Syslog Record Server	279

22.7. Show Syslog Record Server Config Status	280

22.8. Backup OLT Log	280

22.9. Erase OLT Log	280

22.10. 22.10 Show OLT Log	281

23. OLT Alarm Management and Query	281

23.1. Clear Specified Active Alarm Entry	281

23.2. Clear Active Alarm by Specified Alarmlevel	282

23.3. Clear Active Alarm by Specified Board	282

23.4. Clear Active Alarm by Specified GE Port	282

23.5. Clear Active Alarm by Specified PON Port	283

23.6. Clear Active Alarm by Specified XGE Port	283

23.7. Config Alarmlevel for Specified Alarm	284

23.8. Enable and Config Alarm Jitter-interval Time	284

23.9. Disable Alarm Jitter-proof Function	284

23.10. Enable or Disable Specified Alarm Record Output	285

23.11. Enable or Disable Specified Level Alarm Output	285

23.12. Enable or Disable All Alarms Output	286

23.13. Enable or Disable Detail Alarm Output	286

23.14. Show Specified Active Alarm Record	286

23.15. Show Active Alarm Logs by Specified Level	287

23.16. Show Active Alarm Logs by Specified Board	287

23.17. Show Active Alarm Logs by Specified GE Port	288

23.18. Show Active Alarm Logs by Specified PON Port	288

23.19. Show Active Alarm Logs by Specified XGE Port	289

23.20. Show All Active Alarm Log	289

23.21. Show Specified Alarm History Record	290

23.22. Show Alarm History Record by Specified Level	291

23.23. Show Alarm History Record by Specified Board	291

23.24. Show Alarm History Record by Specified GE Port	292

23.25. Show Alarm History Record by Specified PON Port	292

23.26. Show Alarm History Record by Specified XGE Port	293

23.27. Show All Alarm History Record	293

23.28. Show Alarm Jitter-proof Interval Time	294

23.29. Show Alarm Basic Information	294

24. OLT Event Management and Query	296

24.1. Config Event Level	296

24.2. Enable or Disable All Events Output	296

24.3. Enable or Disable Detail Event Output	296

24.4. Enable or Disable Specified Event Output	297

24.5. Enable or Disable Specified Level Event Output	297

24.6. Show All Event History Record	298

24.7. Show Specified Event History Record	298

24.8. Show Specified Level Event History Record	299

24.9. Show Event History Record by Specified Board	299

24.10. Show Event History Record by Specified GE Port	300

24.11. Show Event History Record by Specified PON Port	300

24.12. Show Event History Record by Specified XGE Port	300

24.13. Show Event Basic Information	301

25. Device Diagnostic Management	301

25.1. Ping Diagnostic Test	301

25.2 Traceroute Diagnostic Test	302

Appendix 1	302

Concluding Remarks	303

Read Instructions

- Document Scope

| Reading Object | Product | Products Software Version |  |
| --- | --- | --- | --- |
| C-DATA Employees, FTTX Operation&Maintenance Engineer, C-DATA Customer’s Technical Engineer | Cdata EPON OLT（FD1204S/FD1208S /FD1216S/FD8000-L116） | V1.3.X |  |
| Compiling Department | C-Data Product Management Center Technical Support Department | Document Version | V1.2 |

- Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 2017-12-7 | V1.1 | OLT version switch to V1.2.X, cli command line have been changed.update command line operation fully. | Technical Support Department |
| 2018-7-26 | V1.2 | 1.OLT version switch to V1.3.X, cli command line have been changed,update command line operation fully（include template configuration and routing configuration etc.）. 2.Add FD8000-L116 config instruction | Technical Support Department |

- Conventions for Command Line

| Format | Specification |
| --- | --- |
| Boldface | Key words of command line will be in boldface(unchanged) |
| Italics | Parameters of command line will be in italics(replace with actual value) |
| [] | Parameters in[]is optional |
| (x¦y¦…) | One of parameters in()should be chosen |
| [x¦y¦…] | None or one of parameters in[]should be chosen |
| <x-y> | One number from x to y should be chosen |
| $ | The next line behind$is annotation |

- Conventions for Keyboard Operation

| Format | Specification |
| --- | --- |
| Characters within angle brackets | Represents botton’s name,like<Enter>,<Tab>,<Backspace>,<a>,<?> |
| <button1+button2> | Press button1 and button2 at the same time,like<Ctrl+Alt+A>means pressing button of“Ctrl”,“Alt”,“A”at the same time. |
| <button1,button2> | Press button1 first,release button1,then press button2,like<Alt,F>means pressing“Alt”button first,release“Alt”,then press“F”button” |

- Conventions for symbols

This manual adopts the following highly visible symbols to get users attention when operating,and the explanation of these symbols are as follows:

Watch-out:The matters needs attention in operating,improper operations probably will cause loss of data and damage of device

Warning:Annotation behind this symbol needs special attention,improper operations probably will cause harm to health

Tips:Provide more clear and understandable explanations and descriptions in operating

- Conventions for Words

OLT：Represents the system of FD1204S or FD1216S or FD1208S or F8000-L116,includes main switch processing module and uplink ports connected with uplink devices like switch.

PON：Represents PON protocol processing module and PON ports connected with ONU.

- Precautions

- The command line described in the document is case sensitive in OLT.

- If we meet a command that cannot be inputted or is prompted for error,we can input “?” to see the latter command format.

- Input incomplete commands can be completed by pressing the “Tab” key.

- FD1204S、FD1208S、FD1216S are Pizza-Box OLT，only have one card，so,if we want to enter PON mode,need input interface epon 0/0

- FD8000-L116 is Plug-in card OLT,has four PON card,so the command for entering PON mode is OLT(config)# interface epon 0/<SlotID> ,SlotID is Slot Number,range is 1-4，for example,the command for entering slot 1 is OLT(config)# interface epon 0/1

Command Line Interface View Introduce

View and view switching

Specification:This command line environment includes several views as follows:

- view view：refers to view view or user mode in the below user will enter after inputting password,in which mode only simple commands can be processed.This view shows like:OLT>

- enable view：refers to enable view or privileged mode in the below user will enter after inputting enable in view mode,this mode has higher authorities and shows like:OLT#

- config view：refers to config view or global configuration mode in the below user will enter after inputting config in enable view.This view shows like:OLT(config)#

- ge interface view：refers to ge/gigabit interface view/mode user will enter after inputting interface ge 0/0 in config view.This view shows like:OLT(config-interface-ge-0/0)#

- Xge interface view：refers to xge/10-gigabit interface view/mode user will enter after inputting interface xge 0/0 in config view.This view shows like:OLT(config-interface-xge-0/0)#

- EPON interface view：refers to EPON interface view/mode user will enter after inputting interface EPON 0/0 in config view.This view shows like:OLT(config-interface-epon-0/0)#

- Vlanif three-layer interface view：User will enter this view after inputting interface vlanif vlanID in config view.This view shows like:OLT(config-interface-vlanif-100)#

- Management interface MGMT view：User will enter this view after inputting interface mgmt in config view.This view shows like:OLT(config-interface-mgmt)#

- multicast-vlan view：User will enter this view after inputting multicast-vlan vlanid in config view.This view shows like:OLT(config-multicast-vlan-100)#

- link-aggregation view：User will enter this view after inputtinginterface link-aggregationin config view.This view shows like:OLT(config-interface-aggregation)#

Command Line View Overview

Login

Username:root

Password:admin

Serial Interface

(Bit number: 9600/s

Data bits: 8

Odd-even check: no

Stop bit: 1

Data flow control: no)

PuTTY

(telnet)

view view

OLT>

enable

enable view

OLT#

config

Global configuration mode

OLT(config)#

EPON interface view

OLT(config-interface-epon-0/0)#

EPON port

Ge interface view OLT(config-interface-ge-0/0)#

GE port

Mgmtinterface view OLT(config-interface-mgmt)#

Mgmt interface

Xge interface view

OLT(config-interface-xge-0/0)#

XGE port

Link-aggregation view

multicast-vlan view

OLT(config-multicast-vlan-100)#

multicast-vlan vlanid

vlanID:1-4094

Link-aggregation view

OLT(config-interface-aggregation)#

Vlanif layer3 interface view OLT(config-interface-vlanif-100)#

interface vlanif vlanID

vlanID:1-4094

Acl view

Acl view

OLT(acl-basic-2000)#

btv view

Btv view

OLT(config-btv)#

Enter Enable View

| Command | OLT>enable |
| --- | --- |
| Description | Enter enable view from view mode |

【Example】

Example：Enter enable view from view mode

OLT>enable OLT#

Enter Config View

| Command | OLT#config |
| --- | --- |
| Description | Enter config view from enable mode |

【Example】

Example：Enter enable view from view mode.

OLT#config OLT(config)#

Enter Interface View

| Command | OLT(config)#interface epon <FrameID/SlotID> OLT(config)#interface ge <FrameID/SlotID> OLT(config)#interface link-aggregation OLT(config)#interface mgmt OLT(config)#interface vlanif <vlan id> OLT(config)#interface xge<FrameID/SlotID> |
| --- | --- |
| view | Config view |
| Description | Enter epon/ge/link-aggregation/mgmt/vlanif/xge view from config mode |
| <vlan id> | VLAN ID value,the range is 1-4094 |
| <FrameID/SlotID> | Device frame id/slot id,default as 0/0，FD8000-L116 the Frame default is 0,slot need check insert board to input id,the range is 1-4 |

【Example】

Example 1：Enter vlanif view from config mode

OLT(config)#interface vlanif 100 OLT(config-interface-vlanif-100)#

Example 2：Enter ge view from config mode

OLT(config)#interface ge 0/0 OLT(config-interface-ge-0/0)#exit

Example 3：Enter epon view from config mode

OLT(config)#interface epon 0/0 OLT(config-interface-epon-0/0)#

Enter ACL View

| Command | OLT(config)#acl <acl id> |
| --- | --- |
| view | Config view |
| Description | Enter acl view from config mode |
| <acl id> | <2000-2999>basic acl/ <3000-4999>advanced acl/ <5000-5999>link acl <8000-8199>PON side acl |

【Example】

Example 1：Enter basic acl view from config mode

OLT(config)#acl 2000 ACL ID Create OK! OLT(acl-basic-2000)#

Example 2：Enter advanced acl view from config mode

OLT(config)#acl 3000 ACL ID Create OK! OLT(acl-adv-3000)#

Example 3：Enter pon side acl view from config mode

OLT(config)#acl 8000 OLT(acl-pon-8000)#

Enter Multicast-vlan View

| Command | OLT(config)#multicast-vlan <multicast-vlan id> |
| --- | --- |
| view | Config view |
| Description | Enter multicast-vlan view from config mode |
| <multicast-vlan ID> | Multicast vlan ID,the range is 1-4094. |

【Example】

Example 1：Enter multicast-vlan view from config mode

OLT(config)#multicast-vlan 100 OLT(config-multicast-vlan-100)#

Exit Arbitrary View

| Command | OLT(config)#exit |
| --- | --- |
| Description | Exit current view to previous view. |

【Example】

Example 1：Exit config view to enable view.

OLT(config)#exit OLT#

OLT Device Upgrade Management

Upgrade OLT software

| Command | OLT(config)#load packetfile ftp <ftp-server-ip> <user-name> <user-password> <file-name> |
| --- | --- |
| view | Enable view、config view |
| Description | This command is used to upgrade olt software version and kernel version,it should be use in root account. |
| <ftp-server-ip> | The ip address of ftp server |
| <user-name> | The user name which had set in ftp server |
| <user-password> | The user password which had set in ftp server |
| <file-name> | The name of the OLT software to be downloaded. |

【Example】

Example 1：Upgrade olt software,its file name is FD1216S_FW_V1.0.2_150914_1603.img,the ip address of ftp server is 192.168.1.16,ftp user name is admin,ftp user password is admin.After the olt displays“upgrade OK”,reboot the olt.

OLT(config)#load packetfile ftp 192.168.1.16 admin admin FD1216S_FW_V1.0.2_150 914_1603.img Broadcast message from root: Upgrade is in process. File[FD1216S_FW_V1.0.2_150914_1603.img]download..........OK File[FD1216S_FW_V1.0.2_150914_1603.img]upgrade..........OK

Example 2：Upgrade the kernel of olt,its file name is FD1216S_Kernel_150914_1605.img,the ip address of ftp server is 192.168.1.16,ftp user name is admin,ftp user password is admin.After the olt displays“upgrade OK”,reboot the olt.

OLT(config)#load packetfile ftp 192.168.1.16 admin admin FD1216S_Kernel_150914 _1605.img Broadcast message from root: Upgrade is in process. File[FD1216S_Kernel_150914_1605.img]download.............OK File[FD1216S_Kernel_150914_1605.img]upgrade.............OK

Check OLT Software and Hardware Version

| Command | OLT(config)# show version |
| --- | --- |
| view | enable view or config view |
| Description | This command can check the OLT hardware、software and kernel version information. |

【Example】

Example1：Check OLT firmware version.

OLT(config)# show version Hardware version : V3.0 Firmware version : V1.2.2 (Mon, 17 Jul 2017 14:41:10 +0800) Kernel version : V1.0.0 (Thu, 29 Jun 2017 17:20:12 +0800) OLT(config)#

Show the progress of current load/copy/backup in olt

| Command | OLT(config)#show progress |
| --- | --- |
| view | Enable view,config view |
| Description | This command is used when the device is performing load,copy,and backup operations,if you need to see the progress of the current operation and understand the status of the operation. |

【Example】

Example 1：Check the status of load progress

OLT(config)#show progress load ----------------------------------------------------------------------------- Transmit Protocal:FTP FTP Server:192.168.1.16 FTP User Name:admin FTP Password:admin Transmit FileName:config Transmit Action:Put Transmit Status:Success Transmit Progress:100% ----------------------------------------------------------------------------- Load Operation:Null Load FileName:config -----------------------------------------------------------------------------

OLT Device Management

OLT Reboot

| Command | OLT(config)#reboot |
| --- | --- |
| view | Enable view or config view |
| Description | This command is used to reboot OLT,only the root user group has this permission. |

【Example】

Example1：reboot OLT

OLT#reboot Please check whether data has saved,the unsaved data will lose if reboot syst em.Are you sure to reboot system?(y/n)[n]:y

Config OLT Outband Manager IP

| Command | OLT(config-interface-mgmt)#ip address <ip-address> {<ip-address-mask>¦<length of mask>} |
| --- | --- |
| View | Mgmt view |
| Description | The IP address command is used to configure IP addresses and subnet masks of management interfaces.You can visit OLT by this IP address. |
| <ip-address> | IP address.The IP address is divided into five categories,and the user can choose the appropriate IP subnet according to the actual situation.The host address part is all 0 or all 1 has special function,which cannot be used as the general IP address. |
| <ip-address-mask> | Subnet mask.Format for X.X.X.X |
| <length of mask> | length of subnet mask,the range is 0-32 |

【Example】

Example 1：Config the ip address of mgmt interface as 192.168.5.63,the length of subnet mask is 24.

OLT(config-interface-mgmt)#ip address 192.168.5.63 24 OLT(config-interface-mgmt)#

Config Vlanif Interface IP Address(Inband)

| Command | OLT(config-interface-vlanif-100)#ip address <ip-address> {<ip-address-mask>¦<length of mask>} |
| --- | --- |
| view | Vlanif view |
| Description | The IP address command is used to configure the IP address and subnet mask of the VLAN interface.This command is used when the IP packet in the VLAN is required to participate in the three-tier forwarding.After the IP address of the configuration interface and the subnet mask are successful,the IP packet in the VLAN is forwarded by this ip in three layers. |
| <ip-address> | IP address.The IP address is divided into five categories,and the user can choose the appropriate IP subnet according to the actual situation.The host address part is all 0 or all 1 has special function,which cannot be used as the general IP address. |
| <ip-address-mask> | Subnet mask.Format for X.X.X.X |
| <length of mask> | length of subnet mask the range is 0-32 |

【Example】

Example 1：Config the ip address of vlanif interface as 192.168.100.1,subnet mask is 255.255.255.0.

OLT(config-interface-vlanif-100)#ip address 192.168.100.1 255.255.255.0 OLT(config-interface-vlanif-100)#

Show MGMT IP Address（Inband）

| Command | OLT(config)#show interface mgmt |
| --- | --- |
| view | Config view |
| Description | This command is used to query the ip address,mac address,the Maximum Transmit Unit and etc info of mgmt interface. |

【Example】

Example 1：Show the ip address,mac address,the Maximum Transmit Unit and etc info of mgmt interface

OLT(config)#show interface mgmt Description:Outband management interface The Maximum Transmit Unit is 1500 bytes Internet Address is 192.168.5.63,netmask 255.255.255.0 Hardware address is E0:56:43:A9:B4:19 Recive 96006 packets,10423985 bytes Transmit 10753 packets,783785 bytes OLT(config)#

Show Vlanif IP Address（Outband）

| Command | OLT(config)#show interface vlanif <vlan-interface-number> |
| --- | --- |
| view | Config view |
| Description | This command is used to query the ip address,mac address,the Maximum Transmit Unit and etc info of vlanif interface. |
| <vlan-interface-number> | The id of vlanif interface,its range is 1-4094 |

【Example】

Example 1：Show the ip address,mac address,the Maximum Transmit Unit and etc info of vlanif interface

OLT(config)#show interface vlanif 100 Description:Inband interface vlanif100 The Maximum Transmit Unit is 1500 bytes Internet Address is 192.168.100.1,netmask 255.255.255.0 Hardware address is E0:56:43:A9:B4:1A Recive 0 packets,0 bytes Transmit 0 packets,0 bytes OLT(config)#

Example 2:Show the information of all the vlanif interface

OLT(config)#show interface vlanif Description:Inband interface vlanif100 The Maximum Transmit Unit is 1500 bytes Internet Address is 192.168.100.1,netmask 255.255.255.0 Hardware address is E0:56:43:A9:B4:1A Recive 0 packets,0 bytes Transmit 0 packets,0 bytes Description:Inband interface vlanif200 The Maximum Transmit Unit is 1500 bytes Internet Address is 192.168.6.66,netmask 255.255.255.0 Hardware address is E0:56:43:A9:B4:1A Recive 0 packets,0 bytes Transmit 0 packets,0 bytes OLT(config)#

Show OLT Detail Information

| Command | OLT(config)#show device |
| --- | --- |
| view | Enable view,config view |
| Description | This command is used to Show the device model/mac address/SN/vendor name and etc info of olt. |

【Example】

Example 1：Show the device info of olt.

OLT(config)#show device ----------------------------------------------------------------------------- Device model:epon Device MAC address:E0:56:43:A9:B4:19 Device serial-number:AF1701-16080003 Device vendor name:XPON ----------------------------------------------------------------------------- OLT(config)#

Config OLT User Logout Time

| Command | OLT(config)#exec-timeout <time> |
| --- | --- |
| view | enable view,config view |
| Description | This command is used to configure the user login timeout,which will automatically log out when the user does not do anything with the device at the set time.The default is 300 seconds. |
| <time> | The time of timeout,its unit is second. |

【Example】

Example 1：Config the timeout time as 36000s.

OLT(config)#exec-timeout 36000 OLT(config)#

Show OLT User Logout Time

| Command | OLT(config)#show exec-timeout |
| --- | --- |
| view | Enable view,config view |
| Description | This command is used to show the user timeout time. |

【Example】

Example 1：show the user timeout time

OLT#show exec-timeout Timeout:36000s OLT#

Logout System

| Command | OLT(config)#logout |
| --- | --- |
| view | View view,enable view,config view |
| Description | This command is used to logout current system |

【Example】

Example 1：Logout the system.

OLT#logout >>User name:

Exit OLT Current View Mode

| Command | OLT(config)#end |
| --- | --- |
| view | Enable view,config view |
| Description | This command is used to enter view mode from current view. |

【Example】

Example 1：Exit config view to view mode

OLT(config)#end OLT>

Config OLT DNS server IP Address

| Command | OLT(config)#dns server <ip-addr> |
| --- | --- |
| view | Config view |
| Description | This command is used to configure the ip address of DNS server. |
| <ip-addr> | IP address,format for X.X.X.X |

【Example】

Example 1：Config the ip address of olt’s DNS server as 192.168.5.1

OLT(config)#dns server 192.168.5.1 OLT(config)#

Delete OLT DNS Server IP Address

| Command | OLT(config)#no dns server <ip-addr> |
| --- | --- |
| view | Config view |
| Description | This command is used to delete the ip address of DNS server.when there is no parameter after dns server,it is used to delete the primary and secondary dns server. |
| <ip-addr> | IP address,format for X.X.X.X |

【Example】

Example 1：Remove the DNS server's IP address 192.168.5.1 in olt

OLT(config)#no dns server 192.168.5.1 OLT(config)#

Show OLT DNS Server IP Address

| Command | OLT(config)#show dns server |
| --- | --- |
| view | Config view |
| Description | This command is used to show ip address of DNS server. |

【Example】

Example 1：Show the ip address of the DNS server.

OLT(config)#show dns server IPv4 Dns Servers: Domain-server IpAddress 1 192.168.5.1 OLT(config)

Config OLT Hostname

| Command | OLT(config)#sysname <name> |
| --- | --- |
| view | Config view |
| Description | This command is used to set the olt’s sysname which is show in command windows. |
| <name> | Olt’s name,support 1-16 strings. |

【Example】

Example 1：Config the olt’s sysname as test.

OLT(config)#sysname test test(config)#

OLT Status Monitor

Show OLT Fan Working Status

| Command | OLT(config)#show fan |
| --- | --- |
| view | enable view,config view |
| Description | This command is used to show the working status of fan. |

【Example】

Example 1：Show the working status of fan

OLT#show fan ----------------------------------------------------------------------------- FAN[1]status:Normal(7200RPM) FAN[2]status:Normal(7020RPM) FAN[3]status:Normal(7140RPM) ----------------------------------------------------------------------------- OLT#

Show OLT Working Temperature

| Command | OLT(config)#show temperature |
| --- | --- |
| view | enable view,config view |
| Description | This command is used to show the real time working temperature of olt |

【Example】

Example 1：Show the real time working temperature of olt

OLT#show temperature The temperature of the board:45.0(C) OLT#

Show OLT Memory Usage

| Command | OLT(config)#show memory |
| --- | --- |
| view | enable view,config view |
| Description | This command is used to show the memory usage of OLT. |

【Example】

Example 1：Show the memory usage of OLT.

OLT#show memory ----------------------------------------------------------------------------- Total memory:242MB Free memory:124MB Utilization:49% ----------------------------------------------------------------------------- OLT#

Config OLT System Time

| Command | OLT(config)#time <time> |
| --- | --- |
| view | config view |
| Description | This command is used to Set the system time of olt |
| <time> | Time,format for YYYY/MM/DD-HH:MM:SS |

【Example】

Example 1：Set the system time of olt

OLT(config)#time 2017/09/08-10:44:59 OLT(config)#

Show OLT System Time

| Command | OLT(config)#show time |
| --- | --- |
| view | enable view,config view |
| Description | This command is used to show the system time of olt. |

【Example】

Example 1：show the system time of olt.

OLT(config)#show time 2017-09-08 10:48:58+00:00 OLT(config)#

Show OLT Boot Time and Running Time

| Command | OLT(config)#show uptime |
| --- | --- |
| view | enable view,config view |
| Description | This command is used to show the up time and boot time of the olt |

【Example】

Example 1：Show the up time and boot time of the olt

OLT#show uptime System up time:0 day 17 hour 29 minute 47 second System boot time:Thu Sep 7 17:20:33 2017 OLT#

Config OLT NTP Time and Server

| Command | OLT(config)#ntp-service unicast-service {<ip-addr>¦<domain name>} |
| --- | --- |
| view | config view |
| Description | This command is used to set the ip address of the NTP server. |
| <ip-addr> | Ip address of NTP server,format for X.X.X.X |
| <domain name> | The domain name of NTP server |

【Example】

Example 1：Set the NTP server’s ip address as 202.120.2.101

OLT(config)#ntp-service unicast-service 202.120.2.101 OLT(config)#

Delete NTP Time and Server

| Command | OLT(config)#no ntp-service unicast-service {<ip-addr>¦<domain name>} |
| --- | --- |
| view | Config view |
| Description | This command is used to Delete the NTP server |
| <ip-addr> | IP address of NTP server,format for X.X.X.X |
| <domain name> | The domain name of NTP server |

【Example】

Example 1：Delete the NTP server’s ip address 202.120.2.101

OLT(config)#no ntp-service unicast-service 202.120.2.101 OLT(config)#

Show NTP Server Session Information

| Command | OLT(config)#show ntp-service session |
| --- | --- |
| View | Config view |
| Description | This command is used to show the session info of the NTP server |

【Example】

Example 1：Show the session info of the NTP server

OLT(config)#show ntp-service session clock source:202.120.2.101 clock stratum:0 clock status:configured reference clock ID:0.0.0.0 reach:0 current poll:64 secs now:0 offset:+0.000000ms delay:0.000000 disper:0.000000 OLT(config)#

Config OLT Systme Timezone

| Command | OLT(config)#timezone {gmt+/gmt} -<timezone> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the system timezone of the olt.GMT+represents the eastern time zone,which means local time is faster than Greenwich mean time,and GMT-means the west time zone,which means local time is slower than Greenwich mean time. |
| <timezone> | The time of timezone,format for hh:mm.The max value of eastern timezone is 18:00,and the max value of west timezone is 18:00. |

【Example】

Example 1：Set the system timezone of the olt as gmt+08:00

OLT(config)#timezone gmt+08:00 OLT(config)#

Show OLT System Current Timezone

| Command | OLT(config)#show timezone |
| --- | --- |
| View | Config view |
| Description | This command is used to show the system current timezone of the olt |

【Example】

Example 1：Show the system current timezone of the olt

OLT(config)#show timezone The current time zone:GMT+08:00 OLT(config)#

Show OLT Local MAC Address Information

| Command | OLT(config)#show location<mac-addr> |
| --- | --- |
| view | config view |
| Description | This command is used to show the local mac address info of the olt |
| <mac-addr> | Mac address,format for xx:xx:xx:xx:xx:xx |

【Example】

Example 1：Show the local mac E0:56:43:A9:B4:1A info of the olt.

OLT(config)#show location E0:56:43:A9:B4:1A ----------------------------------------------------------------------------- Total:2 ----------------------------------------------------------------------------- MAC VLAN Port MAC-Type ----------------------------------------------------------------------------- E0:56:43:A9:B4:1A 100 cpu static E0:56:43:A9:B4:1A 200 cpu static ----------------------------------------------------------------------------- OLT(config)#

Show OLT CPU Usage

| Command | OLT(config)#show cpu |
| --- | --- |
| View | Config view |
| Description | This command is used to show the cpu status of the olt. |

【Example】

Example 1：Show the cpu status of the olt

OLT(config)#show cpu ----------------------------------------------------------------------------- Utilization:54% Load Average(1min):8.11 Load Average(5min):8.38 Load Average(15min):8.39 ----------------------------------------------------------------------------- OLT(config)#

Show OLT History Input Command

| Command | OLT(config)#show history |
| --- | --- |
| View | enable view,config view |
| Description | This command is used to show the command input history information of the olt |

【Example】

Example 1：Show the command input history information of the olt

OLT(config)#show history enable config exec-timeout 36000 exit show dns server show cpu OLT(config)#

OLT Configuration Manage

Backup OLT Configuration

| Command | OLT(config)#backup configuration ftp <server-ip-address> <user-name> <user-password> <filename> |
| --- | --- |
| view | enable view,config view |
| Description | Backup the config file of the olt |
| <server-ip-address> | IP address of the ftp server |
| <user-name> | ftp user name |
| <user-password> | ftp user password |
| <filename> | The name of the backup configuration file,set it by yourself,does not require a file format. |

【Example】

Example 1：Backup the device config file,the ftp user name is admin,password is admin,set the backup file’s name as config.

OLT(config)#backup configuration ftp 192.168.1.16 admin admin config Start backup configuration files The backup is successful OLT(config)#

Download OLT Configuration

| Command | OLT(config)#load configuration ftp <server-ip-address> <user-name> <user-password> <filename> |
| --- | --- |
| View | Enable view,config view |
| Description | Download the configuration of the olt |
| <server-ip-address> | IP address of the ftp server |
| <user-name> | ftp user name |
| <user-password> | ftp user password |
| <filename> | The name of the configuration file to be downloaded,set it by yourself,does not require a file format. |

【Example】

Example 1：Download the configuration file,the ftp user name is admin,password is admin,set the file’s name as config.

OLT(config)#load configuration ftp 192.168.1.16 admin admin config The new configuration file will overwrite the old one Are you sure to load new configuration file?(y/n)[n]:y Broadcast message from root: Start loading configuration The loading is successful Note:The configuration file will take effect after reboot OLT(config)#

Show OLT Current Configuration

| Command | OLT(config)#show current-config |
| --- | --- |
| View | Enable view,config view |
| Description | This command is used to show the real time configuration file.This command is used when the user completes a set of configurations and verifies that the configuration is correct and needs to query the currently effective configuration command. |

【Example】

Example 1：Show current configuration

OLT(config)#show current-config Current configuration: ! spanning-tree enable spanning-tree timer max-age 6 spanning-tree timer forward-delay 30 spanning-tree timer hello 1 spanning-tree priority 4096 ! interface ge spanning-tree edged-port 1 enable spanning-tree priority 1 16 spanning-tree cost 1 1600 spanning-tree mcheck 1 enable exit

Save OLT Configuration

| Command | OLT(config)#save |
| --- | --- |
| View | enable view,config view |
| Description | This command is used to save the olt current configuration |

【Example】

Example 1：Save the olt current configuration

OLT(config)#save The percentage of saved data is:0% The percentage of saved data is:4% The percentage of saved data is:9% The percentage of saved data is:13% The percentage of saved data is:18% The percentage of saved data is:22% The percentage of saved data is:27% The percentage of saved data is:31% The percentage of saved data is:36% The percentage of saved data is:40% The percentage of saved data is:45% The percentage of saved data is:68% The percentage of saved data is:81% The percentage of saved data is:95% The percentage of saved data is:100% OLT(config)#

Erase OLT Configuration

| Command | OLT(config)#erase saved-config |
| --- | --- |
| View | Enable view,config view |
| Description | This command is used to erase the configuration,and after executing the command,reboot the OLT,and OLT will restore factory Settings. |

【Example】

Example 1：Erase the saved-config

OLT#erase saved-config This command will clear the active board data that has been saved Please rememb er to backup the system configuration data Are you sure to continue?(y/n)[n]:y Successfully restored factory configuration!

Show OLT Saved-config

| Command | OLT(config)#show saved-config |
| --- | --- |
| View | enable view,config view |
| Description | This command is used to show saved-config |

【Example】

Example 1：show saved-config

OLT(config)#show saved-config #Saving user:root #Saving time:2017-03-20 19:00:02+0800 spanning-tree enable spanning-tree timer max-age 6 spanning-tree timer forward-delay 30 spanning-tree timer hello 1 spanning-tree priority 4096 interface ge spanning-tree edged-port 1 enable spanning-tree priority 1 16 spanning-tree cost 1 1600 spanning-tree mcheck 1 enable exit

OLT Login User Manage

Add OLT New Login Username and Password

| Command | OLT(config)#user add <user-name> <user-password> {admin/guest/root} |
| --- | --- |
| View | Config view |
| Description | This command is used to add new users,new user passwords,and the new user groups are root,admin,and guest respectively. Root:the user has all the permissions for the device. Admin:users have configuration,view permissions,no restart,upgrade permissions. Guest:the user has the view configuration,backup permission. |
| <user-name> | New user name |
| <user-password> | New user password |
| admin/guest/ root | The permissions of the new user,there are root/admin/guest respectively. |

【Example】

Example 1：Create an admin user,its name is test,password is test.

OLT(config)#user add test test admin OLT(config)#

Delete OLT Login User

| Command | OLT(config)#user delete <name> |
| --- | --- |
| View | Config view |
| Descripton | This command is used to delete the user.caution:the root user cannot be delete. |
| <name> | The user name to be delete |

【Example】

Example 1：Delete the user test.

OLT(config)#user delete test OLT(config)#

Change OLT Login User Password

| Command | OLT(config)#user password <user-name> <user-password> |
| --- | --- |
| View | Config view |
| Description | This command is used to change the password of the existing user. |
| <user-name> | The user name of the password to be changed. |
| <user-password> | New password |

【Example】

Example 1：Change the user password to 123456

OLT(config)#user password test 123456 OLT(config)#

Show OLT Exist Username

| Command | OLT(config)#show user |
| --- | --- |
| View | Config view |
| Description | This command is used to show all of the users which had been created in olt |

【Example】

Example 1：Show all of the users which had been created in olt

OLT(config)#show user ----------------------------------------------------------------------------- User Group ---------------------------------------------------------------------------- root root yao guest test admin ----------------------------------------------------------------------------- OLT(config)#

Show OLT Login User Information

| Command | OLT(config)#show client |
| --- | --- |
| View | Config view |
| Description | This command is used to show the information of online user |

【Example】

Example 1：show the information of online user

OLT(config)#show client ID Access-Type User-Name IP-Address Login-Time ---------------------------------------------------------------------------- >32 Telnet root 192.168.5.67 03:52:47 46 Telnet root 192.168.5.20 00:07:45 OLT(config)#

Kill OLT Online User

| Command | OLT(config)#client kick-off <client-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to kick the other online user off |
| <client-id> | Login user ID,the range can be 1-4294967295.This ID can be viewed through the show client command. |

【Example】

Example 1：Kick the cilent 44 off.

OLT#client kick-off 44 The user has been kicked off successfully OLT#

OLT SNMP Config

SNMP Enable and Disable

| Command | OLT(config)#snmp-agent {enable¦disable} |
| --- | --- |
| View | config view |
| Description | The EMS can manager the olt,only if the snmp agent function is enabled.EMS can not manager the olt while this function is disabled. |
| enable¦disable | disable：Disable the SNMP agent feature enable e：Enable the SNMP agent feature |

【Example】

Example 1：Disable the SNMP agent feature

OLT(config)#snmp-agent disable OLT(config)#

Example 2:Enable the SNMP agent feature

OLT(config)#snmp-agent enable OLT(config)#

Show OLT Snmp Agent Status

| Command | OLT(config)#show snmp-agent status |
| --- | --- |
| View | Config view |
| Description | This command is used to show the feature status of snmp agent |

【Example】

Example 1：Show the feature status of snmp agent

OLT(config)#show snmp-agent status Snmp agent status:Enable OLT(config)#

Config OLT SNMP Community

| Command | OLT(config)#snmp-agent community read<community-name> |
| --- | --- |
| View | Config view |
| Description | This command is used to config the read community name of SNMP |
| <community-name> | The name of read community and supports 1-32 characters.Generally set to public. |

【Example】

Example 1：Set the snmp agent read community’s name as public

OLT(config)#snmp-agent community read public

Show OLT SNMP Read Community

| Command | OLT(config)#show snmp-agent community read |
| --- | --- |
| View | Config view |
| Description | This command is used to show the snmp agent read community. |

【Example】

Example 1：Show the snmp agent read community.

OLT(config)#show snmp-agent community read ----------------------------------------------------------------------------- Community-Name VACM-Name View-Name ---------------------------------------------------------------------------- public default all ----------------------------------------------------------------------------- OLT(config)#

Config OLT SNMP Write Community

| Command | OLT(config)#snmp-agent community write<community-name> |
| --- | --- |
| View | Config view |
| Description | This command is used to config the write community name of SNMP |
| <community-name> | The name of write community and supports 1-32 characters.Generally set to private. |

【Example】

Example 1：Set the snmp agent write community’s name as private

OLT(config)#snmp-agent community write private OLT(config)#

Show SNMP Write Community

| Command | OLT(config)#show snmp-agent community read |
| --- | --- |
| View | Config view |
| Description | This command is used to show SNMP agent write community |

【Example】

Example 1：Show SNMP agent write community

OLT(config)#show snmp-agent community write ----------------------------------------------------------------------------- Community-Name VACM-Name View-Name ---------------------------------------------------------------------------- private default all ----------------------------------------------------------------------------- OLT(config)#

Config OLT SNMP Community Group with Authentication

| Command | OLT(config)#snmp-agent group v3<group-name>authentication {[notify-view<none¦all>][read-view<none¦all>][write-view<none¦all>]} |
| --- | --- |
| View | Config view |
| Description | This command is used to set secure based snmp agent group carrying with authentication |
| <group-name> | Group name,it supports 1-32 characters |
| notify-view | Specifies the notification view corresponding to the group name. |
| read-view | Set the read-only view corresponding to the group name |
| write-view | Set the read/write view corresponding to group name |
| none | Mismatch view |
| all | Match all the view |

【Example】

Example 1：Set secure based snmp agent group carrying with authentication,group name is test,corresponding to all the notification view.

OLT(config)#snmp-agent group v3 test authentication notify-view all OLT(config)#

Config OLT SNMP Community Group with Unauth

| Command | OLT(config)#snmp-agent group v3<group-name>noauth {[notify-view <none¦all>][ read-view <none¦all>][ write-view <none¦all>]} |
| --- | --- |
| View | Config view |
| Description | This command is used to Set secure mode based SNMP agent group without authentication |
| <group-name> | Group name,it supports 1-32 characters |
| notify-view | Specifies the notification view corresponding to the group name. |
| read-view | Set the read-only view corresponding to the group name |
| write-view | Set the read/write view corresponding to group name |
| none | Mismatch view |
| all | Match all the view |

【Example】

Example1:Set secure mode based SNMP agent group without authentication,group name is test,corresponding to all the read-view.

OLT(config)#snmp-agent group v3 test noauth read-view all OLT(config)#

Config OLT SNMP Community Group with Privacy

| Command | OLT(config)#snmp-agent group v3<group-name>privacy {[notify-view <none¦all>][ read-view <none¦all>][ write-view <none¦all>]} |
| --- | --- |
| View | Config view |
| Description | This command is used to configure a group of secure mode based SNMP agent which carrises with hidden property. |
| <group-name> | Group name,it supports 1-32 characters |
| notify-view | Specifies the notification view corresponding to the group name. |
| read-view | Set the read-only view corresponding to the group name |
| write-view | Set the read/write view corresponding to group name |
| none | Mismatch view |
| all | Match all the view |

【Example】

Example 1：Set secure mode based SNMP agent group carrying with hidden property,group name is test,corresponding to all the write-view.

OLT(config)#snmp-agent group v3 test privacy write-view all OLT(config)#

Config OLT SNMP Agent Description

| Command | OLT(config)#snmp-agent sys-info description<description> |
| --- | --- |
| View | Config view |
| Description | This command is used to configure SNMP agent system description info |
| <description> | Description info,it supports 1-100 characters,the default value is description. |

【Example】

Example 1：Configure the SNMP agent system description info as test

OLT(config)#snmp-agent sys-info description test OLT(config)#

Configure SNMP Agent Location Information

| Command | OLT(config)#snmp-agent sys-info location<location> |
| --- | --- |
| View | config view |
| Description | This command is used to configure SNMP agent system location info |
| <location> | Description info,it supports 1-100 characters.The default value is location |

【Example】

Example 1：Configure SNMP agent system location info as test1

OLT(config)#snmp-agent sys-info location test1 OLT(config)#

Configure SNMP Agent Contact Infomation

| Command | OLT(config)#snmp-agent sys-info contact <contact> |
| --- | --- |
| View | Config view |
| Description | This command is used to configure snmp agent contact info |
| <contact> | Description info,it supports 1-100 characters.The default value is contact |

【Example】

Example 1：Configure SNMP agent contact info as test2

OLT(config)#snmp-agent sys-info contact test2 OLT(config)#

Configure Snmp Agent System Name

| Command | OLT(config)#snmp-agent sys-info name <name> |
| --- | --- |
| View | Config view |
| Description | This command is used to config snmp agent system name |
| <name> | Description info,it supports 1-100 characters.The default value is name |

【Example】

Example 1：Configure the snmp agent system name as test3

OLT(config)#snmp-agent sys-info name test3 OLT(config)#

Show SNMP Agent System Info

| Command | OLT(config)#show snmp-agent sys-info |
| --- | --- |
| View | Config view |
| Description | This command is used to show snmp agent system info |

【Example】

Example 1：Show snmp agent system info

OLT(config)#show snmp-agent sys-info The name of this managed node: test3 The description of this managed node: test The contact person for this managed node: test2 The physical location of this node: test1 OLT(config)#

Config SNMP Trap IP

| Command | OLT(config)#snmp-agent trap <host-name> <ip-addr> <port> <community-name> |
| --- | --- |
| View | Config view |
| Description | This command is used to configure the alarm receive ip of snmp agent |
| <host-name> | Description info,it supports 1-32 characters |
| <ip-addr> | Alarm receive ip |
| <port> | Receive the alarm port number,the range is 1-65535. |
| <community-name> | Community name,it supports 1-32 characters. |

【Example】

Example 1：Configure the snmp agent alarm receive ip as 192.168.5.185,host is test,port number is 563,community name is public.

OLT(config)#snmp-agent trap test 192.168.5.185 563 public OLT(config)#

Show SNMP Trap IP Infor

| Command | OLT(config)#show snmp-agent trap |
| --- | --- |
| View | Config view |
| Description | This command is used to show SNMP agent alarm receive ip info |

【Example】

Example 1：Show SNMP agent alarm receive ip info

OLT(config)#show snmp-agent trap ----------------------------------------------------------------------------- Index Host-Name IP-Address Port Community-Name ---------------------------------------------------------------------------- 1 test 192.168.5.185 563 public ----------------------------------------------------------------------------- OLT(config)#

Config SNMP Access User&Auth Mode &Password

| Command | OLT(config)#snmp-agent usm-user <user-name> <group-id> authentication-mode md5 <md5-password> privacy-mode des56 <des56-password> |
| --- | --- |
| View | Config view |
| Description | This command is used to mapping the snmp agent access entity user to secure group,and configure authentication mode and password meanwhile.The authentication mode is optional parameter.Caution:it is need to create a group of secure mode based snmp agent. |
| <user-name> | Entity access user name |
| <group-id> | Group id which is based on user secure mode. |
| <md5-password> | User authentication password,its length is 8-64 characters. |
| <des56-password> | 56 bits DES encrypted password,its length is 8-64 characters. |

【Example】

Example 1：Set SNMP agent entity access user as test1,mapping it to group test which is based on secure mode,authentication mode is md5,the password is 12345678,the privacy mode is des56 and its password is 11111111

OLT(config)#snmp-agent usm-user v3 test1 test authentication-mode md5 12345678 privacy-mode des56 111111111 OLT(config)#

Show SNMP Access User

| Command | OLT(config)#show snmp-agent usm-user <user> |
| --- | --- |
| View | config view |
| Description | This command is used to show SNMP agent entity access user |
| <user> | Name of entity access user,it supports 1-64 characters.it’s optional,without this parameter it will show all the entity access user info,if it is added it will show the specified entity access user info. |

【Example】

Example 1：Show all the entity access user info

OLT(config)#show snmp-agent usm-user User name:test Group name:test Authentication mode:md5 Authentication key:12345678 Privacy mode:des56 Privacy key:12345678 User name:test1 Group name:test Authentication mode:md5 Authentication key:12345678 Privacy mode:des56 Privacy key:111111111 Total number:2 OLT(config)#

Example 2：Show SNMP agent entity access user“test”info.

OLT(config)#show snmp-agent usm-user test User name:test Group name:test Authentication mode:md5 Authentication key:12345678 Privacy mode:des56 Privacy key:12345678

OLT Uplink Port Configuration

OLT Uplink Basic Function Configuration

Disable Uplink Port

| Command | OLT(config-interface-ge-0/0)#shutdown <port-list> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to disable the specified ge port. |
| <port-list> | The port list to be configured,format for 1,3-5,8. |

【Example】

Example 1：Disable the uplink port ge1-ge3 of olt.

OLT(config-interface-ge-0/0)#shutdown 1-3 OLT(config-interface-ge-0/0)#

Example 2:Disable the uplink port ge5 and ge7 of olt.

OLT(config-interface-ge-0/0)#shutdown 5,7 OLT(config-interface-ge-0/0)#

Enable Uplink Port

| Command | OLT(config-interface-ge-0/0)#no shutdown <port-list> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to enable the specified ge port. |
| <port-list> | The port list to be configured,format for 1,3-5,8. |

【Example】

Example 1：Enable the uplink port ge1-ge3 of olt.

OLT(config-interface-ge-0/0)#no shutdown 1-3 OLT(config-interface-ge-0/0)#

Example 2:Enable the uplink port ge5 and ge7 of olt.

OLT(config-interface-ge-0/0)#no shutdown 5,7 OLT(config-interface-ge-0/0)#

Config Uplink Port Name

| Command | OLT(config-interface-ge-0/0)#port-name <port-id> <name> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to name the ge port,it is convenient for user to manager. |
| <port-id> | The port id to be set,range for 1-8. |
| <name> | The port name to be set |

【Example】

Example 1：Set the port name of ge1 as test.

OLT(config-interface-ge-0/0)#port-name 1 test OLT(config-interface-ge-0/0)#

Delete Uplink Port Name

| Command | OLT(config-interface-ge-0/0)#no port-name <port-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to restore the name of ge port to the default value. |
| <port-id> | The port id to be set,range for 1-8. |

【Example】

Example 1：Restore the name of ge1 to default value.

OLT(config-interface-ge-0/0)#no port-name 1 OLT(config-interface-ge-0/0)#

Config Uplink Electric Port Auto-negotiation

| Command | OLT(config-interface-ge-0/0)#auto-neg <port-list> {enable¦disable} |
| --- | --- |
| View | GE view |
| Description | This command is used to enable¦disable the auto-negotiation mode of Ethernet port.In the case of enabled,the Ethernet port will automatically negotiate port rate and duplex mode with the docking port,and the system will display as auto-negotiation,with the port rate up to 1000M in this mode.In the case of disabled,the rate and working mode of the port is the default value of the system or the set value(that is,mandatory). |
| <port-list> | The port list to be set,format for 5,6-7,8.Port 1-4 is uplink optical port,it does not support auto-negotiation. |
| enable¦disable | Enable:Enable the function of port auto-negotiation Disable:Disable the function of port auto-negotiation |

【Example】

Example 1：Enable the function of ge5 auto-negotiation

OLT(config-interface-ge-0/0)#auto-neg 5 enable OLT(config-interface-ge-0/0)#

Example 2:Disable the function of ge5 auto-negotiation

OLT(config-interface-ge-0/0)#auto-neg 5 disable OLT(config-interface-ge-0/0)#

Config Uplink Electric Port Duplex Mode

| Command | OLT(config-interface-ge-0/0)#duplex <port-list> {full/half} |
| --- | --- |
| View | GE view |
| Description | This command is used to set the duplex mode of the Ethernet port.it will work in manual setting mode like full or half duplex.the default is full duplex. |
| <port-list> | The port list to be set,format for 5,6-7,8.Port 1-4 is uplink optical port,it does not support duplex mode setting.the default duplex mode of optical port and electrical port both are full duplex. |
| full/half | full:Full duplex half:Half duplex |

【Example】

Example 1：Set the duplex mode of uplink port ge5 as half duplex.

OLT(config-interface-ge-0/0)#duplex 5 half OLT(config-interface-ge-0/0)#

Config Uplink Electric Port Speed

| Command | OLT(config-interface-ge-0/0)#speed <port-list> {10/100} |
| --- | --- |
| View | GE view |
| Description | This command is used to set the Ethernet port rate that will make the port work in manual setting rate. |
| <port-list> | The port list to be set,format for 5,6-7,8.Port 1-4 is uplink optical port,it does not support the rate setting.the default rate of optical port and electric port both are 1000M. |
| 10/100 | 10:10Mbs 100:100Mbs Caution:1000Mbs only support auto-negotiation. |

【Example】

Example 1：Set the rate of ge5 as 100Mbs.

OLT(config-interface-ge-0/0)#speed 5 100 OLT(config-interface-ge-0/0)#

Config Uplink Port MTU

| Command | OLT(config-interface-ge-0/0)#mtu <port-list> <mtu-value> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the mtu（Maximum Transmission Unit，MTU）.The maximum transmission unit represents the size of the maximum transmission packet which transfers in the port for each unit time.The default value is 1500. |
| <port-list> | The port list to be set,format for 5,6-7,8. |
| <mtu-value> | The range of mtu：328~16356.Particularly,the mtu upper limit of pon port supports 2048. |

【Example】

Example 1：Set the mtu of ge1 as 1600.

OLT(config-interface-ge-0/0)#mtu 1 1600 OLT(config-interface-ge-0/0)#

Delete Uplink Port MTU

| Command | OLT(config-interface-ge-0/0)#no mtu <port-list> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to reset the default value of the mtu. |
| <port-list> | The port list to be set,format for 5,6-7,8. |

【Example】

Example 1：Reset the mtu value of GE1 port to default 1500.

OLT(config-interface-ge-0/0)#no mtu 1 OLT(config-interface-ge-0/0)#

Config Uplink Port Flow-control Function

| Command | OLT(config-interface-ge-0/0)#flow-control <port-list>{enable¦disable} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to enable or disable the flow-control function of the Ethernet port. |
| <port-list> | The port list to be set,format for 5,6-7,8. |
| {enable¦disable} | enable:Enable the flow-control disable:Disable the flow-control |

【Example】

Example 1：Enable the flow-control function of port GE5.

OLT(config-interface-ge-0/0)#flow-control 5 enable OLT(config-interface-ge-0/0)#

Config Uplink Port MAC Address Learning Function

| Command | OLT(config-interface-ge-0/0)#mac-address learning port <port-list> {enable¦disable} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to enable or disable the learning mac function of GE port. |
| <port-list> | The port list to be set,the range for 1-16,format for 5,6-7,8. |
| {enable¦disable} | Enable:Enable GE port’s learning mac function. Disable:Disable GE port’s learning mac function. |

【Example】

Example 1：Enable GE1 port’s learning mac function.

OLT(config-interface-ge-0/0)#mac-address learning port 1 enable OLT(config-interface-ge-0/0)#

Show OLT Uplink Port Property and Statuss

| Command | OLT(config-interface-ge-0/0)#show port state {<port-id>¦all} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the status info of GE port. |
| {<port-id>¦all} | Port-ID:Port id to be show,range for 1-8 All:Show the info of all the port |

【Example】

Example 1：Show property and status of all the GE port.

OLT(config-interface-ge-0/0)#show port state all ----------------------------------------------------------------------------- Port Optic Pvid Auto Speed Dup Flow Learn Enable Link Mtu Status Nego/Mbps lex Ctrl ----------------------------------------------------------------------------- ge0/0/1 absence 1 enable 1000 full on enable enable off 1500 ge0/0/2 absence 1 enable 1000 full on enable enable off 1500 ge0/0/3 absence 1 enable 1000 full on enable enable off 1500 ge0/0/4 absence 1 enable 1000 full on enable enable off 1500 ge0/0/5-1 enable 1000 full off enable enable off 1500 ge0/0/6-1 enable 1000 full off enable enable off 1500 ge0/0/7-1 enable 1000 full off enable enable off 1500 ge0/0/8-1 enable 1000 full off enable enable off 1500 -----------------------------------------------------------------------------

Example 2:Show property and status of GE1.

OLT(config-interface-ge-0/0)#show port state 1 ge0/0/1 information summary: port name:epon1 current port state:enable current link state:DOWN The Maximum Transmit Unit:1500 The port 15 minute statistics status:enable The port 24 hour statistics status:disable Link speed:autonegotiation(1000 MBps) link duplex:autonegotiation(FULL) Flow-control:on Maximum number of learned l2 entries:unlimited broadcasts stormcontrol:148800(pps) unknow multicasts stormcontrol:disable unknow unicasts stormcontrol:disable native-vlan:1 Port link-type:Access Tagged VLAN ID:none Untagged VLAN ID: 1 statistics from last clean(maybe the statistics would overflow): Input(total):0 bytes Input:unicast 0,broadcasts 0,multicasts 0,errors 0 Output(total):0 bytes Output:unicast 0,broadcasts 0,multicasts 0,errors 0 OLT(config-interface-ge-0/0)#

Show OLT Uplink Port Optical Power Information

| Command | OLT(config-interface-ge-0/0)#show ddm-info <port-id> |
| --- | --- |
| View | GE view |
| Description | This command is used to show optical power info of optical GE port |
| <port-id> | Port id to be set,range for 1-4 |

【Example】

Example 1：Show the optical power info of optical port GE1.

OLT(config-interface-ge-0/0)#show ddm-info 1 ----------------------------------------------------------------------------- Temperature(C):37.6 Supply Voltage(V):3.32 TX Bias current(mA):32 TX power(dBm):-4.03 RX power(dBm):-15.49 ----------------------------------------------------------------------------- OLT(config-interface-ge-0/0)#

Uplink Port Mirror Manage

Config Uplink Port Mirror Function

| Command | OLT(config-interface-ge-0/0)#mirror src-port <src-port-id> dst-port {ge/xge } <F/S/P> { all/egress/ingress} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the mirror function of the Ethernet port.When it is needed to copy the flow of a port to output in another port,or used to flow monitoring and network fault diagnosis,use this command.when the mirror function of the Ethernet port is set successfully,the message of specified direction in mirror source port will be completely copied to the destination mirror port. |
| <src-port-id> | The port list to be set,the range for 1-8 |
| ge/xge | ge:Giga GE port xge:10gigabit XGE port |
| <F/S/P> | Destination mirror port id,range for 0/0/1-0/0/8. |
| all/egress/ingress | all:Mirror source port Tx and Rx two-way message.Tx and Rx message of mirror source is completely copied and output to destination mirror port. egress:Mirror source port Tx message.Completely copy and output the Tx message of mirror source port to the mirror destination port. ingress:Mirror source port Rx message.Completely copy and output the Rx message of mirror source port to mirror destination port. |

【Example】

Example 1：Mirror both the ingress and egress message of the port GE3 to the port GE5.

OLT(config-interface-ge-0/0)#mirror src-port 3 dst-port ge 0/0/5 all OLT(config-interface-ge-0/0)#

Delete Uplink Port Mirror Function

| Command | OLT(config-interface-ge-0/0)#no mirror src-port<src-port-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to delete the mirror function configuration of the Ethernet port |
| <src-port-id> | Mirror source port id |

【Example】

Example 1：Delete port GE3 mirror function configuration.

OLT(config-interface-ge-0/0)#no mirror src-port 3 OLT(config-interface-ge-0/0)#

Show Uplink Port Mirror Configuration

| Command | OLT(config-interface-ge-0/0)#show mirror |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the Ethernet port mirror function configuration info. |

【Example】

Example 1：Show the GE port mirror function configuration info.

OLT(config-interface-ge-0/0)#show mirror ----------------------------------------------------------------------------- Destnation port:ge0/0/5 Source port Ingress Egress ge0/0/3 Yes Yes ----------------------------------------------------------------------------- OLT(config-interface-ge-0/0)#

Uplink Port Performance Statistics Function

Config Uplink Port Performance Statistics Threshold

| Command | OLT(config-interface-ge-0/0)#statistics port <port-list> threshold <type> <upper-threshold> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the performance statistics threshold of GE port |
| <port-list> | The port list to be set,format for 5,6-7,8. |
| <type-ID> | Range for 1-64,among which: 1:rx-octets:Byte of receive message 2:rx-frames:Frame of receive massage 3:rx-bcasts:Received broadcast message 4:rx-mcasts:Received multicast message 5:rx-64octets:The received message with 64 Bytes 6:rx-65to127octets:The received message with 65-127 Bytes 7:rx-128to255octets:The received message with 128-255 Bytes 8:rx-256to511octets:The received message with 256-511 Bytes 9:rx-512to1023octets:The received message with 512-1023 Bytes 10:rx-1024to1518octets:The received message with 1024-1518 Bytes 13:rx-oversizes:Oversize received packet 20:rx-discards:The discard received message 23:tx-octets:The Byte of transmit message 24:tx-frames:Transmitted frame 25:tx-bcasts:Transmitted broadcast packet 26:tx-mcasts:Transmitted multicast packet 27:tx-64octets:Transmitted packet with 64 bytes 28:tx-65to127octets:Transmitted packet with 65-127 bytes 29:tx-128to255octets:Transmitted packet with 128-255 bytes 30:tx-256to511octets:Transmitted packet with 256-511 bytes 31:tx-512to1023octets:Transmitted packet with 512-1023 bytes 32:tx-1024to1518octets:Transmitted packet with 1024-1518 bytes 35:tx-oversizes:The oversize transmitted message 42:tx-discards:The discard transmitted message |
| <upper-threshold> | The upper limit of threshold,range for 0-4294967295 |
| <lower-threshold> | The lower limit of threshold,range for 0-4294967295 |

【Example】

Example 1：Set the received frame quantities of GE1 port statistics,upper limit for 50000,lower limit for 500.

OLT(config-interface-ge-0/0)#statistics port 1 threshold 35 50000 500 OLT(config-interface-ge-0/0)#

Show Uplink Port Performance Statistics Threshold

Configuration

| Command | OLT(config-interface-ge-0/0)#show statistics port <port-id> threshold |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the configuration of GE port performance statistics threshold |
| <port-id> | Port id to be show,range for 1-8 |

【Example】

Example 1：Show the configuration of GE1 performance statistics threshold

OLT(config-interface-ge-0/0)#show statistics port 1 threshold TX oversize frames:upper:50000 lower:500 OLT(config-interface-ge-0/0)#

Clear Uplink Port Performance Statistics Infor

| Command | OLT(config-interface-ge-0/0)#reset port statistics <port-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to clear the performance statistics info of GE port |
| <port-id> | Port id to be show,range for 1-8 |

【Example】

Example 1：Clear the performance statistics info of GE1 port

OLT(config-interface-ge-0/0)#reset statistics port 1 OLT(config-interface-ge-0/0)#

Config Uplink Port Performance Statistics Period

15minutes

| Command | OLT(config-interface-ge-0/0)#statistics port <port-list> 15min {enable¦disable} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to enable or disable 15 minutes time interval function of GE port performance statistics |
| <port-list> | Port list to be set,format for 1,6-7,8 |
| enable¦disable | enable:Enable 15 minutes time interval performance statistics disable:Disable 15 minutes time interval performance statistics |

【Example】

Example 1：Enable port GE1 15 minutes time interval performance statistics

OLT(config-interface-ge-0/0)#statistics port 1 15min enable OLT(config-interface-ge-0/0)#

Config Uplink Port Performance Statistics Period 24H

| Command | OLT(config-interface-ge-0/0)#statistics port<port-list> 24hour {enable¦disable} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to enable or disable 24 hours time interval function of GE port performance statistics |
| <port-list> | Port list to be set,format for 1,6-7,8 |
| {enable¦disable} | enable:Enable 24 hours time interval performance statistics disable:Disable 24 hours time interval performance statistics |

【Example】

Example 1：Enable port GE1 24 hours time interval performance statistics

OLT(config-interface-ge-0/0)#statistics port 1 24hour enable OLT(config-interface-ge-0/0)#

Show Uplink Port Current 15min Performance Statistics

| Command | OLT(config-interface-ge-0/0)#show statistics port <port-id> current-15min |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the GE port performance statistics for current 15 minutes |
| <port-id> | Port id to be show,range for 1-8 |

【Example】

Example 1：Show the port GE1 performance statistics for current 15 minutes

OLT(config-interface-ge-0/0)#show statistics port 1 current-15min ----------------------------------------------------------------------------- Start time of this interval:2000-01-04 16:39:56+08:00 Total elapsed seconds in this interval:237 ----------------------------------------------------------------------------- RX octets:0 RX frames:0 RX unicast frames:0 RX broadcast frames:0 RX multicast frames:0 RX discard frames:0 RX error frames:0 RX oversize frames:0 RX frames 64 octets:0 RX frames 65 to 127 octets:0 RX frames 128 to 255 octets:0 RX frames 256 to 511 octets:0 RX frames 512 to 1023 octets:0 RX frames 1024 to 1518 octets:0 TX octets:0 TX frames:0 TX unicast frames:0 TX broadcast frames:0 TX multicast frames:0 TX discard frames:0 TX error frames:0 TX oversize frames:0 TX frames 64 octets:0 TX frames 65 to 127 octets:0 TX frames 128 to 255 octets:0 TX frames 256 to 511 octets:0 TX frames 512 to 1023 octets:0 TX frames 1024 to 1518 octets:0 ----------------------------------------------------------------------------- OLT(config-interface-ge-0/0)#

Show Uplink Port Current 24Hs Performance Statistics

| Command | OLT(config-interface-ge-0/0)#show statistics port <port-id> current-24hour |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the GE port performance statistics for current 24 hour |
| <port-id> | Port id to be show,range for 1-8 |

【Example】

Example 1：Show the port GE1 performance statistics for current 24 hours.

OLT(config-interface-ge-0/0)#show statistics port 1 current-24hour ----------------------------------------------------------------------------- Start time of this interval:2000-01-04 16:39:56+08:00 Total elapsed seconds in this interval:371 ----------------------------------------------------------------------------- RX octets:0 RX frames:0 RX unicast frames:0 RX broadcast frames:0 RX multicast frames:0 RX discard frames:0 RX error frames:0 RX oversize frames:0 RX frames 64 octets:0 RX frames 65 to 127 octets:0 RX frames 128 to 255 octets:0 RX frames 256 to 511 octets:0 RX frames 512 to 1023 octets:0 RX frames 1024 to 1518 octets:0 TX octets:0 TX frames:0 TX unicast frames:0 TX broadcast frames:0 TX multicast frames:0 TX discard frames:0 TX error frames:0 TX oversize frames:0 TX frames 64 octets:0 TX frames 65 to 127 octets:0 TX frames 128 to 255 octets:0 TX frames 256 to 511 octets:0 TX frames 512 to 1023 octets:0 TX frames 1024 to 1518 octets:0 ----------------------------------------------------------------------------- OLT(config-interface-ge-0/0)#

Show Uplink Port History 15min Performance Statistics

| Command | OLT(config-interface-ge-0/0)#show statistics port <port-id> historic-15min <interval-number> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the GE port the past 15min performance statistics info |
| <port-id> | Port id to be show,range for 1-8 |
| <interval-number> | Interval number,range for 1-96.That means time=15min*interval number. |

【Example】

Example 1：Show the port GE1 the past 15min performance statistics info

OLT(config-interface-ge-0/0)#show statistics port 1 historic-15min 1 ----------------------------------------------------------------------------- Start time of this interval:2000-01-04 16:39:56+08:00 Interval number of historical 15 minutes:1 The data for this interval is valid Total monitored seconds in the historic interval:900 ----------------------------------------------------------------------------- RX octets:0 RX frames:0 RX unicast frames:0 RX broadcast frames:0 RX multicast frames:0 RX discard frames:0 RX error frames:0 RX oversize frames:0 RX frames 64 octets:0 RX frames 65 to 127 octets:0 RX frames 128 to 255 octets:0 RX frames 256 to 511 octets:0 RX frames 512 to 1023 octets:0 RX frames 1024 to 1518 octets:0 TX octets:0 TX frames:0 TX unicast frames:0 TX broadcast frames:0 TX multicast frames:0 TX discard frames:0 TX error frames:0 TX oversize frames:0 TX frames 64 octets:0 TX frames 65 to 127 octets:0 TX frames 128 to 255 octets:0 TX frames 256 to 511 octets:0 TX frames 512 to 1023 octets:0 TX frames 1024 to 1518 octets:0 ----------------------------------------------------------------------------- OLT(config-interface-ge-0/0)#

Show Uplink Port History 24H Performance Statistics

| Command | OLT(config-interface-ge-0/0)#show statistics port <port-id> historic-24hour <interval-number> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the GE port the past 24 hours performance statistics info |
| <port-id> | Port id to be show,range for 1-8 |
| <interval-number> | Interval number,range for 1-7.That means time=24h*interval number |

【Example】

Example 1：Show the port GE1 the past 24 hours performance statistics info

OLT(config-interface-ge-0/0)#show statistics port 5 historic-24hour 1 The data for this interval is invalid! OLT(config-interface-ge-0/0)#

Uplink Port Storm Control Function

Config Uplink Port Broadcast Storm Control Function

| Command | OLT(config-interface-ge-0/0)#traffic-suppress <port-id> broadcast {enable¦disable} pps <value> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to enable or disable the broadcast storm suppression function and set the pulse value per second of the GE port.Preventing such information from occupying excessive network resources,resulting in network congestion. |
| <port-id> | Port id to be set,range for 1-8 |
| {enable¦disable} | enable:Enable GE port broadcast storm suppression function disable:Disable GE port broadcast storm suppression function |
| <value> | The number of pulses per second,range for 1-1488100,unit for pps |

【Example】

Example 1：Enable GE1 broadcast storm suppression function and set the number of pulses per second as 14000pps.

OLT(config-interface-ge-0/0)#traffic-suppress 1 broadcast enable pps 14000 OLT(config-interface-ge-0/0)#

Config Uplink Port Unknown Multicast Storm Control Function

| Command | OLT(config-interface-ge-0/0)#traffic-suppress <port-id> non-multicast {enable¦disable} pps <value> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to enable or disable the unknown multicast storm suppression function and pulse value per second of the GE port.Preventing such information from occupying excessive network resources,resulting in network congestion. |
| <port-id> | Port id to be set,range for 1-8 |
| {enable¦disable} | enable:Enable GE port broadcast storm suppression function disable:Disable GE port broadcast storm suppression function |
| <value> | The number of pulses per second,range for 1-1488100,unit for pps |

【Example】

Example 1：Enable GE1 unknown multicast storm suppression function and set the number of pulses per second as 14000pps.

OLT(config-interface-ge-0/0)#traffic-suppress 1 non-multicast enable pps 14000 OLT(config-interface-ge-0/0)#

Config Uplink Port Unknown Unicast Storm Control

| Command | OLT(config-interface-ge-0/0)#traffic-suppress <port-id> broadcast {enable¦disable} pps <value> |
| --- | --- |
| view | XGE view or GE view |
| Description | This command is used to enable or disable the unknown unicast storm suppression function and pulse value per second of the GE port.Preventing such information from occupying excessive network resources,resulting in network congestion. |
| <port-id> | Port id to be set,range for 1-8 |
| {enable¦disable} | enable:Enable GE port broadcast storm suppression function disable:Disable GE port broadcast storm suppression function |
| <value> | The number of pulses per second,range for 1-1488100,unit for pps |

【Example】

Example 1：Enable GE1 unknown unicast storm suppression function and set the number of pulses per second as 14000pps.

OLT(config-interface-ge-0/0)#traffic-suppress 1 non-unicast enable pps 14000 OLT(config-interface-ge-0/0)#

Uplink Port Rate Limit Function

Config Uplink Port Upstream and Downstream Rate Limit

| Command | OLT(config-interface-ge-0/0)#port-rate <port-list> {egress/ingress} <rate> |
| --- | --- |
| view | XGE view or GE view |
| Description | This command is used to configure a rate limit for the port,with a rate limit on the downlink or uplink direction of the port. |
| <port-list> | Port list to be set,format for 1,6-7,8 |
| egress/ingress | Egress:downlink Ingress:uplink |
| <rate> | Rate,range for 64-10240000,unit is Kbps.The default value without rate limit. |

【Example】

Example 1：The GE1 port downstream rate limits as 102400

OLT(config-interface-ge-0/0)#port-rate 1 egress 102400 OLT(config-interface-ge-0/0)#

Delete Uplink Port Upstream and Downstream Rate Limit

| Command | OLT(config-interface-ge-0/0)#no port-rate <port-list> {egress/ingress} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to delete the upstream and downstream rate limit of GE port |
| <port-list> | Port list to be set,format for 1,6-7,8 |
| egress/ingress | Egress:downlink Ingress:uplink |
| <rate> | Rate,range for 64-10240000,unit is Kbps. |

【Example】

Example 1：Delete the downstream rate limit of port GE1.

OLT(config-interface-ge-0/0)#no port-rate 1 egress OLT(config-interface-ge-0/0)#

Show Uplink Port Upstream and Downstream Rate Limit Configuration

| Command | OLT(config-interface-ge-0/0)#show port-rate <port-list> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the upstream and downstream rate limitation info of GE port |
| <port-list> | Port list to be set,format for 1,6-7,8 |

【Example】

Example 1：Show the upstream and downstream rate limitation info of port GE1.

OLT(config-interface-ge-0/0)#show port-rate 1 Traffic shaping: ----------------------------------------------------------------------------- port egress ingress ge0/0/1 123000 0 OLT(config-interface-ge-0/0)#

Uplink Port Isolate Function

Config Uplink Port Isolate

| Command | OLT(config-interface-ge-0/0)#isolate <port-list>{ enable¦disable} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to enable or disable the function of GE port isolation.When port isolation is enabled,the port can not communicate with other ports.By default,it is disabled. |
| <port-list> | Port list to be set,format for 1,6-7,8 |
| {enable¦disable} | Enable:enable the port isolation Disable:disable the port isolation |

【Example】

Example 1：Enable the port isolation of GE1.

OLT(config-interface-ge-0/0)#isolate 1 enable OLT(config-interface-ge-0/0)#

Show Uplink Port Isolation Configuration Infor

| Command | OLT(config-interface-ge-0/0)#show port isolate |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the GE port isolation configuration info |

【Example】

Example 1：Show the GE port isolation configuration info

OLT(config-interface-ge-0/0)#show port isolate Isolate among pon port: pon0/0/1 pon0/0/2 pon0/0/3 pon0/0/4 pon0/0/5 pon0/0/6 pon0/0/7 pon0/0/8 pon0/0/9 pon0/0/10 pon0/0/11 pon0/0/12 pon0/0/13 pon0/0/14 pon0/0/15 pon0/0/16 Isolate among uplink port: ge0/0/1 OLT(config-interface-ge-0/0)#

Uplink Port RSTP Function

Config Uplink Port RSTP Cost

| Command | OLT(config-interface-ge-0/0)#spanning-tree cost <port-id> <cost> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set RSTP cost of the GE port.When there are several link and are not root port between two device,the optimal path is decided by port cost. |
| <port-id> | Port id to be set,range for 1-8 |
| <cost> | Cost value,range for 1-200000000. |

【Example】

Example 1：Set the GE1 port RSTP cost as 2000.

OLT(config-interface-ge-0/0)#spanning-tree cost 1 2000 OLT(config-interface-ge-0/0)#

Config Uplink Port RSTP Edged-port

| Command | OLT(config-interface-ge-0/0)#spanning-tree edged-port <port-id> {enable¦disable} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the RSTP edged-port of the GE port.If user specifies a port as edged-port,then when the port migrates forwarding status from congestion status,this port can migrate rapidly doing without waiting for delay time.the user can only set the port which is connected with the terminal as the edged-port.All ports are default to not edged-port. |
| <port-id> | Port id to be set,range for 1-8 |
| {enable¦disable} | enable:Set the port as edged-port disable:Set the port as not edged-port |

【Example】

Example 1：Set the port GE1 as edged-port.

OLT(config-interface-ge-0/0)#spanning-tree edged-port 1 enable OLT(config-interface-ge-0/0)#

Config Uplink Port RSTP Mcheck Property

| Command | OLT(config-interface-ge-0/0)#spanning-tree mcheck <port-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the RSTP mcheck property of GE port.Port mcheck property is used to detected whether the port which is running under STP compatible mode can migrate to RSTP mode.By setting mcheck,you can check whether there is a bridge running STP protocol within the network segment which is connected with current Ethernet port,If yes,RSTP protocol will migrate the protocol running mode of this port to STP mode. |
| <port-id> | Port id to be set,range for 1-8 |

【Example】

Example 1：Set the mcheck of GE1.

OLT(config-interface-ge-0/0)#spanning-tree mcheck 1 OLT(config-interface-ge-0/0)#

Config Uplink Port RSTP Point-to-Point Link Function

| Command | OLT(config-interface-ge-0/0)#spanning-tree point-to-point <port-id> {auto/true/false} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set point-to-point link of GE port spanning tree.If bridge works in RSTP mode,two ports which is connected by p2p link can migrate to forwarding status by sending synchronization message,it reduces the needless transfer delay time,if set this parameter as auto-mode,RSTP protocol can detect whether current Ethernet port has connected with point-to-point link automatically.The user can set by manually whether current Ethernet port connects with the p2p link.The recommendation is auto-mode. |
| <port-id> | Port id to be set,range for 1-8 |
| auto/true/false | auto:Set the point-to-point link as auto-mode true:Connect GE port to point-to-point link false:Disconnect GE port to point-to-point link |

【Example】

Example 1：Set the point-to-point link function of GE1 as true.

OLT(config-interface-ge-0/0)#spanning-tree point-to-point 1 true OLT(config-interface-ge-0/0)#

Config Uplink Port RSTP Priority

| Command | OLT(config-interface-ge-0/0)#spanning-tree priority <port-id> <port-priority> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the RSTP priority of GE port.By setting the priority of the Ethernet port,You can specify that a particular Ethernet port is contained within the spanning tree.Generally,the smaller of the setting value is,the higher of the port priority,this Ethernet port is likely to include in spanning tree.If all the Ethernet port of the bridge adapt to the same index number,the priority of the Ethernet port depends on the index number of the Ethernet port. |
| <port-id> | Port id to be set,range for 1-8 |
| <port-priority> | Port priority,range for 0-240,step length for 16.the default is 128 |

【Example】

Example 1：Set the spanning tree priority of the GE1 as 160.

OLT(config-interface-ge-0/0)#spanning-tree priority 1 160 OLT(config-interface-ge-0/0)#

Show Uplink Port RSTP Configuration

| Command | OLT(config-interface-ge-0/0)#show port spanning-tree <port-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the RSTP configuration info of the GE port. |
| <port-id> | Port id to be show,range for 1-8 |

【Example】

Example 1：Show the RSTP configuration info of the port GE1.

OLT(config-interface-ge-0/0)#show port spanning-tree 1 -----------------------ge0/0/1 RSTP STATUS:-------------------------------- Port STP Mode:RSTP Port Priority:128 Port Path Cost:20000 Port Edge Admin:NON-Edge Port Edge Status:NEdge Port Link Type Admin:Auto Port Link Type Status:P2P Port Role:Unknown Port State:Down ----------------------------------------------------------------------------- OLT(config-interface-ge-0/0)#

OLT Uplink Port VLAN Config

Config Uplink Port VLAN Mode

| Command | OLT(config-interface-ge-0/0)#vlan mode <port-id> {access/hybrid/trunk} |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the vlan mode of GE port,the default is access mode.In each vlan mode,the message processing way of the port is shown inAppendix 1. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8. |
| access/hybrid/trunk | Access:This kind of port only belongs to one vlan,generally it is used to connect to computer. Trunk:This kind of ports can allow multi vlan pass,can receive and transfer the message of different vlan.Usually,it is used to connect to the port between switches. Hybrid:This kind of port allows multiple vlan pass,can receive and transfer the message of different vlan.It can be used to connect the port between switch or connect to the PC. |

【Example】

Example 1：Set the vlan mode of GE1 as access.

OLT(config-interface-ge-0/0)#vlan mode 1 access OLT(config-interface-ge-0/0)#

Config Uplink Port Native-vlan

| Command | OLT(config-interface-ge-0/0)#vlan native-vlan <port-list> <vlan-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set native vlan of the GE port.In each vlan mode,the message processing way of the port is shown inAppendix 1. |
| <port-list> | Port list to be set,range for 1-8,format for 1,2-3,4 |
| <vlan-id> | VLAN ID,range for 1-4094. |

【Example】

Example 1：Set the native vlan of the GE1 as 10.

OLT(config-interface-ge-0/0)#vlan native-vlan 1 10 OLT(config-interface-ge-0/0)#

Config Uplink Port Native-vlan Priority

| Command | OLT(config-interface-ge-0/0)#vlan native-vlan-priority <port-list> <priority> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the native vlan priority of the GE port,the default both are 0. |
| <port-list> | Port list to be set,range for 1-8,format for 1,2-3,4 |
| <priority> | Priority,range for 0-7 |

【Example】

Example 1：Set the native vlan priority of the GE1 port as 1.

OLT(config-interface-ge-0/0)#vlan native-vlan-priority 1 1 OLT(config-interface-ge-0/0)#

Config Uplink Port Access Mode VLAN

| Command | OLT(config-interface-ge-0/0)#vlan access <port-id> <vlan-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set Access vlan of the GE port,the default access vlan both are 1.In each vlan mode,the message processing way of the port is shown inAppendix 1. |
| <port-id> | Port id to be set,range for 1-8 |
| <vlan-id> | Access VLAN ID,range for 1-4094 |

【Example】

Example1:Set the access vlan of the GE port as 100.

OLT(config-interface-ge-0/0)#vlan access 1 100 OLT(config-interface-ge-0/0)#

Config Uplink Port Hybrid Mode VLAN

| Command | OLT(config-interface-ge-0/0)#vlan hybrid <port-id> {tagged¦untagged} <vlan-list> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set hybrid vlan of the GE port,In each vlan mode,the message processing way of the port is shown in Appendix 1. |
| <port-id> | Port id to be set,range for 1-8 |
| {tagged¦untagged} | tagged:Add corresponding vlan tag for the output message untagged:Peel off corresponding vlan tag for output message |
| <vlan-list> | VLAN ID,range for 1-4094.Format can be 1,11-27,100 |

【Example】

Example 1：Add hybrid vlan of GE1 as 10-15 untagged.

OLT(config-interface-ge-0/0)#vlan hybrid 1 untagged 10-15 ge0/0/1:hybrid vlan added,failed:0,success:6 OLT(config-interface-ge-0/0)#

Example 2:Add hybrid vlan of GE 1 as 101 tagged.

OLT(config-interface-ge-0/0)#vlan hybrid 1 tagged 101 ge0/0/1:hybrid vlan added,failed:0,success:1 OLT(config-interface-ge-0/0)#

Delete Uplink Port Hybrid Mode VLAN

| Command | OLT(config-interface-ge-0/0)#no vlan hybrid <port-id> {tagged¦untagged} <vlan-list> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to delete the hybrid vlan of GE port.In each vlan mode,the message processing way of the port is shown inAppendix 1. |
| <port-id> | Port id to be set,range for 1-8 |
| {tagged¦untagged} | tagged:Add corresponding vlan tag for the output message untagged:Peel off corresponding vlan tag for output message |
| <vlan-list> | VLAN ID,range for 1-4094.Format can be 1,11-27,100 |

【Example】

Example 1：Delete GE1 hybrid vlan 10-15 tagged.

OLT(config-interface-ge)#no vlan hybrid 1 tagged 10-15 OLT(config-interface-ge)#

Config Uplink Port Trunk Mode VLAN

| Command | OLT(config-interface-ge-0/0)#vlan trunk <port-id> <vlan-list> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the trunk vlan of GE port.In each vlan mode,the message processing way of the port is shown inAppendix 1. |
| <port-id> | Port id to be set,range for 1-8 |
| <vlan-list> | VLAN list,range for 1-4094.Format can be 1,11-27,100 |

【Example】

Example 1：Set GE1 trunk vlan as 10-15.

OLT(config-interface-ge-0/0)#vlan trunk 1 10-15 ge0/0/1:trunk vlan allowed,failed:0,success:6 OLT(config-interface-ge-0/0)#

Delete Uplink Port Trunk Mode VLAN

| Command | OLT(config-interface-ge-0/0)#no vlan trunk <port-id> <vlan-list> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to delete the trunk vlan of GE port.In each vlan mode,the message processing way of the port is shown inAppendix 1. |
| <port-id> | Port id to be delete,range for 1-8 |
| <vlan-list> | VLAN list,range for 1-4094.Format can be 1,11-27,100 |

【Example】

Example 1：Delete GE1 trunk vlan 10-15.

OLT(config-interface-ge-0/0)#no vlan trunk 1 10-15 OLT(config-interface-ge-0/0)#

Config Uplink Port Translate Mode VLAN

| Command | OLT(config-interface-ge-0/0)#vlan translate <port-list> <old-vlan> <new-vlan> <new-priority> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to set the translate vlan of GE port.In the direction of upstream,it will transfer the old vlan into new vlan and update to new priority. |
| <port-list> | Port list to be set,range for 1-8 |
| <old-vlan> | Old vlan id,range for 1-4094 |
| <new-vlan> | New VLAN ID,range for 1-4094 |
| <new-priority> | New vlan priority,range for 0-7 |

【Example】
Example 1：Translate the GE1’s old vlan 10 into new vlan 11 and the new priority translates into 3.

OLT(config-interface-ge-0/0)#vlan translate 1 10 11 3 OLT(config-interface-ge-0/0)#

Delete Uplink Port Translate Mode VLAN

| Command | OLT(config-interface-ge-0/0)#no vlan translate <port-list> <vlan-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to delete the translate vlan of GE port. |
| <port-list> | Port id to be delete,range for 1-8 |
| <vlan-id> | VLAN id,range for 1-4094. |

【Example】

Example 1：Delete the GE1 translate vlan 10.

OLT(config-interface-ge-0/0)#no vlan translate 1 10 OLT(config-interface-ge-0/0)#

Config Uplink Port Protocol VLAN

| Command | OLT(config-interface-ge-0/0)#protocol-vlan <protocol-index> {add/delete} port <port-list> <vlan-ID> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to bind a protocol vlan index for the port and port vlan,firstly it’s need to create a protocol vlan. |
| <protocol-index> | Protocol vlan index,range for 1-16 |
| add/delete | add:Add vlan delete:Delete vlan |
| <port-list> | Port list to be set,format for 1,6-7,8 |
| <vlan-ID> | VLAN ID,range for 1-4094 |

【Example】

Example 1：Bind GE1 to protocol vlan index 1 and add into vlan 100.

OLT(config-interface-ge-0/0)#protocol-vlan 1 add port 1 100 OLT(config-interface-ge-0/0)#

Show OLT Uplink Port VLAN Configuration

| Command | OLT(config-interface-ge-0/0)#show port vlan <port-id> |
| --- | --- |
| View | XGE view or GE view |
| Description | This command is used to show the vlan info of GE port. |
| <port-id> | Port id to be show,range for 1-8 |

【Example】

Example 1：Show vlan info of port GE1.

OLT(config-interface-ge-0/0)#show port vlan 1 ----------------------------------------------------------------------------- Port:ge0/0/1 Mode:Access Native-Vlan:1 Priority:0 ----------------------------------------------------------------------------- Tagged-Vlan: - ----------------------------------------------------------------------------- Untagged-Vlan: 1 ----------------------------------------------------------------------------- OLT(config-interface-ge-0/0)#

- OLT PON Port Configuration

OLT PON Port Property Config

Disable PON Port

| Command | OLT(config-interface-epon-0/0)#shutdown <port-list> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to disable the specified pon port. |
| <port-list> | Port list to be set,range for 1-16,format for 1,3-5,8 |

【Example】

Example 1：Disable pon port 1-3.

OLT(config-interface-epon-0/0)#shutdown 1-3 OLT(config-interface-epon-0/0)#

Example 2:Disable pon port 5 and 7.

OLT(config-interface-epon-0/0)#shutdown 5,7 OLT(config-interface-epon-0/0)#

Enable PON Port

| Command | OLT(config-interface-epon-0/0)#no shutdown <port-list> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable the specified pon port |
| <port-list> | Port list to be set,range for 1-16,format for 1,3-5,8 |

【Example】

Example 1：Enable pon port 1-3.

OLT(config-interface-epon-0/0)#no shutdown 1-3 OLT(config-interface-epon-0/0)#

Example 2:Enable pon port 5 and 7.

OLT(config-interface-epon-0/0)#no shutdown 5,7 OLT(config-interface-epon-0/0)#

Config PON Port Name

| Command | OLT(config-interface-epon-0/0)#port-name <port-id> <name> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the name of pon port which is convenient for user to management |
| <port-id> | Port id to be set,range for 1-16 |
| <name> | Port name to be set |

【Example】

Example 1：Set the name of pon port as test.

OLT(config-interface-epon-0/0)#port-name 1 test OLT(config-interface-epon-0/0)#

Delete PON Port Name

| Command | OLT(config-interface-epon-0/0)#no port-name <port-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to reset the name of pon port to default value. |
| <port-id> | Port id to be set,range for 1-16 |

【Example】

Example 1：Reset the name of pon1 to default value.

OLT(config-interface-epon-0/0)#no port-name 1 OLT(config-interface-epon-0/0)#

Config PON Port MTU

| Command | OLT(config-interface-epon-0/0)#mtu <port-list> <mtu-value> |
| --- | --- |
| view | EPON interface view |
| Description | This command is used to set the mtu（Maximum Transmission Unit，MTU）.The maximum transmission unit represents the size of the maximum transmission packet which transfers in the port for each unit time.The default value is 1500. |
| <port-list> | Port list to be set,range for 1-16,format for 1,3-5,8 |
| <mtu-value> | The range of mtu is 328~2048.specifically,the max mtu value of pon port is 2048. |

【Example】

Example 1：Set mtu value of pon 1 as 1600.

OLT(config-interface-epon-0/0)#mtu 1 1600 OLT(config-interface-epon-0/0)#

Delete PON Port MTU

| Command | OLT(config-interface-epon-0/0)#no mtu <port-list> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to restore the pon port's mtu value as default 1500. |
| <port-list> | Port list to be set,range for 1-16,format for 16-7,8 |

【Example】

Example 1：Restore the mtu value of pon1 as default 1500.

OLT(config-interface-epon-0/0)#no mtu 1 OLT(config-interface-epon-0/0)#

Config PON Port Encrypt Mode and Key-exchange-time

| Command | OLT(config-interface-epon-0/0)#port encrypt <port-list> mode {aes-128/disable/triple-churning} key-exchange-time <time> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the encrypt mode and key-exchange-time.The default is not encrypted. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| aes-128/disable/triple-churning | aes-128:An encrypted mode disable:Disable port encrypting function triple-churning:An encrypted mode |
| <time> | Key exchange time,range for 1-708,unit is second.It is optional. |

【Example】

Example 1：Set pon 1 encryption mode as aes-128

OLT(config-interface-epon-0/0)#port encrypt 1 mode aes-128 OLT(config-interface-epon-0/0)#

Config PON Port Flow-control Function

| Command | OLT(config-interface-epon-0/0)#flow-control <port-list> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the flow-control function of pon port. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| {enable¦disable} | enable:Enable flow-control function of pon port disable:Disable flow-control function of pon port |

【Example】

Example 1：Enable pon1 flow-control function.

OLT(config-interface-epon-0/0)#flow-control 1 enable OLT(config-interface-epon-0/0)#

Config Pon Port MAC Address Learning Function

| Command | OLT(config-interface-epon-0/0)#mac-address learning port <port-list> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set pon port mac address learning function |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| {enable¦disable} | Enable:Enable pon port mac address learning function Disable:Disable pon port mac address learning function |

【Example】

Example 1：Enable pon1 mac address learning function

OLT(config-interface-epon-0/0)#mac-address learning port 1 enable OLT(config-interface-epon-0/0)#

Show OLT PON Port Property and Status

| Command | OLT(config-interface-epon-0/0)#show port state {<port-id>/all} |
| --- | --- |
| View | EPON interface view |
| Desscription | This command is used to show the property info of pon port |
| <port-id>/all | Port-ID:Port id to be show,range for 1-16 All:Show the property info of all the pon port |

【Example】

Example 1：Show the property info of all the pon port

OLT(config-interface-epon-0/0)#show port state all ----------------------------------------------------------------------------- F/S P Pvid Flow MAC Enable ONU MTU Link Auto Bandwidth Auth Ctrl Learn State Isolate Find(Kbps)Mode ---------------------------------------------------------------------------- 0/0 1 1 on en en en 1500 off en 1000000 auto 0/0 2 1 on en en en 1500 off en 1000000 auto 0/0 3 1 on en en en 1500 off en 1000000 loid 0/0 4 1 on en en en 1500 off en 1000000 auto 0/0 5 1 on en en en 1500 off en 1000000 auto 0/0 6 1 on en en en 1500 off en 1000000 auto 0/0 7 1 on en en en 1500 off en 1000000 auto 0/0 8 1 on en en en 1500 off en 1000000 auto 0/0 9 1 on en en en 1500 off en 1000000 auto 0/0 10 1 on en en en 1500 off en 1000000 auto 0/0 11 1 on en en en 1500 off en 1000000 mac 0/0 12 1 on en en en 1500 off en 1000000 auto 0/0 13 1 on en en en 1500 off en 1000000 auto 0/0 14 1 on en en en 1500 off en 1000000 auto 0/0 15 1 on en en en 1500 off en 1000000 auto 0/0 16 1 on en en en 1500 off en 1000000 auto ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Example 2:Show the property info of pon1.

OLT(config-interface-epon-0/0)#show port state 1 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:1 Port Name:pon0/0/1 Enable state:Enable Encrpyt state:Disable Key exchange time:- ONU isolate state:Enable Optical Module status:Normal Link state:Down Auto find:Enable Auth Mode:auto Policy Auth:Disable Available bandwidth:1000000(Kbps) Native vlan:1 Maximum Transmit Unit:1500 Flow-control:On Maximum learned l2 entries:unlimited Broadcast storm control:disable Unknow multicast storm control:disable Unknown unicast storm control:disable Port 15 minute statistics status:enable Port 24 hour statistics status:enable ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show OLT PON Port Optical Power Information

| Command | OLT(config-interface-epon-0/0)#show port ddm-info <port-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show configuration info of pon port such as optical power,optical module temperature,voltage,serial number and etc. |
| <port-id> | Port id to be show,range for 1-16 |

【Example】

Example 1：Show optical power info of pon1.

OLT(config-interface-epon-0/0)#show port ddm-info 1 ----------------------------------------------------------------------------- Temperature(C):41.9 Supply Voltage(V):3.32 TX Bias current(mA):11 TX power(dBm):4.66 RX power(dBm):- ---------------------------------------------------------------------------- Vendor:T&W Product name:TW5441H-C3AL Version:1.0 Serial number:85165803D ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Config OLT PON Port Detect ONU Long Laser Function

Config Auto Detect ONU Long Laser Function

| Command | OLT(config-interface-epon-0/0)#anti-rogueont auto-detect <port-id> {on/off} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable the function of pon port automatic detecting onu long laser.In the case of enabling,when the onu appears long laser,olt will deliver an alarm info. |
| <port-id> | Port id to be set,range for 1-16,format for 1,6-7,8 |
| on/off | on:Enable pon port automatic detecting onu long laser off:Disable pon port automatic detecting onu long laser |

【Example】

Example 1：Enable pon1 automatic detecting onu long laser

OLT(config-interface-epon-0/0)#anti-rogueont auto-detect 1 on OLT(config-interface-epon-0/0)#

Config Manual Detect ONU Long Laser Function

| Command | OLT(config-interface-epon-0/0)#anti-rogueont manual-detect <port-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the pon port to detect the onu long laser by manually.The pon port start to detect onu long laser after executing this command. |
| <port-id> | Port id to be set,range for 1-16,format for 1,6-7,8 |

【Example】

Example 1：Set pon1 manually detecting onu long laser.

OLT(config-interface-epon-0/0)#anti-rogueont manual-detect 1 OLT(config-interface-epon-0/0)#

Show Detect ONU Long Laser Configuration

| Command | OLT(config-interface-epon-0/0)#show anti-rogueont auto-detect <port-id> status |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show the configuration info that pon port automatically detects onu long laser. |
| <port-id> | Port id to be set,range for 1-16,format for 1,6-7,8 |

【Example】

Example 1：Show the configuration info that pon1 automatically detects the onu long laser.

OLT(config-interface-epon-0/0)#show anti-rogueont auto-detect 1 status ----------------------------------------------------------------------------- Detetion switch:off Detetion interval:15(min) ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Config OLT PON Port Mirror Function

Config Pon Port Mirror Function

| Command | OLT(config-interface-epon-0/0)#mirror src-port <src-port-id> dst-port {ge/xge} <F/S/P> { all/egress/ingress} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the mirror function of the pon port.When it is needed to copy and output the flow of some pon port to other GE port or used to flow detection,network fault diagnosis and data analysis,use this command.When the pon port mirror is set successfully,the specified message of source mirror port will be completely copied to destination mirror port. |
| <src-port-id> | Source mirror port to be set,range for 1-16 |
| ge/xge | ge:Giga GE port xge:10Giga GE port |
| <F/S/P> | Destination mirror port id,range for 0/0/1-0/0/8. |
| all/egress/ingress | all:Tx and Rx double direction message of source mirror port.Completely copy and output the rx and tx message of source mirror port to the destination mirror port. egress:The tx message of source mirror port.Completely copy and output the tx message of source mirror port to destination mirror port. ingress:The rx message of source mirror port.Completely copy and output the rx message of source mirror port to destination mirror port. |

【Example】

Example 1：Mirror the egress and ingress message of pon1 to GE5.

OLT(config-interface-epon-0/0)#mirror src-port 1 dst-port ge 0/0/5 all OLT(config-interface-epon-0/0)#

Delete PON　Port Mirror Function

| Description | OLT(config-interface-epon-0/0)#no mirror src-port <src-port-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to cancel the pon port mirror setting. |
| <src-port-id> | Source mirror port id,range for 1-16 |

【Example】

Example 1：Cancel pon1 mirror setting.

OLT(config-interface-epon-0/0)#no mirror src-port 1 OLT(config-interface-epon-0/0)#

Show PON Port Mirror Configuration

| Command | OLT(config-interface-epon-0/0)#show mirror |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show the pon port mirror setting info. |

【Example】

Example 1：Show mirror setting info.

OLT(config-interface-epon-0/0)#show mirror ----------------------------------------------------------------------------- Destnation port:ge0/0/5 Source port Ingress Egress ge0/0/2 Yes Yes pon0/0/1 Yes Yes ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

OLT PON Port Performance Statistics Function

Config PON Port Performance Statistics Period 15min

| Command | OLT(config-interface-epon-0/0)#statistics port <port-list>15min {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set time interval of pon port performance statistic as 15min. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| {enable¦disable} | enable:Enable 15min performance statistics disable:Disable 15min performance statistics |

【Example】

Example 1：Enable pon1 15min performance statistics function.

OLT(config-interface-epon-0/0)#statistics port 1 15min enable OLT(config-interface-epon-0/0)#

Config PON Port Performance Statistics Period 15H

| Command | OLT(config-interface-epon-0/0)#statistics port <port-list> 24hour {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable the 24h time interval performance statistics of pon port. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| {enable¦disable} | enable:Enable 24h performance statistics disable:Disable 24h performance statistics |

【Example】

Example 1：Enable pon1 24h time interval performance statistics function.

OLT(config-interface-epon-0/0)#statistics port 1 24hour enable OLT(config-interface-epon-0/0)#

Config PON Port Performance Statistics Threshold

| Command | OLT(config-interface-epon-0/0)#statistics port <port-list> threshold <type> <upper-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the threshold of pon port performance statistics. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| <type-ID> | Range can be 1-64,among which: 1:rx-octets:Byte of received message 2:rx-frames:Frame of received message 3:rx-bcasts:The received broadcast message 4:rx-mcasts:The received multicast message 5:rx-64octets:The received frame packet length for 64 bytes 6:rx-65to127octets:The received frame packet length for 65-127 bytes 7:rx-128to255octets:The received frame packet length for 128-255 bytes 8:rx-256to511octets:The received frame packet length for 256-511 bytes 9:rx-512to1023octets:The received frame packet length for 512-1023 bytes 10:rx-1024to1518octets:The received frame packet length for 1024-1518 bytes 13:rx-oversizes:The oversize received packet 20:rx-discards:The discarded message at receiving 23:tx-octets:The byte of transmitted message 24:tx-frames:The frame of transmitted message 25:tx-bcasts:The transmitted broadcast packet 26:tx-mcasts:The transmitted multicast packet 27:tx-64octets:The transmitted frame packet length for 64 bytes 28:tx-65to127octets:The transmitted frame packet length for 65-127 bytes 29:tx-128to255octets:The transmitted frame packet length for 128-255 bytes 30:tx-256to511octets:The transmitted frame packet length for 256-511 bytes 31:tx-512to1023octets:The transmitted frame packet length for 512-1023 bytes 32:tx-1024to1518octets:The transmitted frame packet length for 1024-1518 bytes 35:tx-oversizes:The oversize transmitted packet 42:tx-discards:The discarded packet at transmitting |
| <upper-threshold> | Upper limit threshold,range for 0-4294967295 |
| <lower-threshold> | Lower limit of threshold,range for 0-4294967295 |

【Example】

Example 1：Set the pon port statistics received frame quantity upper limit and lower limit as 50000 and 500.

OLT(config-interface-epon-0/0)#statistics port 1 threshold 35 50000 500 OLT(config-interface-epon-0/0)#

Clear PON Port Performance Statistics Infor

| Command | OLT(config-interface-epon-0/0)#reset statistics port <port-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to clear the pon port performance statistics info. |
| <port-id> | Port id to be set,range for 1,6-7.8 |

【Example】

Example 1：Clear pon1 performance statistics info.

OLT(config-interface-epon-0/0)#reset statistics port 1 OLT(config-interface-epon-0/0)#

Show PON Port Current 15min Performance Statistics

| Command | OLT(config-interface-epon-0/0)#show statistics port <port-id> current-15min |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show current 15min statistics info of pon port. |
| <port-id> | Port id to be show,range for 1-16. |

【Example】

Example 1：Show current 15min statistics info of pon1.

OLT(config-interface-epon-0/0)#show statistics port 1 current-15min ----------------------------------------------------------------------------- Start time of this interval:2000-01-01 08:59:05+08:00 Total elapsed seconds in this interval:619 ----------------------------------------------------------------------------- RX octets:0 RX frames:0 RX unicast frames:0 RX broadcast frames:0 RX multicast frames:0 RX discard frames:0 RX error frames:0 RX oversize frames:0 RX frames 64 octets:0 RX frames 65 to 127 octets:0 RX frames 128 to 255 octets:0 RX frames 256 to 511 octets:0 RX frames 512 to 1023 octets:0 RX frames 1024 to 1518 octets:0 TX octets:0 TX frames:0 TX unicast frames:0 TX broadcast frames:0 TX multicast frames:0 TX discard frames:0 TX error frames:0 TX oversize frames:0 TX frames 64 octets:0 TX frames 65 to 127 octets:0 TX frames 128 to 255 octets:0 TX frames 256 to 511 octets:0 TX frames 512 to 1023 octets:0 TX frames 1024 to 1518 octets:0 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show PON Port Current 24Hs Performance Statistics

| Command | OLT(config-interface-epon-0/0)#show statistics port <port-id> current-24hour |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show current 24h statistics info of pon port |
| <port-id> | Port id to be set,range for 1-16 |

【Example】

Example 1：Show current 24h statistics info of pon 1.

OLT(config-interface-epon-0/0)#show statistics port 1 current-24hour ----------------------------------------------------------------------------- Start time of this interval:2000-01-01 09:00:23+08:00 Total elapsed seconds in this interval:724 ----------------------------------------------------------------------------- RX octets:0 RX frames:0 RX unicast frames:0 RX broadcast frames:0 RX multicast frames:0 RX discard frames:0 RX error frames:0 RX oversize frames:0 RX frames 64 octets:0 RX frames 65 to 127 octets:0 RX frames 128 to 255 octets:0 RX frames 256 to 511 octets:0 RX frames 512 to 1023 octets:0 RX frames 1024 to 1518 octets:0 TX octets:0 TX frames:0 TX unicast frames:0 TX broadcast frames:0 TX multicast frames:0 TX discard frames:0 TX error frames:0 TX oversize frames:0 TX frames 64 octets:0 TX frames 65 to 127 octets:0 TX frames 128 to 255 octets:0 TX frames 256 to 511 octets:0 TX frames 512 to 1023 octets:0 TX frames 1024 to 1518 octets:0 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show PON Port History 15min Performance Statistics

| Command | OLT(config-interface-epon-0/0)#show statistics port <port-id> historic-15min <interval-number> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show performance statistics of pon port over the past 15min |
| <port-id> | Port id to be set,range for 1-16 |
| <interval-number> | Interval number,range for 1-96.Each interval for 15min,so the time=15min*interval number. |

【Example】

Example 1：Show the past 15min statistics info of pon1.

OLT(config-interface-epon-0/0)#show statistics port 1 historic-15min 1 ----------------------------------------------------------------------------- Start time of this interval:2000-01-01 08:59:05+08:00 Interval number of historical 15 minutes:1 The data for this interval is valid Total monitored seconds in the historic interval:900 ----------------------------------------------------------------------------- RX octets:0 RX frames:0 RX unicast frames:0 RX broadcast frames:0 RX multicast frames:0 RX discard frames:0 RX error frames:0 RX oversize frames:0 RX frames 64 octets:0 RX frames 65 to 127 octets:0 RX frames 128 to 255 octets:0 RX frames 256 to 511 octets:0 RX frames 512 to 1023 octets:0 RX frames 1024 to 1518 octets:0 TX octets:0 TX frames:0 TX unicast frames:0 TX broadcast frames:0 TX multicast frames:0 TX discard frames:0 TX error frames:0 TX oversize frames:0 TX frames 64 octets:0 TX frames 65 to 127 octets:0 TX frames 128 to 255 octets:0 TX frames 256 to 511 octets:0 TX frames 512 to 1023 octets:0 TX frames 1024 to 1518 octets:0 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show PON Port History 24H Performance Statistics

| Command | OLT(config-interface-epon-0/0)#show statistics port <port-id> historic-24hour <interval-number> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show the performance statistics of PON over the past 24 hours. |
| <port-id> | Port id to be show,range for 1-16. |
| <interval-number> | Interval number,range for 1-7.Each interval for 24h,so the time=24h*interval number. |

【Example】

Example 1：Show the performance statistics of pon1 over the past 24h.

OLT(config-interface-epon-0/0)#show statistics port 1 historic-24hour 1 The data for this interval is invalid! OLT(config-interface-epon-0/0)#

Show PON Port Performance Statistics Threshold Configuration

| Command | OLT(config-interface-epon-0/0)#show statistics port <port-id> threshold |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show the threshold configuration of pon port performance statistics |
| <port-id> | Port id to be set,range for 1-16. |

【Example】

Example 1：Show threshold configuration of pon1 performance statistics.

OLT(config-interface-epon-0/0)#show statistics port 1 threshold TX oversize frames:upper:50000 lower:500 OLT(config-interface-epon-0/0)#

OLT PON Port Storm Control Function

Config PON Port Broadcast Storm Control Function

| Command | OLT(config-interface-epon-0/0)#traffic-suppress <port-id> broadcast {enable¦disable} pps <value> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable broadcast storm suppression function and set the pulse number per second of pon port.Preventing such message from occupying excessive network source to result in network congestion. |
| <port-id> | Port id to be set,range for 1-16 |
| {enable¦disable} | Enable:Enable broadcast storm suppression function of pon port disable:Disable broadcast storm suppression function of pon port |
| <value> | Pulse number per second,range for 1-1488100,unit is pps |

【Example】

Example 1：Enable broadcast storm suppression function of pon1 and set the pulse number as 14000 pps.

OLT(config-interface-epon-0/0)#traffic-suppress 1 broadcast enable pps 14000 OLT(config-interface-epon-0/0)#

Config PON Port Unknown Multicast Storm Control Function

| Command | OLT(config-interface-epon-0/0)#traffic-suppress <port-id> non-multicast {enable¦disable} pps<value> |
| --- | --- |
| View | PON view |
| Description | This command is used to enable or disable unknown multicast storm suppression function of pon port and set pulse number per second.preventing such message from occupying excessive network source to result in network congestion. |
| <port-id> | Port id to be set,range for 1-16. |
| {enable¦disable} | Enable:Enable unknown multicast storm suppression function of pon port disable:Disable unknown multicast storm suppression function of pon port |
| <value> | Pulse number per second,range for 1-1488100,unit is pps |

【Example】

Example 1：Enable unknown multicast storm suppression function of pon1 and set the pulse number as 14000 pps.

OLT(config-interface-epon-0/0)#traffic-suppress 1 non-multicast enable pps 14000 OLT(config-interface-epon-0/0)#

Config PON Port Unknown Unicast Storm Control

| Command | OLT(config-interface-epon-0/0)#traffic-suppress <port-id> broadcast {enable¦disable} pps <value> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable unknown unicast storm suppression function of pon port and set pulse number per second.preventing such message from occupying excessive network source to result in network congestion. |
| <port-id> | Port id to be set,range for 1-16. |
| {enable¦disable} | Enable:Enable unknown unicast storm suppression function of pon port disable:Disable unknown unicast storm suppression function of pon port |
| <value> | Pulse number per second,range for 1-1488100,unit is pps |

【Example】

Example1:Enable unknown unicast storm suppression function of pon1 and set the pulse number as 14000 pps.

OLT(config-interface-epon-0/0)#traffic-suppress 1 non-unicast enable pps 14000 OLT(config-interface-epon-0/0)#

OLT PON Port Rate Limit Function

Config PON Port Egress and Ingress Rate Limit

| Command | OLT(config-interface-epon-0/0)#port-rate <port-list> {egress/ingress} <rate> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set a rate limitation value for egress or ingress message of pon port. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| egress/ingress | Egress:Downstream direction Ingress:Upstream direction |
| <rate> | Rate,range for 64-10240000,unit is Kbps.Pon port is without limitation by default |

【Example】

Example 1：Limit downstream rate of pon1 as 102400Kbps.

OLT(config-interface-epon-0/0)#port-rate 1 egress 102400 OLT(config-interface-epon-0/0)#

Delete PON Port Egress and Ingress Rate Limit

| Command | OLT(config-interface-epon-0/0)#no port-rate <port-list> {egress/ingress} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to cancel rate limitation of pon port. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| {egress/ingress} | Egress:downstream Ingress:upstream |
| <rate> | Rate,range for 64-10240000,unit is Kbps.Pon port is without limitation by default |

【Example】

Example 1：Cancel downstream rate limitation of pon1.

OLT(config-interface-epon-0/0)#no port-rate 1 egress OLT(config-interface-epon-0/0)#

Show PON Port Egress and Ingress Rate Limit Configuration

| Command | OLT(config-interface-epon-0/0)#show port-rate <port-list> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show upstream and downstream rate limited configuration info of pon port |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |

【Example】

Example 1：Show upstream and downstream rate limited configuration of pon1.

OLT(config-interface-epon-0/0)#show port-rate 1 Traffic shaping: ----------------------------------------------------------------------------- port egress ingress pon0/0/1 0 0 OLT(config-interface-epon-0/0)#

OLT PON Port Isolate Function

Config PON Port Isolate

| Command | OLT(config-interface-epon-0/0)#isolate <port-list> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable the isolating function between port to port.The port can not communicate with other port when the isolating function is enabled.The default is enabled. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| {enable¦disable} | Enable:Enable port to port isolating function Disable:Disable port to port isolating function |

【Example】

Example 1：Enable isolating function of pon1.

OLT(config-interface-epon-0/0)#isolate 1 enable OLT(config-interface-epon-0/0)#

Config PON Port Ont-isolate Function

| Command | OLT(config-interface-epon-0/0)#ont-isolate <port-list> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable ont-isolate function of pon port.The ont that located in the same pon port can not communicate with each other when ont-isolate function is enabled.By default it is enabled. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| {enable¦disable} | Enable:Enable ont-isolate function Disable:Disable ont-isolate function |

【Example】

Example 1：Disable ont-isolate function of pon1.

OLT(config-interface-epon-0/0)#ont-isolate 1 disable OLT(config-interface-epon-0/0)#

Show PON Port Isolation Configuration Info

| Command | OLT(config-interface-epon-0/0)#show port isolate |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show isolating configuration info of pon port. |

【Example】

Example1:Show isolating configuration info of pon port

OLT(config-interface-epon-0/0)#show port isolate Isolate among pon port: pon0/0/1 pon0/0/2 pon0/0/3 pon0/0/4 pon0/0/5 pon0/0/6 pon0/0/7 pon0/0/8 pon0/0/9 pon0/0/10 pon0/0/11 pon0/0/12 pon0/0/13 pon0/0/14 pon0/0/15 pon0/0/16 Isolate among uplink port: ge0/0/1 OLT(config-interface-epon-0/0)#

OLT PON Port VLAN Config

Config PON Port VLAN Mode

| Command | OLT(config-interface-epon-0/0)#vlan mode <port-list> {access/hybrid/trunk} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set vlan mode of pon port.The default is access mode.The processing way to message of the port is shown onAppendix 1 |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| access/hybrid/trunk | Access:This kind of port only belongs to one vlan,generally it is used to connect to the computer Trunk:This kind of port allows to several vlan passing,it can receive and transmit different vlan message,generally,it is used to connect to the port between Switch. Hybrid:This kind of port allows to several vlan passing,it can receive and transmit different vlan message,generally,it is used to connect to the port between Switch or connect to computer. |

【Example】

Example 1：Set the vlan mode of pon1 as access.

OLT(config-interface-epon-0/0)#vlan mode 1 access OLT(config-interface-epon-0/0)#

Config PON Port Native-vlan

| Command | OLT(config-interface-epon-0/0)#vlan native-vlan <port-list> <vlan-ID> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set native vlan of pon port,the default native vlan is 1.The processing way to message of the port is shown onAppendix 1 |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| <vlan-ID> | VLAN ID,range for 1-4094 |

【Example】

Example 1：Set the native vlan of pon1 as 10.

OLT(config-interface-epon-0/0)#vlan native-vlan 1 10 OLT(config-interface-epon-0/0)#

Config PON Port Native-vlan Priority

| Command | OLT(config-interface-epon-0/0)#vlan native-vlan-priority <port-list> <priority> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the native vlan priority of pon port,the default is 0. |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| <priority> | Priority,range for 0-7 |

【Example】

Example 1：Set native vlan of pon1 as 1.

OLT(config-interface-epon-0/0)#vlan native-vlan-priority 1 1 OLT(config-interface-epon-0/0)#

Config PON Port Access Mode VLAN

| Command | OLT(config-interface-epon-0/0)#vlan access <port-list> <vlan-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set access vlan of pon port,the default access vlan is 1.The processing way to message of the port is shown onAppendix 1 |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| <vlan-id> | Access VLAN ID,range for 1-4094 |

【Example】

Example 1：Set the access vlan of pon1 as 100.

OLT(config-interface-epon-0/0)#vlan access 1 100 OLT(config-interface-epon-0/0)#

Config PON Port Hybrid Mode VLAN

| Command | OLT(config-interface-epon-0/0)#vlan hybrid <port-list> <{tagged¦untagged}> <vlan-list> |
| --- | --- |
| View | PON view |
| Description | This command is used to set hybrid vlan of pon port.The processing way to message of the port is shown onAppendix 1 |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| {tagged¦untagged} | tagged:Add vlan tag for tx message untagged:Peel off the vlan tag for the tx message |
| <vlan-list> | VLAN ID,range for 1-4094.format for 1,11-27,100 |

【Example】

Example 1：Set vlan mode of pon1 as hybrid and add untagged vlan 10-15 to it.

OLT(config-interface-epon-0/0)#vlan hybrid 1 untagged 10-15 pon0/0/1:hybrid vlan added,failed:0,success:6 OLT(config-interface-ge-0/0)#

Example 2:Set vlan mode of pon1 as hybrid and add vlan 101 tagged to it.

OLT(config-interface-epon-0/0)#vlan hybrid 1 tagged 101 pon0/0/1:hybrid vlan added,failed:0,success:1 OLT(config-interface-epon-0/0)#

Delete PON Port Hybrid Mode VLAN

| Command | OLT(config-interface-epon-0/0)#no vlan hybrid <port-id> <{tagged¦untagged}> <vlan-list> |
| --- | --- |
| View | PON view |
| Description | This command is used to delete the hybrid vlan of pon port.The processing way to message of the port is shown onAppendix 1 |
| <port-id> | Port ID to be delete,range for 1-16 |
| {tagged¦untagged} | tagged:Add vlan tag for tx message untagged:Peel off the vlan tag for the tx message |
| <vlan-list> | VLAN ID,range for 1-4094.format for 1,11-27,100 |

【Example】

Example 1：Delete hybrid vlan 10-15 tagged of pon1.

OLT(config-interface-epon-0/0)#no vlan hybrid 1 tagged 10-15 OLT(config-interface-epon-0/0)#

Config PON Port Trunk Mode VLAN

| Command | OLT(config-interface-epon-0/0)#vlan trunk <port-list> <vlan-list> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the trunk vlan of pon port.The processing way to message of the port is shown onAppendix 1 |
| <port-list> | Port list to be set,range for 1-16,format for 1,6-7,8 |
| <vlan-list> | VLAN ID,range for 1-4094.Format can be 1,11-27,100 |

【Example】

Example1:Add trunk vlan 10-15 to pon1.

OLT(config-interface-epon-0/0)#vlan trunk 1 10-15 pon0/0/1:trunk vlan allowed,failed:0,success:6 OLT(config-interface-epon-0/0)#

Delete PON Port Trunk Mode VLAN

| Command | OLT(config-interface-epon-0/0)#no vlan trunk <port-id> <vlan-list> |
| --- | --- |
| View | PON view |
| Description | This command is used to delete the trunk vlan of pon port.The processing way to message of the port is shown onAppendix 1 |
| <port-id> | Port ID to be delete,range for 1-16 |
| <vlan-list> | VLAN ID,range for 1-4094.Format can be 1,11-27,100 |

【Example】

Example 1：Delete trunk vlan 10-15 of pon1.

OLT(config-interface-epon-0/0)#no vlan trunk 1 10-15 OLT(config-interface-epon-0/0)#

Config PON Port Translate Mode VLAN

| Command | OLT(config-interface-epon-0/0)#vlan translate <port-list> <old-vlan> <new-vlan> <new-priority> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set translate vlan of pon port.In the upstream direction,it can translate old vlan into new vlan and update new priority |
| <port-list> | Port list to be set,range for 1-16,format is 1,6-7,8 |
| <old-vlan> | Old VLAN ID,range for 1-4094 |
| <new-vlan> | New VLAN ID,range for 1-4094 |
| <new-priority> | New VLAN priority,range for 0-7 |

【Example】

Example 1：Translate vlan 10 of pon1 into vlan 11 and update the priority as 3.

OLT(config-interface-epon-0/0)#vlan translate 1 10 11 3 OLT(config-interface-epon-0/0)#

Delete PON Port Translate Mode VLAN

| Command | OLT(config-interface-ge-0/0)#no vlan translate <port-list> <vlan-list> |
| --- | --- |
| View | PON view |
| Description | This command is used to delete the translate VLAN of pon port.In the upstream direction,it can translate old vlan into new vlan and update new priority |
| <port-list> | Port list to be delete,range for 1-16 |
| <vlan-list> | VLAN list to be delete,range for 1-4094 |

【Example】

Example 1：Delete translate vlan 10 of pon1.

OLT(config-interface-epon)#no vlan translate 1 10 OLT(config-interface-epon)#

Config PON Port QinQ VLAN

| Command | OLT(config-interface-epon-0/0)#vlan qinq <port-list> cvlan-range <start-cvlan> <end-cvlan> <qinq-svlan> <svlan-priority> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set qinq vlan of pon port.In upstream direction,it can add a SVLAN for the stream between start-CVLAN and end-CVLAN.It need to create a SVLAN in global mode(config view) |
| <port-list> | Port list to be set,range for 1-16,format is 1,6-7,8 |
| <start-cvlan> | The inner start vlan,range for 1-4094 |
| <end-cvlan> | The inner end vlan,range for 1-4094 |
| <qinq-svlan> | The outer vlan,range for 1-4094 |
| <svlan-priority> | SVLAN priority,range for 0-7 |

【Example】

Example 1：The qinq setting of pon1:add a svlan 20 for the stream which is between vlan 10-15 and set its priority as 0.

OLT(config-interface-epon-0/0)#vlan qinq 1 cvlan-range 10 15 20 0 OLT(config-interface-epon-0/0)#

Delete PON Port QinQ VLAN

| Command | OLT(config-interface-epon-0/0)#no vlan qinq <port-list> cvlan-range <start-cvlan> <end-cvlan> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to delete qinq vlan of pon port.In upstream direction,it can add a SVLAN for the stream between start-CVLAN and end-CVLAN.It need to create a SVLAN in global mode(config view) |
| <port-list> | Port list to be set,range for 1-16,format is 1,6-7,8 |
| <start-cvlan> | The inner start vlan,range for 1-4094 |
| <end-cvlan> | The inner end vlan,range for 1-4094 |

【Example】

Example 1：Delete the qinq setting of pon1.

OLT(config-interface-epon-0/0)#no vlan qinq 1 cvlan-range 10 15 OLT(config-interface-epon-0/0)#

Config PON Port Aggregation VLAN

| Command | OLT(config-interface-epon-0/0)#vlan aggregation pon-port <port-list> cvlan-range <start-cvlan> <end-cvlan> <aggregation-svlan> <svlan-priority> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set aggregation vlan of pon port.In upstream direction,it can translate the vlan which range is from start-cvlan to end-cvlan into svlan. |
| <port-list> | Port list to be set,range for 1-16,format is 1,6-7,8 |
| <start-cvlan> | The inner start vlan,range for 1-4094 |
| <end-cvlan> | The inner end vlan,range for 1-4094 |
| <aggregation-svlan> | SVLAN ID,the vlan after translating,range for 1-4094 |
| <svlan-priority> | Priority of SVLAN,range for 0-7 |

【Example】

Example 1：Translate the cvlan 10-15 of pon1 upstream direction into svlan 20.

OLT(config-interface-epon-0/0)#vlan aggregation pon-port 1 cvlan-range 10 15 20 0 OLT(config-interface-epon-0/0)#

Config PON Port Protocol VLAN

| Command | OLT(config-interface-epon-0/0)#protocol-vlan <protocol-index> {add¦delete} port <port-list> <vlan-ID> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to bind a protocol vlan index for port and port vlan.Firstly,it needs to create a protocol-vlan in config view |
| <protocol-index> | Protocol vlan index,range for 1-16 |
| {add¦delete} | add:Add vlan delete:Delete vlan |
| <port-list> | Port list to be set,range for 1-16,format is 1,6-7,8 |
| <vlan-ID> | VLAN ID,range for 1-4094 |

【Example】

Example 1：Bind pon1 to protocol vlan index 1 and add it into vlan 100.

OLT(config-interface-epon-0/0)#protocol-vlan 1 add port 1 100 OLT(config-interface-epon-0/0)#

Show OLT PON Port VLAN Configuration

| Command | OLT(config-interface-ge-0/0)#show port vlan <port-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show vlan info od pon port |
| <port-id> | Port-ID:Port id to be show,range for 1-16 |

【Example】

Example 1：Show vlan info of pon1.

OLT(config-interface-epon-0/0)#show port vlan 1 ----------------------------------------------------------------------------- Port:pon0/0/1 Mode:Access Native-Vlan:1 Priority:0 ----------------------------------------------------------------------------- Tagged-Vlan: - ----------------------------------------------------------------------------- Untagged-Vlan: 1 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show PON Port Translate VLAN Configuration

| Command | OLT(config-interface-epon-0/0)#show vlan translate all |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show vlan translating info of all pon port |

【Example】

Example 1：Show VLAN translating info of all the pon port

OLT(config-interface-epon-0/0)#show vlan translate all ----------------------------------------------------------------------------- index port oldvlan newvlan priority mode ----------------------------------------------------------------------------- 1 pon0/0/1 21 30--Translate 2 pon0/0/1 10-12 20 0 QinQ ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show PON Port Aggregation VLAN Configuration

| Command | OLT(config-interface-epon-0/0)#show vlan aggregation pon-port {<port-id>/all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show aggregation vlan info of pon port |
| <port-id>/all | Port-ID:Port ID to be show,range for 1-16 All:Show aggregation vlan info of all the pon port |

【Example】

Example 1：Show aggregation vlan info of all the pon port

OLT(config-interface-epon-0/0)#show vlan aggregation pon-port all ----------------------------------------------------------------------------- index port ontId cvlan svlan priority ----------------------------------------------------------------------------- 1 0/0/1--10-13 20 2 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

- OLT MAC Address Table Manage

Config OLT MAC-address Black-hole

| Command | OLT(config)#mac-address black-hole vlan <vlan-id> <mac-address> |
| --- | --- |
| View | Config view |
| Description | This command is used to specify a black hole mac address table.If source mac address or destination mac address of some message is equal to the mac address in black hole mac address table,the Switch will discard this message. |
| <vlan-id> | VLAN id,range for 1-4094 |
| <mac-address> | Mac address,format for XX:XX:XX:XX:XX:XX |

【Example】

Example 1：Add mac 00:00:00:12:34:56 to black hole mac address table of vlan 100

OLT(config)#mac-address black-hole vlan 100 00:00:00:12:34:56 OLT(config)#

Delete MAC-Address Black-hole

| Command | OLT(config)#no mac-address black-hole vlan <vlan-id> <mac-address> |
| --- | --- |
| View | Config view |
| Description | This command is used to delete black-hole mac address |
| <vlan-id> | VLAN ID,range for 1-4094。 |
| <mac-address> | Mac address,format for XX:XX:XX:XX:XX:XX |

【Example】

Example 1：Delete vlan 100 black-hole mac address 00:00:00:12:34:56

OLT(config)#no mac-address black-hole vlan 100 00:00:00:12:34:56 OLT(config)#

Config OLT Mac Address Entries limit

| Command | OLT(config)#mac-address limit port {epon/ge/xge } F/S <port-list> <number> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the maximum mac address learning entry,when the quantity of mac address is out of this value,OLT will discard the other mac address except the learned mac. |
| epon/ge/xge | epon:PON port ge:ge uplink port xge:xge 10giga uplink port |
| F/S | FrameID/SlotID,<0-0>/<0-0>,the value of 1U olt is 0/0 |
| <port-list> | Port list to be set,range for 1-16,format is 1,6-7,8 |
| <number> | The number of mac address,range for 0-8092,zero means without limitation.The default is 0. |

【Example】

Example 1：Set the maximum learning mac address entry of GE5 as 500

OLT(config)#mac-address limit port ge 0/0 5 500 OLT(config)#

Add Static MAC Address Bind Function

| Command | OLT(config)#mac-address static port {epon/ge/ xge} F/S/P {lag <manual-group-ID>/<lacp-group-ID>)} vlan <vlan-ID> <mac-address> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the static mac address.With this function,the devices needn’t mac address learning process,it can transfer the message according to static mac. |
| epon/ge/xge/lag | epon:Pon port,range for 0/0/1-16 ge:GE uplink port,range for 0/0/1-8 xge:10giga xge uplink port,range for 0/0/1-2 Lag:Port aggregation group,range for 1-8,9-16 |
| F/S/P | FrameID/SlotID/PortID,<0-0>/<0-0>/<1-16> |
| <manual-group-ID>¦<lacp-group-ID> | manual-group-ID,range for 1-8 lacp-group-ID,range for 9-16 |
| <vlan-ID> | VLAN ID,range for 1-4094。 |
| <mac-address> | Mac address,format for XX:XX:XX:XX:XX:XX |

【Example】

Example 1：Bind mac address e0:67:b3:12:eb:f6 with GE5 and vlan 100.

OLT(config)#mac-address static port ge 0/0/5 vlan 100 e0:67:b3:12:eb:f6 OLT(config)#

Example 2:Bind mac address e0:67:b3:12:eb:f6 with pon1 and vlan 100.

OLT(config)#mac-address static port epon 0/0/1 vlan 100 e0:67:b3:12:eb:f7 OLT(config)#

Example 3:Bind mac address e0:67:b3:12:eb:f6 with XGE1 and vlan 100.

OLT(config)#mac-address static port xge 0/0/1 vlan 100 e0:67:b3:12:eb:f8 OLT(config)#

Example 4:Bind mac address e0:67:b3:12:eb:f6 with lag1 and vlan 100.

OLT(config)#mac-address static port lag 1 vlan 100 e0:67:b3:12:eb:f9 OLT(config)#

Delete Static MAC Address bind

| Command | OLT(config)#no mac-address static port {epon/ge/ xge} F/S/P {lag <manual-group-ID>¦<lacp-group-ID>)} vlan <vlan-ID> <mac-address> |
| --- | --- |
| View | Config view |
| Description | This command used to delete static mac address of olt |
| epon/ge/xge/lag | epon:Pon port,range for 0/0/1-16 ge:GE uplink port,range for 0/0/1-8 xge:10giga xge uplink port,range for 0/0/1-2 Lag:Port aggregation group,range for 1-8,9-16 |
| F/S/P | FrameID/SlotID/PortID，<0-0>/<0-0>/<1-16>， |
| <manual-group-ID>/<lacp-group-ID> | FrameID/SlotID/PortID,<0-0>/<0-0>/<1-16> |
| <vlan-ID> | manual-group-ID,range for 1-8 lacp-group-ID,range for 9-16 |
| <mac-address> | VLAN ID,range for 1-4094。 |

【Example】

Example 1：No bind the mac address e0:67:b3:12:eb:f6 with the GE5 and vlan 100.

OLT(config)#no mac-address static port ge 0/0/5 vlan 100 e0:67:b3:12:eb:f6 OLT(config)#

Example 2:No bind the mac address e0:67:b3:12:eb:f6 with the pon1 and vlan 100.

OLT(config)#no mac-address static port epon 0/0/1 vlan 100 e0:67:b3:12:eb:f7 OLT(config)#

Example 3:No bind the mac address e0:67:b3:12:eb:f6 with the XGE1 and vlan 100.

OLT(config)#no mac-address static port xge 0/0/1 vlan 100 e0:67:b3:12:eb:f8 OLT(config)#

Example 4:No bind the mac address e0:67:b3:12:eb:f6 with the lag1 and vlan 100.

OLT(config)#no mac-address static port lag 1 vlan 100 e0:67:b3:12:eb:f9 OLT(config)#

Config OLT MAC Address Aging Time

| Command | OLT(config)#mac-address timer {<aging-time>/no-aging} |
| --- | --- |
| View | Config view |
| Description | This command is used to set the dynamic table body aging time of the system mac address table.it takes effect immediately after successful setting,system will check the dynamic address by timing,if the system has not transmit or receive any message with specified source mac address during the aging time,this mac address will be deleted from mac address table.Dynamic mac address aging timer can release the source of mac address table to learn new mac address. |
| {<aging-time>/no-aging} | <aging-time>:mac address aging time,range for 10-1000000,unit is second no-aging:Set mac address without aging time.when it is no need to open mac address aging function,use this parameter |

【Example】

Example 1：Set mac address aging time as 1000 second.

OLT(config)#mac-address timer 1000 OLT(config)#

Clear OLT MAC Address Table

| Command | OLT(config)#mac-address flush {all/dynamic/black-hole/static} |
| --- | --- |
| View | Config view |
| Description | This command is used to clear the mac address table of olt |
| {all/dynamic/black-hole/static} | All:All the mac address in the table Dynamic:Dynamic mac address black-hole:Black hole mac address Static:Static mac address |

【Example】

Example 1：Clear all the mac address in the mac address table.

OLT(config)#mac-address flush all OLT(config)#

Clear OLT Port MAC Address Table

| Command | OLT(config)#mac-address flush port {epon/ge/ xge} F/S/P / {lag <manual-group-ID>/<lacp-group-ID>/all/dynamic/ static } |
| --- | --- |
| View | Config view |
| Description | This command is used to clear the MAC address learned by the port of OLT |
| {epon/ge/xge/lag} | epon:Pon port,range for 0/0/1-16 ge:GE uplink port,range for 0/0/1-8 xge:10giga xge uplink port,range for 0/0/1-2 Lag:Port aggregation group,range for 1-8,9-16 |
| F/S/P | FrameID/SlotID/PortID，<0-0>/<0-0>/<1-16>， |
| <manual-group-ID>/<lacp-group-ID> | FrameID/SlotID/PortID,<0-0>/<0-0>/<1-16> |
| all/dynamic/static | All:All the mac address in the table Dynamic:Dynamic mac address Static:Static mac address |

【Example】

Example 1：Clear the MAC address learned by GE1.

OLT(config)#mac-address flush port ge 0/0/1 all OLT(config)#

According OLT VLAN Clear MAC Address Table

| Command | OLT(config)#mac-address flush vlan<vlan-ID> {all/black-hole/dynamic/static} |
| --- | --- |
| View | Config view |
| Description | This command is used to clear the mac address learned by the vlan of olt |
| <vlan-ID> | Vlan id |
| all/black-hole/dynamic/static | All:All the mac address in the table Dynamic:Dynamic mac address black-hole:Black hole mac address Static:Static mac address |

【Example】

Example 1：Clear all the mac address learned by vlan 100.

OLT(config)#mac-address flush vlan 100 all OLT(config)#

Show OLT MAC Address Table

| Command | OLT(config)#show mac-address all |
| --- | --- |
| View | Config view |
| Description | This command is used to show all the mac address learn by olt |

【Example】

Example 1：Show all the mac address learned by olt.

OLT(config)#show mac-address all ----------------------------------------------------------------------------- Total:3 ----------------------------------------------------------------------------- MAC VLAN Port MAC-Type ----------------------------------------------------------------------------- E0:56:43:A9:B4:1A 100 cpu static E0:56:43:A9:B4:1A 200 cpu static E0:56:43:A9:B4:1A 1000 cpu static ----------------------------------------------------------------------------- OLT(config)#

Show OLT MAC Address Black Hole

| Command | OLT(config)#show mac-address black-hole |
| --- | --- |
| View | Config view |
| Description | This command is used to show all the black hole mac address of olt |

【Example】

Example1:Show all the black hole mac address of olt.

OLT(config)#show mac-address black-hole <cr>-Please press ENTER to execute command OLT(config)#show mac-address black-hole ----------------------------------------------------------------------------- Total:1 ----------------------------------------------------------------------------- MAC VLAN Port MAC-Type ----------------------------------------------------------------------------- 00:12:13:23:45:32 100 cpu blackhole ----------------------------------------------------------------------------- OLT(config)#

Show OLT Dynamic MAC Address Table

| Command | OLT(config)#show mac-address dynamic |
| --- | --- |
| View | Config view |
| Description | This command is used to show all the dynamic mac address learned by olt |

【Example】

Example 1：Show all the dynamic mac address learned by olt.

OLT(config)#show mac-address dynamic ----------------------------------------------------------------------------- Total:3 ----------------------------------------------------------------------------- MAC VLAN Port MAC-Type ----------------------------------------------------------------------------- 02:02:5C:6E:0F:17 1 ge0/0/5 dynamic F4:06:69:B3:74:8C 1 ge0/0/5 dynamic 00:0A:C2:22:B0:9D 1 ge0/0/5 dynamic ----------------------------------------------------------------------------- OLT(config)#

Show MAC Address Table From ONU

| Command | OLT(config)#show mac-address ont F/S/P <onu-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to show the mac address learned by onu |
| F/S/P | FrameID/SlotID/PortID,range for<0-0>/<0-0>/<1-16> |
| <onu-id> | ONU id,range for 1-64 |

【Example】

Example 1：Show mac address learned by pon1 ont 4.

OLT(config)#show mac-address ont 0/0/1 4 ----------------------------------------------------------------------------- Total:1 ----------------------------------------------------------------------------- MAC VLAN Port ONU-Id MAC-Type ----------------------------------------------------------------------------- E0:67:B3:0D:0E:01 1 pon0/0/1 4 dynamic ----------------------------------------------------------------------------- OLT(config)#

Show MAC Address Table From PON Port

| Command | OLT(config)#show mac-address port epon F/S/P with-ont-location |
| --- | --- |
| View | Config view |
| Description | This command is used to show the mac address learned by pon port and show the onu id that the mac address had through |
| F/S/P | FrameID/SlotID/PortID,range for<0-0>/<0-0>/<1-16> |
| with-ont-location | This parameter is optional,if it is added,it will show the onu id that the mac address had through |

【Example】

Example 1：Show the mac address learned by pon1 and show the onu id that the mac address had through

OLT(config)#show mac-address port epon 0/0/1 with-ont-location ----------------------------------------------------------------------------- Total:1 ----------------------------------------------------------------------------- MAC VLAN Port ONU-Id MAC-Type ----------------------------------------------------------------------------- E0:67:B3:0D:0E:01 1 pon0/0/1 4 dynamic ----------------------------------------------------------------------------- OLT(config)#show mac-address port epon 0/0/1 ----------------------------------------------------------------------------- Total:1 ----------------------------------------------------------------------------- MAC VLAN Port MAC-Type ----------------------------------------------------------------------------- E0:67:B3:0D:0E:01 1 pon0/0/1 dynamic ----------------------------------------------------------------------------- OLT(config)#

Show MAC Address Table From GE Port

| Command | OLT(config)#show mac-address port ge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to show the mac address learned by GE port. |
| <F/S/P> | FrameID/SlotID/PortID,range for<0-0>/<0-0>/<1-8> |

【Example】

Example 1：Show the mac address learned by GE5

OLT(config)#show mac-address port ge 0/0/5 ----------------------------------------------------------------------------- Total:9 ----------------------------------------------------------------------------- MAC VLAN Port MAC-Type ----------------------------------------------------------------------------- 3C:95:09:4F:30:D1 1 ge0/0/5 dynamic 84:5B:12:66:C0:E2 1 ge0/0/5 dynamic 02:02:5C:6E:0F:17 1 ge0/0/5 dynamic E0:67:B3:46:50:DD 1 ge0/0/5 dynamic 00:E0:FC:09:BC:F9 1 ge0/0/5 dynamic B8:81:98:78:36:10 1 ge0/0/5 dynamic 00:DB:DF:9C:FA:0E 1 ge0/0/5 dynamic 00:0A:C2:22:B0:9D 1 ge0/0/5 dynamic 6C:3B:6B:32:83:1C 1 ge0/0/5 dynamic ----------------------------------------------------------------------------- OLT(config)#

Show MAC Address Table From XGE Port

| Command | OLT(config)#show mac-address port xge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to show the mac address learned by XGE port. |
| <F/S/P> | FrameID/SlotID/PortID，<0-0>/<0-0>/<1-2> |

【Example】

Example 1：Show mac address learned by XGE1.

OLT(config)#show mac-address port xge 0/0/1 There is not any MAC address record! OLT(config)#

Show MAC Address Table From Aggregation Group

| Command | OLT(config)#show mac-address port lag {<manual group id>¦<Lacp group id>} |
| --- | --- |
| View | Config view |
| Description | This command is used to show mac address learned by port aggregation group. |
| {<Manual group id>¦<Lacp group id>} | manual-group-ID,range for 1-8 lacp-group-ID,range for 9-16 |

【Example】

Example 1：Show mac address learned by port link aggregation group 1.

OLT(config)#show mac-address port lag 1 There is not any MAC address record! OLT(config)#

Show OLT Static MAC Address Tables

| Command | OLT(config)#show mac-address static |
| --- | --- |
| View | Config view |
| Description | This command is used to show all the static mac address of olt. |

【Example】

Example 1：Show all the static mac address of olt.

OLT(config)#show mac-address static ----------------------------------------------------------------------------- Total:3 ----------------------------------------------------------------------------- MAC VLAN Port MAC-Type ----------------------------------------------------------------------------- E0:56:43:A9:B4:1A 100 cpu static E0:56:43:A9:B4:1A 200 cpu static E0:56:43:A9:B4:1A 1000 cpu static ----------------------------------------------------------------------------- OLT(config)#

Show OLT MAC Address Aging Time Configuration

| Command | OLT(config)#show mac-address timer |
| --- | --- |
| View | Config view |
| Description | This command is used to show the mac address aging time of OLT. |

【Example】

Example 1：Show mac address aging time of olt.

OLT(config)#show mac-address timer MAC aging time:300s OLT(config)#

Show MAC Address Table From Specified Vlan

| Command | OLT(config)#show mac-address vlan <vlan-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to show the mac address of specified vlan. |
| <vlan-id> | VLAN ID to be show,range for 1-4094 |

【Example】

Example 1：Show mac address of vlan 100.

OLT(config)#show mac-address vlan 100 ----------------------------------------------------------------------------- Total:2 ----------------------------------------------------------------------------- MAC VLAN Port MAC-Type ----------------------------------------------------------------------------- E0:56:43:A9:B4:1A 100 cpu static 00:12:13:23:45:32 100 cpu blackhole ----------------------------------------------------------------------------- OLT(config)#

OLT Global VLAN Configurationss

OLT VLAN Basic Configuration

Create OLT VLAN or VLAN List

| Command | OLT(config)#vlan vlan-list |
| --- | --- |
| View | Config view |
| Description | This command is used to create a vlan or a vlan list. |
| <vlan-list> | ID of vlan,range for 1-4094 |

【Example】

Example 1：Create vlan 100.

OLT(config)#vlan 100 Create vlan successfully:100 OLT(config)#

Example 2:Create a vlan list 110-120.

OLT(config)#vlan 110-120 Create vlan successfully:110-120 OLT(config)#

Detele OLT VLAN or VLAN List

| Command | OLT(config)#no vlan <vlan-list> |
| --- | --- |
| View | Config view |
| Description | This command is used to delete one or batch of vlan |
| <vlan-list> | VLAN id to be delete,range for 1-4094 |

【Example】

Example 1：Delete vlan 100.

OLT(config)#no vlan 100 Delete vlan successfully:100 OLT(config)#

Example 2:Delete vlan list 110-120.

OLT(config)#no vlan 110-120 Delete vlan successfully:110-120 OLT(config)#

Config OLT VLAN or VLAN List Name

| Command | OLT(config)#vlan-name <vlan-list> < vlan-name> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the vlan name. |
| <vlan-name> | Vlan name,length for 1-17 letters |

【Example】

Example 1：Set the name of vlan 100 as test.

OLT(config)#vlan-name 100 test OLT(config)#

Example 2:Set the name of vlan list 100-120 as test.

OLT(config)#vlan-name 100-120 test OLT(config)#

Delete VLAN or VLAN List Name

| Command | OLT(config)#no vlan-name <vlan-list> <vlan-name> |
| --- | --- |
| View | Config view |
| Description | This command is used to delete the name of vlan. |
| <vlan-list> | VLAN id to be set,range for 1-4094 |

【Example】

Example 1：Delete the name of vlan 100.

OLT(config)#no vlan-name 100 OLT(config)#

Example 2:Delete the name of vlan list 100-120.

OLT(config)#no vlan-name 100-120 OLT(config)#

Show OLT VLAN Configuration

| Coammand | OLT(config)#show vlan {all/<vlan-list>} |
| --- | --- |
| View | Config view |
| Description | This command is used to show vlan info. |
| <vlan-list> | VLAN id to be show,range for 1-4094 |

【Example】

Example 1：Show the info of vlan 100.

OLT(config)#show vlan 100 ----------------------------------------------------------------------------- Vlan-ID:100 Vlan-Name:test Untagged-Ports:- Tagged-Ports:- ----------------------------------------------------------------------------- OLT(config)#

Example 2:Show info of all the vlan.

OLT(config)#show vlan all ----------------------------------------------------------------------------- Vlan-ID:1 Vlan-Name:vlan1 Untagged-Ports: ge0/0/1 ge0/0/2 ge0/0/3 ge0/0/4 ge0/0/5 ge0/0/6 ge0/0/7 ge0/0/8 xge0/0/1 xge0/0/2 pon0/0/1 pon0/0/2 pon0/0/3 pon0/0/4 pon0/0/5 pon0/0/6 pon0/0/7 pon0/0/8 pon0/0/9 pon0/0/10 pon0/0/11 pon0/0/12 pon0/0/13 pon0/0/14 pon0/0/15 pon0/0/16 lag1 lag2 lag3 lag4 lag5 lag6 lag7 lag8 lag1 lag2 lag3 lag4 lag5 lag6 lag7 lag8 Tagged-Ports:- ----------------------------------------------------------------------------- Vlan-ID:2 Vlan-Name:vlan2 Untagged-Ports:- Tagged-Ports:-

Show OLT VLAN Translate Configuraiton

| Command | OLT(config)#show vlan translate all |
| --- | --- |
| View | Config view |
| Description | This command is used to show vlan translating list. |

【Example】

Example 1：Show translating list of vlan 100.

OLT(config)#show vlan translate all ----------------------------------------------------------------------------- index port oldvlan newvlan priority mode ----------------------------------------------------------------------------- 1 pon0/0/5 800 800--Translate 2 pon0/0/14 2000-2124 38 0 QinQ 3 pon0/0/1 1000 37 0 QinQ 4 pon0/0/14 1000 37 0 QinQ ----------------------------------------------------------------------------- OLT(config)#

Vlanif Configuration

Create or Delete Vlanif Interface

| Command | OLT(config)#{no} interface vlanif <vlan-list> |
| --- | --- |
| View | Config view |
| Description | This command is used to create or delete a vlanif interface |
| <vlan-list> | VLAN id to be set,range for 1-4094 |

【Example】

Example 1：Create a interface vlanif 100 and enter the configure view,the precondition is that the vlan 100 had been created.

OLT(config)#interface vlanif 100 OLT(config-interface-vlanif-100)#

Example 2:Delete the vlanif 100.

OLT(config)#no interface vlanif 100 OLT(config)#

Create or Delete Vlanif IP Address

| Command | OLT(config-interface-vlanif-100)#{no} ip address <Ip address> {<IP address mask¦length-mask>} |
| --- | --- |
| View | Vlanif interface |
| Description | This command is used to create or delete the ip address of vlanif interface |
| <Ip address> | IP address of vlanif,format for X.X.X.X |
| <IP address mask> | IP address mask of vlanif interface,format for X.X.X.X |
| <length-mask> | Length of net mask,range for 0-32 |

【Example】

Example 1：Set the ip address and net mask of vlanif interface as 192.168.1.100 and

255.255.255.0

OLT(config-interface-vlanif-100)#ip address 192.168.1.100 255.255.255.0 OLT(config-interface-vlanif-100)#

Example 2:Delete the ip address of vlanif.

OLT(config-interface-vlanif-100)#no ip address OLT(config-interface-vlanif-100)#

Config Vlanif Interface Description

| Command | OLT(config-interface-vlanif-100)#description <description information> |
| --- | --- |
| View | VLANIF view |
| Description | This command is used to set the description of vlanif interface |
| <description information> | The description of vlanif,length for 1-128 |

【Example】

Example 1：Set the description of vlanif 100 as test.

OLT(config)#interface vlanif 100 OLT(config-interface-vlanif-100)#description test Set interface description successfully! OLT(config-interface-vlanif-100)#

Show Vlanif Interface Detail Information

| Command | OLT(config)#show interface vlanif <vlan-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to show the detail info of one or whole vlanif interface |
| <vlan-id> | VLAN ID to be show |

【Example】

Example 1：Show the info of vlanif 100

OLT(config)#show interface vlanif 100 Description:Inband interface vlanif100 The Maximum Transmit Unit is 1500 bytes Internet Address is 192.168.1.100,netmask 255.255.255.0 Hardware address is E0:56:43:A9:B4:1A Recive 0 packets,0 bytes Transmit 0 packets,0 bytes OLT(config)#

Example 2:Show the info of whole vlanif interface.

OLT(config)#show interface vlanif Description:Inband interface vlanif100 The Maximum Transmit Unit is 1500 bytes Internet Address is 192.168.1.100,netmask 255.255.255.0 Hardware address is E0:56:43:A9:B4:1A Recive 0 packets,0 bytes Transmit 0 packets,0 bytes Description:Inband interface vlanif1000 The Maximum Transmit Unit is 1500 bytes Internet Address is 192.168.2.100,netmask 255.255.255.0 Hardware address is E0:56:43:A9:B4:1A Recive 0 packets,0 bytes Transmit 0 packets,0 bytes OLT(config)#

VLAN Policy Configuration

Add VLAN Policy Based On Mac address

| Command | OLT(config)#mac-vlan <mac-address> <vlan-id> <priority> |
| --- | --- |
| View | Config view |
| Description | This command is used to add mac-vlan,when the ingress message of olt is untagged and the destination mac is equal to the setting mac too,this message will be added a corresponding vlan and priority label. Delete mac-vlan:no mac-vlan<mac-address>/all |
| <mac-address> | Mac address,format for xx.xx.xx.xx.xx.xx。 |
| <vlan-id> | VLAN id,range for 1-4094. |
| <priority> | priority |

【Example】

Example 1：Add mac-vlan 100 and priority 0 to mac address 13:20:12:08:97:23

OLT(config)#mac-vlan 13:20:12:08:97:23 100 0 OLT(config)#

Show MAC-VLAN Entry

| Command | OLT(config)#show mac-vlan all |
| --- | --- |
| View | Config view |
| Description | This command is used to show mac-vlan entry |

【Example】

Example 1：Show mac-vlan entry

OLT(config)#show mac-vlan all ----------------------------------------------------------------------------- index mac-address vlan priority 1 13:20:12:08:97:23 100 0 OLT(config)#

Add VLAN Policy Based On IP Address

| Command | OLT(config)#ip-subnet-vlan <ip-addr> {length-mask¦mask} <vlan-id> <priority> |
| --- | --- |
| View | Config view |
| Description | This command is used to add ip-subnet-vlan,when the ingress message of olt is untagged and the destination ip address is equal to the setting ip address too,this message will be added a corresponding vlan and priority label. Delete ip-subnet-vlan entry:no ip-subnet-vlan<ip address><length>/<mask> |
| <ip-addr> | IP address,format for x.x.x.x |
| <length-mask¦mask> | length of net mask,range for 0-32 Mask:net mask,format for x.x.x.x |
| <vlan-id> | VLAN id,range for 1-4094 |
| <priority> | VLAN priority |

【Example】

Example 1：Create an ip-subnet-vlan,set ip address as 192.168.5.34,net mask length for 24,vlan for 100,priority for 0.

OLT(config)#ip-subnet-vlan 192.168.5.34 24 100 0 OLT(config)#

Show IP-Subnet-VLAN Entry

| Command | OLT(config)#show ip-subnet-vlan all |
| --- | --- |
| View | Config view |
| Description | This command is used to show the entry of ip-subnet-vlan |

【Example】

Example 1：Show all the entry of ip-subnet-vlan.

OLT(config)#show ip-subnet-vlan all ----------------------------------------------------------------------------- ip-address netmask vlan priority 192.168.5.0 255.255.255.0 100 0 ----------------------------------------------------------------------------- OLT(config)#

Add VLAN Policy Based On Protocol

| Command | OLT(config)#protocol-vlan <protocol-index> {at¦ipv4¦ipv6¦} {<ethernetii¦snap} OLT(config)#protocol-vlan <protocol-index> <ipx> {ethernetii¦snap¦llc¦snap} OLT(config)#protocol-vlan <protocol-index> mode {ethernetii¦snap} etype <ethertype id> |
| --- | --- |
| View | Config view |
| Description | This command is used to create protocol-vlan.Protocol-vlan can be bound to GE port,pon port,xge port,it is used to protocol translation for switch data. Delete the protocol vlan:no protocol-vlan<1-16>/all |
| <protocol-index> | protocol-vlan index,range for 1-16 |
| <parameter> | at:appletalk protocol ipv4:IPv4 protocol ipv6:IPv6 protocol Ipx:IPx protocol ethernetii:Type of Ethernet protocol snap:Type of snap protocol llc:Type of llc protocol raw:Type of raw protocol etype:ethertype |
| <ethertype id> | The number of ethertype,range for 0x0001-0xffff |

【Example】

Example 1：Create a protocol-vlan which index is 1,mode is ethernetii and bind it to GE1 and vlan 100.

OLT(config)#protocol-vlan 1 mode ethernetii etype 0x8100 OLT(config)#interface ge 0/0 OLT(config-interface-ge-0/0)#protocol-vlan 1 add port 1 100 OLT(config-interface-ge-0/0)#

Show Protocol-vlan Entry

| Command | OLT(config)#show protocol-vlan all |
| --- | --- |
| View | Config view |
| Description | This command is used to show all the protocol-vlan entry. |

【Example】

Example 1：Show all the protocol-vlan entry.

OLT(config)#show protocol-vlan all ----------------------------------------------------------------------------- index frame ethtype port vlan id 1 ethii unknow(0x8100)ge0/0/1 100 ----------------------------------------------------------------------------- OLT(config)#

OLT IGMP Configuration

Config IGMP Fast-leave Function

| Command | OLT(config)#igmp fast-leave {on¦off} |
| --- | --- |
| View | Config view |
| Description | igmp fast-leave off: Close igmp-snooping igmp fast-leave function.After executing this command,after the ont receiving igmp leave message of user,it needs to send specific group query message to assure whether the user is online,if the group query message has timeout,but ont still has not receive the user report message,ont will infer the user has offline and renew the local multicast table entry.When the user needn’t cut the channel fastly,using this parameter. igmp fast-leave on: Open igmp-snooping igmp fast-leave function.After executing this command,after the ont receiving igmp leave message of user,ont renew the local multicast table immediately according to the igmp leave message with no need of sending specific group query message to assure whether the user has offline.When the user needs to cut the channel fastly,using this parameter. |
| {on¦off} | Off:Close igmp-snooping igmp fast-leave function of olt On:Open igmp-snooping igmp fast-leave function of olt |

【Example】

Example 1：Open igmp-snooping igmp fast-leave function of olt

OLT(config)#igmp fast-leave on OLT(config)#

Config IGMP Mode

| Command | OLT(config)#igmp mode {ctc¦snooping¦proxy¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to set the mode of igmp |
| {ctc¦snooping¦proxy¦disable} | igmp mode ctc: Ctc is a controlled multicast mode.traditional multicast service is uncontrollable,user can join in a multicast group by sending igmp report message,then it can receive the multicast message of the multicast group.Its core idea is to set the limits of authority for the user's accessing to a multicast group.when the user request to join in a multicast group,olt must authenticate to this request,and reject to illegal or ultra vires request.Ctc mode of olt reaches its goal by intercepting the igmp report message which is sent by ont to control the creation of second layer multicast transmission table.After the olt receiving report message of multicast user,olt finds the user’s using authority template according to the user vlan.if the multicast group is not included in the using authority template,olt will intercept this report message and do not generate transmission table,thus the user can not receive the multicast data stream;.if the multicast group is included in the using authority template list,and if the list joins in template by view mode,the report message is permitted to pass.if the list joins in template by preview mode,the report message can pass too,but the olt will activate a timer,when it is timeout,the multicast group transmission table will be deleted,the igmp report message will be intercept,the preview function is realized by this way. Otherwise,ont multicast mode must be set as ctc mode,the command is“ont multicast-mode”,in this mode,when the multicast user has passed the authentication,olt will issue the corresponding extension oam message to maintain the multicast table entry of ont to realize the controlling of multicast service. igmp mode snooping Set the igmp mode of multicast vlan as IGMP snooping.IGMP snooping obtains relevant info to maintain the multicast transmission table by monitoring the communication between the user and multicast router.system does not make any process to multicast message of this multicast vlan,just transparent it. igmp mode proxy: Set the igmp mode of multicast vlan as IGMP proxy.igmp proxy intercepts the igmp message between user and multicast router and proceeds coherent processing,then transmits it to the upper multicast router.From the view of user,the system is equivalent to multicast server;From the view of upper device,the system is equivalent to multicast user.IGMP proxy mode degrees the multicast protocol message traffic in the network. igmp mode disable:Close the multicast function |

【Example】

Example 1：Set igmp mode as proxy.

OLT(config)#igmp mode proxy OLT(config)#

Config IGMP Proxy Parameter

| Command | OLT(config)#igmp proxy {gen_interval¦gen_response¦robustness¦source-ip¦sp_count¦sp_interval¦sp_response} |
| --- | --- |
| View | Config view |
| Description | This command is used to set parameter of IGMP Proxy |
| <gen_interval¦gen_response¦robustness¦source-ip¦sp_count¦sp_interval¦sp_response> | gen_interval-General query interval This command is used to set general query interval.system send the gen_interval aiming at all program to assure whether the user is watching a program.If the system has not receive the report message of user,it will be regarded as that there is no user watching this program,and this program data stream will be stopped.it can avoid the bandwidth waste from that the user does not watch program but still receiving the multicast stream. gen_response-General query max response time//This command is used to set general query max response time. robustness-Robustness keyword This command is used to set robustness keyword.According to network stability variation,the user hopes to adjust to robustness keyword,using this command.System uses this robustness keyword to assure the aging time of multicast user after setting.Robustness keyword is a coefficient which is used to enhance the robustness of the system,it directly effects to the length of multicast user aging time,in addiction,it effects to the frequency of gen_interval message.If a subnet is likely to occur to packet loss,robustness keyword should be increase to ensure the stability of multicast user. source-ip-Source ip of igmp proxy message This command is used to set the source ip of gen_interval or specific group query message which is sent to user by igmp router.If this ip does not be assigned,system will send the gen_interval query message or specific group message with the default ip. sp_count-igmp specific query count This command is used to set the query counts of igmp specific query.System aims at a specific program and sends N(N is set by this command)times specific igmp query message to assure whether the user is watching this program,if there is no user’s feedback report message,system will regard that no user is watching this program and the system will not send the program data to user,it can avoid that the user does not watching this program but still receiving the multicast stream,it prevents system from bandwidth waste. sp_interval-Specific query interval This command is used to set specific query interval.system send the specific query message aiming at a specified program to assure whether the user is watching a program.If the system has not receive the report message of user,it will be regarded as that there is no user watching this program,and this program data stream will be stopped.it can avoid the bandwidth waste from that the user does not watch program but still receiving the multicast stream. sp_response-Specific query max response time |

【Example】

Example 1：Set igmp proxy specific query count as 1,gen_interval as 250s,gen_response as 10,robustness as 2

OLT(config)#igmp proxy sp_count 1 OLT(config)#igmp proxy gen_interval 250 OLT(config)#igmp proxy gen_response 10 OLT(config)#igmp proxy robustness 2 OLT(config)#

Config IGMP Forwarding Policy

| Command | OLT(config)#igmp policy {discard¦pass} |
| --- | --- |
| View | Config view |
| Description | This command is used to set transmission policy of igmp message |
| {discard¦pass} | Discard:Discard the unknown igmp protocol message Pass:the igmp protocol message is set as transparent |

【Example】

Example 1：Discard the unknown igmp protocol message

OLT(config)#igmp policy discard OLT(config)#

Show IGMP Configuration

| Command | OLT(config)#show igmp config |
| --- | --- |
| View | Config view,btv view,multicast vlan view |
| Description | This command is used to show igmp config.including igmp state,fast leave state,General query max response time(s),query interval,source ip and etc. |

【Example】

Example 1：Show igmp config.

OLT(config)#show igmp config ----------------------------------------------------------------------------- Global config: Igmp mode:Proxy Igmp policy:Discard Fast leave:Off ----------------------------------------------------------------------------- Proxy config: Robustness count:2 General query max response time(s):10 General query interval(s):125 Specific query interval(ms):1000 Specific query count:2 Specific query max response time(ms):800 Source ip of the proxy:192.168.1.253 ----------------------------------------------------------------------------- OLT(config)#

Show IGMP Forwarding Table

| Command | OLT(config)#show igmp group {all¦ip-address¦vlan} |
| --- | --- |
| View | Config view,btv view,multicast vlan view |
| Description | This command is used to show igmp group. |
| {all¦ip-address¦vlan} | All:show all the igmp group. vlan-id:show igmp group of specified vlan id ip-address:show igmp group of specified channel ip address |

【Example】

Example 1：Show all the igmp group.

OLT(config)#show igmp group all ERROR:There is not any group address record. OLT(config)#

Config Multicast VLAN

| Command | OLT(config)#multicast-vlan <vlan-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to create multicast vlan and enter multicast vlan mode,”no”command is used to delete it.Multicast vlan is a kind of vlan application,in this mode,user can set relevant parameters of multicast. |
| <vlan-id> | Multicast vlan id.only after the corresponding vlan is created the multicast vlan can be used. |

【Example】

Example 1：Create multicast vlan 100 and enter enter multicast vlan mode.

OLT(config)#multicast-vlan 100 OLT(multicast-vlan-100)#

Config IGMP Match Group

| Command | OLT(config)#igmp match group ip <start-ip> to-ip <end-ip> |
| --- | --- |
| View | multicast-vlan view |
| Description | This command is used to set dynamic program library,this range ip address makes up a channel group,it can set limits of authority for these channel in btv mode. |
| <start-ip> | Match start ip,it must be multicast ip address |
| <end-ip> | Match end ip,it must be multicast ip address |

【Example】

Example 1：Set a dynamic program library:224.1.1.1-224.2.2.2

OLT(config-multicast-vlan-100)#igmp match group ip 224.1.1.1 to-ip 224.2.2.2 OLT(config-multicast-vlan-100)#

Show IGMP Match Group

| Command | OLT(config)#show igmp match group {vlan-id¦all} |
| --- | --- |
| View | multicast-vlan view,config view |
| Description | This command is used to show igmp match group |
| {vlan-id¦all} | vlan-id:multicast vlan id all:all of the multicast vlan |

【Example】

Example 1：Show igmp match group vlan 100

OLT(config)#show igmp match group vlan 100 Total Match Group:1 ----------------------------------------------------------------------------- MVlan Match Mode Program 100 disable 224.1.1.1-224.2.2.2 ----------------------------------------------------------------------------- OLT(config)#

Add IGMP Program

| Command | OLT(config-multicast-vlan-100)#igmp program add program-index <Program-index> ip <ip-addr> |
| --- | --- |
| View | multicast-vlan view |
| Description | This command is used to add static multicast program.it needs to pre-allocation multicast program library,The authorized user can view or preview the program in the specified multicast vlan. |
| <Program-index> | Multicast program index |
| <ip-addr> | multicasst ip address,format is X.X.X.X |

【Example】

Example 1：Set static multicast program,and binds it to program index 2.

OLT(config-multicast-vlan-100)#igmp program add program-index 2 ip 239.1.1.1 OLT(config-multicast-vlan-100)#

Add Batch IGMP Program

| Command | OLT(config-multicast-vlan-100)#igmp program add program-index <Program-index> batch ip <ip-addr> to-ip <ip-addr> |
| --- | --- |
| View | multicast-vlan view |
| Description | This command is used to add static multicast program.If igmp match mode is enabled,it needs to pre-allocation multicast program library,The authorized user can view or preview the program in the specified multicast vlan. |
| <Program-index> | Igmp program index |
| <ip-addr> | It refers to the beginning and ending igmp IP,forming an igmp range. |

【Example】

Example 1：It configures batch static igmp program from 224.1.1.1 to 224.1.1.1.3,and binding program-index 3.

OLT(config-multicast-vlan-100)#igmp program add program-index 3 batch ip 224.1.1.1 to-ip 224.1.1.3 OLT(config-multicast-vlan-100)#

Delete IGMP Program

| Command | OLT(config-multicast-vlan-100)#igmp program delete {all¦program-index<Program-index>} |
| --- | --- |
| View | multicast-vlan view |
| Description | This command is used to delete igmp program.When it doesn’t want users to watch multicast program,using this command to delete igmp program from program database.Once deleting one program,users will not watch this program. |
| <Program-index> | igmp program index |
| <all> | all igmp programs |

【Example】

Example 1：Delete static igmp program-index 2

OLT(config-multicast-vlan-100)#igmp program delete program-index 2 OLT(config-multicast-vlan-100)#

Config IGMP Router-port

| Command | OLT(config-multicast-vlan-100)#igmp router-port <port-id> |
| --- | --- |
| View | multicast-vlan view |
| Description | This command is used to configure igmp router-port.For ctc mode,users need to configure router-port to realize the below devices forwarding. |
| <port-id> | ge/xge port number |

【Example】

Example 1：configure ge5 port as router-port

OLT(config-multicast-vlan-100)#igmp router-port ge 0/0/5 OLT(config-multicast-vlan-100)#

Show IGMP Router-port

| Command | OLT(config-multicast-vlan-100)#show igmp router-port vlan <vlan-id> |
| --- | --- |
| View | multicast-vlan view |
| Description | This command is used to view IGMP router-port |
| <vlan-id> | Multicast-vlan id |

【Example】

Example 1：View multicast-vlan 100 router-port

OLT(config-multicast-vlan-100)#show igmp router-port vlan 100 VID:100 Router:ge0/0/5 OLT(config-multicast-vlan-100)#

Configure Unknow-multicast Forwarding Policy

| Command | OLT(config-multicast-vlan-100)#igmp multicast-unknown policy {discard¦transparent} |
| --- | --- |
| View | multicast-vlan view |
| Description | This command is used to configure multicast-unknown service flow suppression policy.If service flow carriers special-purpose multicast-unknown,thus configuring transparent.No special-purpose multicast-unknown occupies bandwidth,thus setting discard. |
| {discard¦transparent} | Discard:System discard received multicast-unknown service flow Transparent:System transparent received multicast-unknown service flow. |

【Example】

Example 1：Configure multicast-unknown policy as discard.

OLT(config-multicast-vlan-100)#igmp multicast-unknown policy discard OLT(config-multicast-vlan-100)#

Show Unknow-multicast Forwarding Policy

| Command | OLT(config-multicast-vlan-100)#show igmp multicast-unknown policy vlan <vlan-id> |
| --- | --- |
| View | multicast-vlan view |
| Description | This command is used to view multicast-unknown service flow suppress policy. |
| <vlan-id> | Multicast-vlan id |

【Example】

Example 1：View multicast-unknown service flow suppression policy

OLT(config-multicast-vlan-100)#show igmp multicast-unknown policy vlan 100 Unknown multicast policy is discard. OLT(config-multicast-vlan-100)#

Add IGMP User（Member） in Multicast-vlan

| Command | OLT(config-multicast-vlan-100)#igmp member user-index <User-index> |
| --- | --- |
| View | Multicast-vlan view |
| Description | This command is used to add igmp user to multicast-vlan |
| <User-index> | configured igmp user |

【Example】

Example 1：Add igmp user 1 to multicast-vlan 100

OLT(config-multicast-vlan-100)#igmp member user-index 1 OLT(config-multicast-vlan-100)#

Show Multicast-vlan Member Configuration

| Command | OLT(config-multicast-vlan-100)#show igmp multicast-vlan-member {all¦vlan-id} |
| --- | --- |
| View | Config view and multicast-vlan view |
| Description | This command is used to view multicast-vlan member |
| <all¦vlan-id> | all:view all vlan-id:view the specified vlan-id member |

【Example】

Example 1：View all multicast-vlan member

OLT(config-multicast-vlan-100)#show igmp multicast-vlan-member all Total Mvlan Member:1 ----------------------------------------------------------------------------- User-Index Port ONUId Vlan Authority Mvlan Max-program 1 pon0/0/1 1 100 no-auth 100 8 ----------------------------------------------------------------------------- OLT(config-multicast-vlan-100)#

Config IGMP User Bind Rights Profile

| Command | OLT(config-btv)#igmp control {bind/delete} user-index <user-index> profile-index <profile-index> |
| --- | --- |
| View | btv view |
| Description | This command is used to bind igmp user and rights profile. |
| <User-index> | configured user group |
| <profile-index> | Configured rights profile |

【Example】

Example 1：bind igmp user 1 and rights profile 1

OLT(config-btv)#igmp control bind user-index 1 profile-index 1 OLT(config-btv)#

STP Configuration

Global STP Config

| Command | OLT(config)#spanning-tree {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to on/off the global STP protocol.All the configuration about STP protocol will make effect only if the STP is enabled in global mode and port. |
| {enable¦disable} | Enable:Enable global STP protocol Disable:Disable global STP protocol |

【Example】

Example 1：Enable STP in global mode.

OLT(config)#spanning-tree enable OLT(config)#

Show STP Configuraiton

| Command | OLT(config)#show spanning-tree info |
| --- | --- |
| View | Config view |
| Description | This command is used to show configuration info of STP |

【Example】

case1:Show configuration info of STP

OLT(config)#show spanning-tree info ----------------------------------------------------------------------------- RSTP switch status:Enable Bridge ID[PRI-MAC]:32768-e0:56:43:a9:b4:1a Root Bridge ID[PRI-MAC]:32768-e0:56:43:a9:b4:1a Bridge max age(s):20 Bridge hello time(s):2 Bridge forward delay(s):15 Transmit Hold Count:3 Root Path Cost:0 ----------------------------------------------------------------------------- Port Status: ----------------------------------------------------------------------------- Port Priority Path Edge Link Role State Cost Status Type ----------------------------------------------------------------------------- ge0/0/1 128 20000 NEdge P2P Unknown Down ge0/0/2 128 20000 NEdge P2P Unknown Down ge0/0/3 128 20000 NEdge P2P Unknown Down ge0/0/4 128 20000 NEdge P2P Unknown Down ge0/0/5 128 20000 NEdge P2P Unknown Down ge0/0/6 128 200000 NEdge P2P Designated Forwarding ge0/0/7 128 20000 NEdge P2P Unknown Down ge0/0/8 128 20000 NEdge P2P Unknown Down xge0/0/1 128 2000 NEdge P2P Unknown Down xge0/0/2 128 2000 NEdge P2P Unknown Down lag1 128 20000 NEdge P2P Unknown Down lag2 128 20000 NEdge P2P Unknown Down lag3 128 20000 NEdge P2P Unknown Down lag4 128 20000 NEdge P2P Unknown Down lag5 128 20000 NEdge P2P Unknown Down lag6 128 20000 NEdge P2P Unknown Down lag7 128 20000 NEdge P2P Unknown Down lag8 128 20000 NEdge P2P Unknown Down lagL9 128 20000 NEdge P2P Unknown Down lagL10 128 20000 NEdge P2P Unknown Down lagL11 128 20000 NEdge P2P Unknown Down lagL12 128 20000 NEdge P2P Unknown Down lagL13 128 20000 NEdge P2P Unknown Down lagL14 128 20000 NEdge P2P Unknown Down lagL15 128 20000 NEdge P2P Unknown Down lagL16 128 20000 NEdge P2P Unknown Down ----------------------------------------------------------------------------- OLT(config)#

Config STP Priority

| Command | OLT(config)#spanning-tree priority <rstp bridge priority> |
| --- | --- |
| View | Config view |
| Description | This command is used to set priority of specified spanning tree for the device.Whether the device will be select as root bridge depends on the priority of device.When it needs to specify a device as root bridge,use this command to set the priority of device.whether the device will be selected as root bridge of spanning tree deciding by the priority value. |
| <rstp bridge priority> | Priority of device.Step size for 4096.The smaller the priority is,the higher class the device. Range for 0-61440,Step size for 4096. |

【Example】

Example 1：Set the priority of spanning tree as 4096.

OLT(config)#spanning-tree priority 4096 OLT(config)#

Config STP Bridge Forward-delay Time

| Command | OLT(config)#spanning-tree timer forward-delay <timer> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the device forward-delay time of spanning tree.To prevent the device from occurring to temporary loop,it needs to wait for sometime in device status migrating.After setting the forward delay time,status migrate according to this time interval.Range for 4-30s. |
| <timer> | Status migrate time interval.it relates to the switch network diameter.generally,the bigger the diameter is,the longer forward delay time should be set.Range for 4-30s.The default value is 15s. |

【Example】

Example 1：Set the forward delay time as 20s.

OLT(config)#spanning-tree timer forward-delay 20 OLT(config)#

Config STP Bridge Hello Message Send Period Time

| Command | OLT(config)#spanning-tree timer hello <time> |
| --- | --- |
| View | Config view |
| Description | This command is used to set transmission period of the spanning tree hello time.Network bridge sends the hello message within a certain time interval,it is used to affirm whether the link is normal.After setting,device will send the hello message to neighbor Switch according to corresponding time interval. |
| <time> | Hello time interval.The appropriate hello time interval can ensure that the switch can find the link fault in the network in time without taking up too much network resources.Range for 1-2s,the default is 2s. |

【Example】

Example 1：Set the hello time’s transmission period as 1s.

OLT(config)#spanning-tree timer hello 1 OLT(config)#

Config STP Bridge Max-age Time

| Command | OLT(config)#spanning-tree timer max-age <time> |
| --- | --- |
| View | Config view |
| Description | This command is used to set max age time.It is used to estimate whether the message is time out.After setting max age time,the device will discard the time out message.range for 6-40s,the default is 20s. |
| <time> | Max age time |

【Example】

Example 1：Set the max age time as 6s.

OLT(config)#spanning-tree timer max-age 6 OLT(config)#

Config STP BPDU TX Hold Count

| Command | OLT(config)#spanning-tree hold-count <hold-count> |
| --- | --- |
| View | Config view |
| Description | BPDU is the message frame exchanged between the switches that run STP.BPDU includes path and priority info of STP,STP determines the root bridge and root bridge path by BPDU. |
| <hold-count> | BPDU transmission rate,the maximum transmission number of BPDU in each hello time period,range for 1-10,unit is pps. |

【Example】

Example 1：Set the hold-count as 2 pps.

OLT(config)#spanning-tree hold-count 2 OLT(config)#

Config OLT Port STP Cost

| Command | OLT(config)#spanning-tree port <port-id> cost <cost> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the cost of GE port spanning tree.when there are multi links between two devices but not the root port,device determines the optimal path by the cost. |
| <port-id> | Port id to be set |
| <cost> | Cost value,range for 1-200000000 |

【Example】

Example 1：Set the cost of GE1 spanning tree as 2000.

OLT(config)#spanning-tree port ge 0/0/1 cost 2000 OLT(config)#

Config OLT Port STP Edged-port

| Command | OLT(config)#spanning-tree port <port-id> edged-port {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to set spanning tree edged-port of GE port.If user specifies a port as edged-port,then the port can rapidly migrate from blocking status to forwarding status without waiting for delay time.User can only set the port which is connected with terminal as edged-port.The default is not edged-port. |
| <port-id> | Port id to be set |
| {enable¦disable} | enable:Set this port as edged-port disable:Set this port as non-edged-port |

【Example】

Example 1：Set GE1 as edged-port.

OLT(config)#spanning-tree port ge 0/0/1 edged-port enable OLT(config)#

Config OLT Port STP Mcheck Property

| Command | OLT(config)#spanning-tree port <port-id> mcheck |
| --- | --- |
| View | GE view |
| Description | This command is used to set mcheck property of GE port spanning tree.Mcheck property is used to detect whether the device which is running in STP compatible mode can transfer to RSTP mode.We can check whether there are any network bridge existing in the network segment connected with current ethernet port.If it is yes,RSTP protocol will transfer the port protocol running mode to STP mode. |
| <port-id> | Port id to be set,range for 1-8 |

【Example】

Example 1：Set the mheck property of GE1.

OLT(config)#spanning-tree port ge 0/0/1 mcheck OLT(config)#

Config OLT Port STP  Point-to-Poing Function

| Command | OLT(config)#spanning-tree port <port-id> point-to-point {auto/true/false} |
| --- | --- |
| View | Config view |
| Description | This command is used to set P2P function of GE port spanning tree.if the bridge is working in RSTP mode,a couple of port connected by P2P link can migrate to forwarding status by transferring synchronize message which decreases unnecessary transmission delay time.If set it as auto mode,RSTP protocol can detect automatically whether the current ethernet port has connection with P2P link.it can set as manual mode also,but what we suggestion is setting it as auto mode. |
| <port-id> | Port id to be set,range for 1-8 |
| {auto/true/false} | auto:Auto connect to P2P link true:Connect the GE port with P2P link false:Disconnect the GE port with P2P link |

【Example】

Example 1：Set the running mode of GE1 P2P link as true.

OLT(config)#spanning-tree port ge 0/0/1 point-to-point true OLT(config)#

Config OLT Port STP Priority

| Command | OLT(config)#spanning-tree port <port-id> priority <port-priority> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the priority of GE port.It can include specified ethernet port into spanning tree by setting ethernet port priority,generally,the smaller the value is,the higher the port priority is,and this port is more likely to include into spanning tree.If all the ethernet port of network bridge adopts to the same priority value,the priority of ethernet port is depended on port index number. |
| <port-id> | Port id to be set,range for 1-8 |
| <port-priority> | Port priority,range for 0-240,step size for 16.The default value is 128. |

【Example】

Example 1：Set the priority as 160 for GE1 spanning tree.

OLT(config)#spanning-tree port ge 0/0/1 priority 160 OLT(config)#

Show OLT Port STP Configuration

| Command | OLT(config)#show spanning-tree port {ge¦xge} <port-id> |
| --- | --- |
| view | Config view |
| Description | This command is used to show spanning tree configuration info of GE port |
| <port-id> | Port id to be set. |

【Example】

Example 1：Show spanning tree configuration info of GE1.

OLT(config)#show spanning-tree port ge 0/0/1 -----------------------ge0/0/1 RSTP STATUS:-------------------------------- Port STP Mode:RSTP Port Priority:160 Port Path Cost:2000 Port Edge Admin:Edge Port Edge Status:Edge Port Link Type Admin:P2P Port Link Type Status:P2P Port Role:Unknown Port State:Down ----------------------------------------------------------------------------- OLT(config)#

Show OLT Link-aggregation Group STP Configuration

| Command | OLT(config)#show spanning-tree link-aggregation group <group-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to show configuration info of spanning tree link-aggregation group. |
| <group-id> | Group id to be set. |

【Example】

Example 1：show configuration info of spanning tree link-aggregation group1

OLT(config)#show spanning-tree link-aggregation group 1 -----------------------lag1 RSTP STATUS:-------------------------------- Port STP Mode:RSTP Port Priority:128 Port Path Cost:20000 Port Edge Admin:NON-Edge Port Edge Status:NEdge Port Link Type Admin:Auto Port Link Type Status:P2P Port Role:Unknown Port State:Down ----------------------------------------------------------------------------- OLT(config)#

ACL Configuration Manage

ACL Apply Time-range Cofiguration

ACL time range is distributed into relative time and absolute time:

Relative time:Periodicity time,for example,from 8:30 to 18:30 every Monday.

Absolute time:From a specific time to another specific time,for example,from 06/08/2006 12:00am to 08/08/2006 18:00.

Config ACL Apply Relative Time-range

| Command | OLT(config)#time-range <time-name> <start-time> to <end-time> <days> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the time range of relative time |
| <time-name> | Time range name,it is quoted when setting ACL rules. |
| <start-time> | The start time of relative time,format for HH:MM.It determines a time range with the end time,days define the date when the time range take effect,triple parameters determine a time range. |
| <end-time> | The end time of relative time,format for HH:MM.It determines a time range with the start time,days define the date when the time range take effect,triple parameters determine a time range. |

【Example】

Example 1：Set a relative time,names it as worktime,the effective time is from 8:00 to 18:30 every Monday.

OLT(config)#time-range worktime 08:00 to 18:30 mon OLT(config)#

Config ACL Apply Absolute Time-range

| Command | OLT(config)#time-range <time-name> from <start-time> to <end-time> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the time range of absolute time. |
| <time-name> | Name of time range,it is quoted when setting ACL rules. |
| <start-time> | The start time of absolute time,format for HH:MM YYYY/MM/DD |
| <end-time> | The end time of absolute time,format for HH:MM YYYY/MM/DD |

【Example】

Example 1：Set an absolute time,names it as test,the effective is from 5/1/2017 8:40 to 12/7/2017 18:00.

OLT(config)#time-range test from 8:40 2017/5/1 to 18:00 2017/12/7 OLT(config)#

Delete ACL Apply Time-range

| Command | OLT(config)##no time-range {time-name/all} |
| --- | --- |
| View | Config view |
| Description | This command is used to delete the time range that had been set |
| {time-name} | Name of time range,input the name of time range to be deleted,“all”means all the time range that had set. |

【Example】

Example 1：Delete the time range named test

OLT(config)#no time-range test OLT(config)#

Show ACL Apply Time-range Configuration

| Command | OLT(config)##show time-range {time-name/all} |
| --- | --- |
| View | Config view |
| Description | This command is used to show the configured time range |
| {time-name} | Name of time range,input the name of time range to be deleted,“all”means all the time range that had set. |

【Example】

Example 1：Show the time range named test.

OLT(config)#show time-range test Current time is 2017-12-07 11:40 Thurday Time-range:test(Active) From 2017-05-01 08:40 to 2017-12-07 18:00 OLT(config)#

Config OLT Basic ACL Function

| Command | OLT(acl-basic-2000)#rule <rule-id> {permit¦deny} source {ip-address¦any} <sour-wildcard> time-range <name> |
| --- | --- |
| View | basic acl view |
| Description | This command is used to create ACL rule in Acl-basic mode or Acl6-basic mode.when it needs to set rule according to source ip address of message,using this command.We can use packet filter command to filter the message by quoting the ACL rule after setting the acl rule,or uses cooperatively with qos strategy to provide QoS. “no rule”is used to delete acl rule. |
| <rule-id> | Rule id of ACL,the bigger the id is,the higher the priority. |
| {permit¦deny} | Deny:Deny the matched message flow passing Permit:Permit the matched message flow passing |
| {ip-address¦any} | ip-address:The source ip segment in ACL rule. any:Match to all source ip address |
| <sour-wildcard> | wildcard mask:ip address and wildcard mask are used to match the host id,it tells to the device should be match to which bit of an ip address by labeling“0”and“1”.“1”means overlook this bit,“0”means this bit needing to be check. |
| <name> | Set the effective time range of ACL rule |

【Example】

Example 1：In worktime range,specify the GE1 receive the message of 10.10.10.2 only.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range working-day OLT(acl-basic-2000)#rule 1 deny source any time-range working-day OLT(acl-basic-2000)#exit OLT(config)#packet-filter inbound 2000 port ge 0/0 1 OLT(config)#

Config OLT Advanced ACL Function

| Command | OLT(acl-adv-3000)# When the protocol is TCP,the command based ipv4 for: rule <rule-id> {permit¦ deny} tcp {[soure <ip-address> <sour-wildcard>] ¦ any] ¦ [destination <ip-address> <sour-wildcard> ¦ any] ¦ [src-port <port-list>] ¦ [dest-port <port-list>] ¦ [precedence <procedence-value> ] ¦ [dscp <dscp-value>] ¦[time-range <time-range-name>]} When the protocol is UDP,the command based ipv4 for: rule <rule-id> {permit¦deny} udp {[soure <ip-address> <sour-wildcard>] ¦ any] ¦ [destination <ip-address> <sour-wildcard> ¦ any] ¦ [src-port <port-list>] ¦ [dest-port <port-list>] ¦ [precedence <procedence-value> ] ¦ [dscp <dscp-value>] ¦[time-range <time-range-name>]} When the protocol is IP,the command based ipv4 for: rule <rule-id> {permit¦deny} ip {[soure <ip-address> <sour-wildcard>] ¦ any] ¦ [destination <ip-address> <sour-wildcard> ¦ any] ¦ [precedence <procedence-value> ] ¦ [dscp <dscp-value>] ¦[time-range <time-range-name>]} When the protocol is ipinip,the command based ipv4 for: rule <rule-id> {permit¦deny}) ipinip {[soure <ip-address> <sour-wildcard>] ¦ any] ¦ [destination <ip-address> <sour-wildcard> ¦ any] ¦ [precedence <procedence-value> ] ¦ [dscp <dscp-value>] ¦[time-range <time-range-name>]} When the protocol is icmp,the command based ipv4 for: rule <rule-id> id {permit¦deny} icmp {[soure <ip-address> <sour-wildcard>] ¦ any] ¦ [destination <ip-address> <sour-wildcard> ¦ any] ¦ [precedence <procedence-value> ] ¦ [dscp <dscp-value>] ¦[time-range <time-range-name>]} When the protocol is other protocol,the command based ipv4 for: rule <rule-id> {permit¦deny} <protocol-id> {[soure <ip-address> <sour-wildcard>] ¦ any] ¦ [destination <ip-address> <sour-wildcard> ¦ any] ¦ [precedence <procedence-value> ] ¦ [dscp <dscp-value>] ¦[time-range <time-range-name>]} Adjust the rule command format as: rule <rule-id> {down/move/up} Delete or modify the format of command based ipv4 for: no rule <rule-id> {source¦destination¦precedence¦dscp¦src-port¦dest-port¦time-range¦precedence¦all} |
| --- | --- |
| View | adv acl view |
| Description | This command is used to create ACL rule in Acl-adv mode or Acl6-adv mode.when it needs to set rule according to source address/destination address/ip protocol type/of message or aiming at the protocol characteristic,using this command.We can use packet filter command to filter the message by quoting the ACL rule after setting the acl rule,or uses cooperatively with qos strategy to provide QoS. “no rule”is used to delete acl rule. |
| <rule-id> | Rule id of ACL,the bigger the id is,the higher the priority. |
| {permit¦deny} | Deny:Deny the matched message flow passing Permit:Permit the matched message flow passing |
| {ip-address¦any} | ip-address:The source ip segment in ACL rule. any:Match to all source ip address |
| {sour-wildcard} | wildcard mask:ip address and wildcard mask are used to match the host id,it tells to the device should be match to which bit of an ip address by labeling“0”and“1”.“1”means overlook this bit,“0”means this bit needing to be check. |
| <time-range-name> | The effective time range of acl |
| <soure> | Source ip address of acl matched message |
| <destination> | Destination ip address of acl matched message |
| <precedence> | The priority of ip layer matched by acl |
| <dscp> | Differentiated services code point |
| <dest-port> | The destination port of TCP or UDP matched by ACL |
| <src-port> | The source port of TCP or UDP matched by ACL |
| <Ipinip> | ACL matches to ipinip(double ip layer)message.That is to say ip data encapsulation and tunnel,which encapsulates ip within ip,protocol number is 4,it is the same as its definition in RFC 2003.It describes how to obtain the ip datagram and load to another ip datagram.In mobile IP,the new header specifies how the encapsulated datagram is sent to the forwarding address of the mobile node. |

【Example】

Example 1：Create an advanced ACL and matches it to all icmp message.

OLT(acl-adv-3000)#rule 1 permit icmp OLT(acl-adv-3000)#

Example 2:Delete ACL rule 1.

OLT(acl-adv-3000)#no rule 1 all OLT(acl-adv-3000)#

Config OLT Link ACL Function

| Command | OLT(acl-link-5000)#rule <rule-id> {permit¦deny} {[cos <cos-value> ] ¦ [destination <des-mac-address> <mac-addrmac-wildcard>] ¦ [ source <src-mac-address> <mac-addrmac-wildcard>] ¦ [ inner-cos <inner-cos-value>] ¦ [ vlan <vlan-id>] ¦ [inner-vlan <inner-vlan-id>] ¦ [ type <Ethernet-type>] ¦[time-range <time-range-name>]} |
| --- | --- |
| View | Link acl view |
| Description | This command is used to create ACL rule in Acl-link mode.when it needs to set rule according to link layer info such as source mac address/source VLAN ID/second layer protocol type/destination mac address of message and etc,using this command.We can use packet filter command to filter the message by quoting the ACL rule after setting the acl rule,or uses cooperatively with qos strategy to provide QoS. “no rule”is used to delete acl rule. |
| <rule-id> | Rule id of ACL,the bigger the id is,the higher the priority. |
| {permit¦deny} | Deny:Deny the matched message flow passing Permit:Permit the matched message flow passing |
| <destination> | The destination mac address of message matched by acl |
| <source> | The source mac address of message matched by acl |
| <mac-addr> | Mac address |
| <mac-wildcard> | wildcard mask of mac address:mac address and wildcard mask are used to match the host mac address,it tells to the device should be match to which bit of a mac address by labeling“0”and“1”.“1”means overlook this bit,“0”means this bit needing to be check. |
| <time-range-name> | The effective time range of acl |
| <inner-cos-value> | Match the inner vlan cos value of second layer message |
| <cos-value> | Match the outer vlan cos value of second layer message |
| <vlan-id> | Match the outer vlan id |
| <inner-vlan-id> | Match the inner vlan id |
| <Ethernet-type> | Match ethernet type field |

【Example】

Example 1：In work time range,specifying the GE1 receives the message from destination mac address 22-22-22-22-22 only.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 5000 OLT(acl-link-5000)#rule 2 permit destination 22:22:22:22:22:22 0000-0000-0000 OLT(acl-link-5000)#rule 1deny destination 22:22:22:22:22:22 FFFF-FFFF-FFFF OLT(acl-link-5000)#exit OLT(config)#packet-filter inbound 5000 port ge 0/0 1 OLT(config)#

Config OLT PON ACL Function

| Command | OLT(acl-pon-8000)#{[8021p]¦[dscp]¦[end]¦[eth-type]¦[exit]¦[ip]¦[mac]¦[no]¦[port]¦[protocol]¦[show]¦[tag-num]¦[vlan-id]} |
| --- | --- |
| View | Pon acl view |
| Description | This command is used to create ACL rule in pon-acl mode.when it needs to set rule according to link layer info such as source mac address/source VLAN ID/second layer protocol type/destination mac address of message and etc,using this command.We can use packet filter command to filter the message by quoting the ACL rule after setting the acl rule,or uses cooperatively with qos strategy to provide QoS. “no rule”is used to delete acl rule. |
| [8021p] | IEEE 8021p matching |
| [dscp] | DSCP matching |
| [end] | End current mode and change to view mode |
| [eth-type] | Ethernet type matching |
| [exit] | Exit current mode and down to previous mode |
| [ip] | IP address matching |
| [mac] | MAC address matching |
| [no] | Negate a command or set its defaults |
| [port] | IP protocol port matching |
| [protocol] | IP protocol matching |
| [show] | Show information |
| [tag-num] | VLAN tag number matching |
| [vlan-id] | VLAN ID matching |

【Example】

Example 1：Apply acl 8001 to pon 1 downstream data flow.its function is adding inner vlan 121 for downstream data flow in pon 1 which carries with dst-ip 192.168.1.1 and src-ip 192.168.2.2

OLT(config)#acl 8001 OLT(acl-pon-8001)#ip dst-ip 192.168.1.1 src-ip 192.168.2.2 Exit! OLT(config)#Traffic-modify pon-port 0/0 1 downstream precedence 4 acl 8001 add-inner-vlan 121

Config ONU ACL Function

| Command | OLT(acl-onu-9000)#rule {rule-id} match {[cos]¦[dscp-v4]¦[dscp-v6]¦[dst-ip]¦[dst-mac]¦[dst-port]¦[eth-type]¦[ip-version]¦[protocol]¦[src-ip]¦[src-mac]¦[src-port]¦[vlan-id]} |
| --- | --- |
| View | Onu acl |
| Description | This command is used to create ACL rule in pon-acl mode.when it needs to set rule according to link layer info such as source mac address/source VLAN ID/second layer protocol type/destination mac address of message and etc,using this command.We can use packet filter command to filter the message by quoting the ACL rule after setting the acl rule,or uses cooperatively with qos strategy to provide QoS. “no rule”is used to delete acl rule. |
| <rule-id> | Rule id of ACL,the bigger the id is,the higher the priority. |
| cos | 802.1p priority |
| dscp-v4 | DSCPv4 |
| dscp-v6 | DSCPv6 |
| dst-ip | Destination IP address |
| dst-mac | Destination MAC address |
| dst-port | Destination IP protocol port |
| eth-type | Ethernet type |
| ip-version | IP version |
| protocol | IP protocol |
| src-ip | Source IP address |
| src-mac | Source MAC address |
| src-port | Source IP protocol port |
| vlan-id | VLAN ID |

【Example】

Example 1：Create acl 9000 and set rule 1,apply it to ont 1’s eth 1 in pon 1,its function is mapping the flow which carries with dst-ip 192.168.1.3 to queue 0 and marks its priority as 0.

OLT(config)#acl 9000 ACL ID Create OK! OLT(acl-onu-9000)#rule 1 match dst-ip always-match 192.168.1.3 OLT(acl-onu-9000)#exit OLT(config)#interface epon 0/0 OLT(config-interface-epon-0/0)#ont port classification 1 1 eth 1 acl 9000 rule 1 precedence 4 queue-mapped 0 priority-mark 0 OLT(config)#

Show OLT ACL Configuration

| Command | OLT(config)#show acl {acl-id¦all/detail} |
| --- | --- |
| View | Config view |
| Description | This command is used to show the configuration of acl. |
| <acl-id> | ACL id to be show |
| <all> | Show all the acl |
| <detail> | Detail info |

【Example】

Example 1：Show the configuration fo all acl

OLT(config)#show acl all Basic ACL 2000,0 rules hold Advanced ACL 3000,0 rules hold Link ACL 5000,0 rules hold OLT(config)#

Modify OLT ACL Rule ID

| Command | OLT(configacl-basic-2000)#rule <ruld id> {up/down/move to} |
| --- | --- |
| View | basic acl view,adv acl view,Link acl view |
| Description | acl acl-id down:The rule id is reduced by one without changing the rule content acl acl-id up:Add one for rule id without changing the rule content acl acl-id move to:Modify the value of rule-id to a specify rule id without change the rule content. (the rule which had been bound to the port can not adjust the value of rule id) |
| <ruld id> | Rule id |
| {up/down/move to} | down:The rule id is reduced by one without changing the rule content up:Add one for rule id without changing the rule content move to:Modify the value of rule-id to a specified rule id without change the rule content. |

【Example】

Example 1：Modify rule 1 as rule 3.

OLT(acl-basic-2000)#rule 1 move to 3 OLT(acl-basic-2000)#

OLT QOS Configuraiton

Config OLT Global QOS mode

| Command | OLT(config)#qos global mode {device-based¦port-based} |
| --- | --- |
| View | Config view |
| Description | This command is used to set global QOS mode |
| {device-based¦port-based} | device-based:device-based QOS port-based:port-based QOS |

【Example】

Example 1：Set device-based QOS.

OLT(config)#qos global mode device-based OLT(config)#

Config Traffic Control Based On ACL Rule

Config Packet Filter Based On ACL Rulesss

| Command | OLT(config)#packet-filter {inbound¦outbound} <acl-id> rule-id <rule-id> port {ge¦ pon¦xge} <port-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to match the acl rule for the specified port.When it needs to filter the the flow in port by acl rule,using this command. “no packet-filter......”is used to delete the acl rule in specified port. |
| {inbound¦outbound} | inbound:The ingress flow Outbound:The egress flow |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-id> | Port list |

【Example】

Example 1：In the work time range,specify GE1 can only receive the message from mac address 22-22-22-22-22-22.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range worktime OLT(acl-basic-2000)#rule 1 deny source any time-range worktime OLT(acl-basic-2000)#exit OLT(config)#packet-filter inbound 2000 port ge 0/0 1 OLT(config)#

Changes Traffic DSCP Based On ACL Rule

| Command | OLT(config)#traffic-dscp {inbound¦outbound} <acl-id> rule-id <rule-id> port {ge¦pon¦xge} <port-id> remark-dscp <remark-dscp-value> |
| --- | --- |
| View | Config view |
| Description | This command is used to match the acl for specified port to re-mark the value of DSCP. “no traffic-dscp......”is used cancel the acl in specified port. |
| {inbound¦outbound} | inbound:The ingress flow Outbound:The egress flow |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-id> | Port list |
| <remark-dscp-value> | Remark DSCP value |

【Example】

Example 1：In work timme range,re-mark the DSCP value as 43 for the message from 10.10.10.2.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range worktime OLT(acl-basic-2000)#exit OLT(config)#traffic-dscp inbound 2000 port ge 0/0 1 remark-dscp 43 OLT(config)#

Config Traffic Mirror Based On ACL Rule

| Command | OLT(config)#traffic-mirror inbound <acl-id> rule-id <rule-id> port {ge¦ pon¦xge} <port-id> to {ge¦xge} <port-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to mirror the acl matched flow for specified port. “no traffic-mirror......”is used to cancel the acl matched flow mirror in specified port. |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-id> | Port id |

【Example】

Example 1：In work time range,mirror the message from GE1 10.10.10.2 to the port 2.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range worktime OLT(acl-basic-2000)#exit OLT(config)#traffic-mirror inbound 2000 port ge 0/0 1 to ge 0/0 2 OLT(config)#

Config Traffic Limit Based On ACL Rule

| Command | OLT(config)##traffic--limit {inbound¦outbound} <acl-id> rule-id <rule-id> port {ge¦pon¦xge} <port-id> cir <rate-value> pir <rate-value> exceed {drop¦remark-dscp} |
| --- | --- |
| View | Config view |
| Description | This command is used to set traffic-limit for specified port by match the acl. “no traffic-limit......”is used to delete the acl matched traffic-limit in specified port. |
| {inbound¦outbound} | inbound:The ingress flow Outbound:The egress flow |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-list> | Port list |
| {drop¦remark-dscp} | drop:discard remark:Re-mark the DSCP value |

【Example】

Example 1：In work time range,set the traffic-limit for the ingress direction message from GE1 10.10.10.2.In which the cir is 1M,pir is 100M,and discard the transfinite flow.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range worktime OLT(acl-basic-2000)#exit OLT(config)#traffic-limit inbound 2000 port ge 0/01 cir 1024 pir 102400 exceed drop OLT(config)#

Add Traffic Outer VLAN Based On ACL Rule

| Command | OLT(config)##traffic-outervlan inbound <acl-id> rule-id <rule-id> port {ge¦pon¦xge} <port-id> vlan <vlan-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to add outer vlan for acl matched folw in specified port. “no traffic-outervlan......”is used to cancel the command mention above. |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-id> | Port id |
| <vlan-id> | Outer vlan id |

【Example】

Example 1：In work time range,add outer vlan 10 for the message from GE1 10.10.10.2

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range worktime OLT(acl-basic-2000)#exit OLT(config)#traffic-outervlan inbound 2000 port ge 0/0/1 vlan 10 OLT(config)#

Translate Traffic VLAN Based On ACL Rule

| Command | OLT(config)##traffic-translate inbound <acl-id> rule-id <rule-id> port {ge¦pon¦xge} <port-id> vlan <vlan-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to translate the vlan id of acl matched port’s egress or ingress flow into new vlan id. |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-id> | Port id |
| <vlan-id> | New vlan id |

【Example】

Example 1：Translate the vlan id of the flow in GE5 which had been bound to acl2000 and rule2 into vlan 19.

OLT(config)#traffic-translate inbound 2000 rule-id 2 port ge 0/0/5 vlan 19 OLT(config)#

Modify Traffic VLAN Priority Based On ACL Rule

| Command | OLT(config)#traffic-priority inbound <acl-id> rule-id <rule-id> port{ge¦ pon¦xge} <port-id> remark-priority <pri-value> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the acl vlan priority for specified port. “no traffic-priority......”is used to cancel the vlan priority on specified port. |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-id> | Port id |
| <pri-value> | Priority value |

【Example】

Example 1：In work time range,set the vlan priority as 2 for the message from GE1 10.10.10.2.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range worktime OLT(acl-basic-2000)#exit OLT(config)#traffic-priority inbound 2000 port ge 0/0 1 remark-priority 2 OLT(config)#

Config Traffic Redirect Based On ACL Rule

| Command | OLT(config)##traffic-redirect inbound <acl-id> rule-id <rule-id> port {ge¦ pon¦xge} <port-id> to {ge¦xge} |
| --- | --- |
| View | Config view |
| Description | This command is used to set traffic-redirect for acl matched flow in specified port. “no traffic-redirect”is used to cancel the traffic-redirect. (Redirect the acl matched message in specified port or port list to other port and transfer.After setting,the old port will not transfer the redirect message,but it will be transferred by the new port.In addition,a correct vlan setting in the port is needed) |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-list> | Port list |

【Example】

Example 1：In work time range,redirect the message from GE1 10.10.10.2 to GE2.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range worktime OLT(acl-basic-2000)#exit OLT(config)#traffic-redirect inbound 2000 port ge 0/0 1 to ge 0/0 2 OLT(config)#

Modify Traffic TOS Value Based On ACL Rule

| Command | OLT(config)#traffic-tos {inbound¦outbound} <acl-id> rule-id <rule-id> port {ge¦ pon¦xge} <port-id> remark-tos <Tos value> |
| --- | --- |
| View | Config view |
| Description | This command is used to modify the tos value of the acl matched value. |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-id> | Port list |
| <Tos value> | Tos vlaue |

【Example】

Example 1：In work time range,modify the tos value of the message from GE1 10.10.10.2 as 3.

OLT(config)#time-range worktime 8:00 to 18:00 working-day OLT(config)#acl 2000 OLT(acl-basic-2000)#rule 2 permit source 10.10.10.2 0.0.0.0 time-range worktime OLT(acl-basic-2000)#exit OLT(config)#traffic-tos inbound 2000 rule-id 2 port ge 0/0 1 remark-tos 3 OLT(config)#

Config ACL Action and Bind ACL to PON Port

| Command | OLT(config)#traffic-modify pon-port <port-id> {downstream¦upstream} precedence <precedence-id> acl <acl-id> <action> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the pon-acl matched flow(it uses with pon acl 8000-8199) |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-id> | Port id |
| <Precedence-id> | ACL priority,the command with higher priority will execute firstly. |
| <downstream¦upstream> | downstream upstream |
| <action> | 8021p:vlan priority deny:Deny corresponding flow passing add-inner-vlan:Add inner vlan for the highest priority acl matched flow in pon port. add-top-vlan:Add outer vlan for the highest priority acl matched flow in pon port. strip-inner-vlan:Peel the inner vlan for the highest priority acl matched flow in pon port. strip-top-vlan:Peel the outer vlan for the highest priority acl matched flow in pon port. swap-inner-vlan:Replace the inner vlan of the highest priority acl matched flow in pon port. swap-top-vlan:Replace the outer vlan of the highest priority acl matched flow in pon port. |

【Example】

Example 1：Add outer vlan 1000 for the flow which vlan is 100-200 in pon1 upstream direction.

OLT(config)#acl 8000 OLT(acl-pon-8000)#vlan-id top-vid 100 to 200 OLT(acl-pon-8000)#exit OLT(config)#traffic-modify pon-port 0/0 1 upstream precedence 4 acl 8000 add-top-vlan 1000 OLT(config)#

Config ACL Action and Bind ACL to ONU

| Command | OLT(config)#traffic-modify pon-port <port-id> <onu-id> <downstream¦upstream> precedence <precedence-id> acl <acl-id> <action> |
| --- | --- |
| View | Config view |
| Description | The traffic-modify command is used to modify the data of the PON port and the ONU (this command is used together with the pon acl (8000-8199) part) |
| <acl-id> | ACL id |
| <rule-id> | Rule id in acl |
| <port-list> | Port list |
| <onu-id> | ONU id |
| <Precedence-id> | ACL priority,the command with higher priority will execute firstly. |
| <downstream¦upstream> | downstream upstream |
| <action> | 8021p:vlan priority deny:Deny corresponding flow passing add-inner-vlan:Add inner vlan for the highest priority acl matched flow in pon port. add-top-vlan:Add outer vlan for the highest priority acl matched flow in pon port. strip-inner-vlan:Peel the inner vlan for the highest priority acl matched flow in pon port. strip-top-vlan:Peel the outer vlan for the highest priority acl matched flow in pon port. swap-inner-vlan:Replace the inner vlan of the highest priority acl matched flow in pon port. swap-top-vlan:Replace the outer vlan of the highest priority acl matched flow in pon port. |

【Example】

Example 1： Apply the outer vlan 1000 to the data of the uplink data stream vlan of the ONU1 under the PON from 100 to 200.

OLT(config)# acl 8000 OLT(acl-pon-8000)# vlan-id top-vid 100 to 200 OLT(acl-pon-8000)# exit OLT(config)# traffic-modify ont 0/0 1 1 upstream precedence 4 acl 8000 add-top-vlan 1000 OLT(config)#

Show ACL Bind Configuration

| Command | OLT(config)#show traffic-modify {all¦pon-port¦ont} |
| --- | --- |
| View | Config view |
| Description | This command is used to show configuration info of pon-acl traffic-modify |
| {all¦ont-pon¦ont} | all:show all the configuration info of pon-acl pon-port:show pon-acl configuration info of pon port ont:show pon-acl configuration info of onu |

【Example】

Example 1：Show all the configuration info of pon-acl

OLT(config)#show traffic-modify all traffic-modify pon-port 0/0 1 upstream precedence 4 acl 8000 add-top-vlan 1000 traffic-modify ont 0/0 1 1 upstream precedence 4 acl 8000 swap-top-vlan 1000 OLT(config)#

Show Port Packet-filter Policy Configuration

| Command | OLT(config)#show packet-filter {all¦port} |
| --- | --- |
| View | Config view |
| Description | This command is used to show packet-filter strategy in the port |
| {all¦port} | all:Show all the packet filter configuration port:Show packet filter strategy of specified port |

【Example】

Example 1：Show all the packet filter configuration

OLT(config)#show packet-filter all ----------------------------------------------------------------------------- inbound acl 2000 rule 1 port ge 0/0 1 running inbound acl 2000 rule 2 port ge 0/0 1 not running inbound acl 2000 rule 3 port ge 0/0 1 not running ----------------------------------------------------------------------------- OLT(config)#

Show Port QOS Configuration Information

| Command | OLT(config)#show qos-info {all¦traffic-dscp¦traffic-tos¦traffic-limit¦traffic-mirror¦traffic-outervlan¦traffic-priority¦traffic-redirect¦traffic-statistic¦traffic-translate¦} port {ge¦ pon¦xge} <port-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to show qos strategy of the port |
| <all> | All of the qos strategy |
| <port-list> | Port list |

【Example】

Example 1：Show all the QOS strategy of GE1.

OLT(config)#show qos-info all port ge 0/0/1 traffic-tos on ge 0/0/1: Inbound: Matches:acl 2000 rule 1 running Remark-tos:3 OLT(config)#

Config OLT QOS Queue

Config OLT QOS Queue Mapping Mode

| Command | OLT(config)#qos cosq-map mode {cos¦diffserv¦tos} |
| --- | --- |
| View | Config view |
| Description | This command is used to set the mapping mode of system queue |
| <cos¦diffserv¦tos> | Cos:The mapping mode is based on 802.1p diffserv:The mapping mode is based on diffserv Tos:The mapping mode is based on tos |

【Example】

Example 1：Set the QOS mapping mode as 802.1p.

OLT(config)#qos cosq-map mode cos OLT(config)#

Config QOS Queue Mapping Mode Based On 802.1p

| Command | OLT(config)#qos cosq-map cos0 <Queue id> cos1 <Queue id> cos2 <Queue id> cos3 <Queue id> cos4 <Queue id> cos5 <Queue id> cos6 <Queue id> cos7 <Queue id> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the mapping table of system queue and 802.1p |
| <Queue id> | Queue id,range for 0-7 |

【Example】

Example 1：Set the mapping table of system queue and 802.1p

OLT(config)#qos cosq-map cos0 1 cos1 2 cos2 3 cos3 4 cos4 5 cos5 6 cos6 7 cos7 0 OLT(config)# OLT(config)#show qos queue-scheduler Queue scheduler mode:SP ----------------------------------------------------------------------------- Queue Scheduler Mode WRR Weight Bandwidth(kbps) ----------------------------------------------------------------------------- 0 SP-- 1 SP-- 2 SP-- 3 SP-- 4 SP-- 5 SP-- 6 SP-- 7 SP-- ----------------------------------------------------------------------------- Queue map mode:Cos ----------------------------------------------------------------------------- Priority Queue ----------------------------------------------------------------------------- 0 1 1 2 2 3 3 4 4 5 5 6 6 7 7 0 -----------------------------------------------------------------------------

Config OLT QOS Queue Scheduled Mode

| Command | OLT(config)#qos queue-scheduler strict-priority OLT(config)#qos queue-scheduler wrr <queue0-weight queue1-weight queue2-weight queue3-weight queue4-weight queue5-weight queue6-weigh tqueue7-weight> OLT(config)#qos queue-scheduler bandwidth cos0 <bandwidth> cos1 <bandwidth> cos2 <bandwidth> cos3 <bandwidth> cos4 <bandwidth> cos5 <bandwidth> cos6 <bandwidth> cos7 <bandwidth> |
| --- | --- |
| View | Config view |
| Description | This command is used to set system queue scheduled mode.The message which is sending from the same port is divided into several queue by Queue scheduling,and schedules them between queue and queue,it decides the sending sequence of queue.when the user needs to select different queue scheduling mode according to the importance of service,and ensure that the QoS guarantees are still available for important business when the network is blocked,using this command.After setting,the system will send the queue message according to new dispatching mode. System support PQ,WRR,WRR+PQ dispatching mode.when a queue is null,it can switch to next queue immediately and dispatch,which can make full use of bandwidth source. |
| strict-priority | Strict-priority scheduling,When this mode is applied,the system schedules strictly according to the priority of the queue.Only high priority queue is null,the low priority queue can be scheduled.The disadvantage of the PQ scheduling mode shows as follows: When the blocking is happening,if the high priority queue has some groups existing for a long time,corresponding apply will time out for the reason that the message with low priority can not be scheduled in time. |
| wrr | Weighted Round Robin.When this mode is applied,it needs to match a weight for each queue(weight means the obtained resource proportion),it takes turns to scheduling according to the weight in queue and assure each queue can obtain definite service.Each queue has the same priority but different weights,the bigger weight it is,the longer scheduling time obtained by this queue.In this way it can assure the lowest priority queue which can obtain definite service at least.avoiding that the message in low priority queue can not obtain service for a long time when adopting the PQ scheduling mode. queue0-weight/queue1-weight/queue2-weight/queue3-weight/queue4-weight/queue5-weight/queue6-weight/queue7-weight:the weight of each queue.system supports 8 queues,the weight’s sum of 8 queues is 100. |
| WRR+PQ | WRR+PQ scheduling mode is a mixture of WRR and PQ scheduling modes.When the scheduling mode is WRR,and the weight value of queue has 0,the queue scheduling mode is PQ+WRR.In this mode,system will schedule the queue with 0 weight first according to PQ mode,and then schedules the queue with non-zero weight according to WRR mode,the priority of PQ queue is higher than WRR queue in the meantime. System default scheduling mode is PQ mode. |

【Example】

Example 1：Set the scheduling mode of pon1 as WRR mode,let the messages with various kinds of priority can be scheduled.The weight of queue 0-7 is 15 15 20 10 10 10 10 10.

OLT(config)#qos queue-scheduler port-based epon 0/0 1 wrr 15 15 20 10 10 10 10 10 OLT(config)#

Show OLT QOS Queue Schedule Mode

| Command | OLT(config)#show qos queue-schedule |
| --- | --- |
| View | Config view |
| Description | This command is used to show system queue-schedule mode. |

【Example】

Example 1：Show system queue-schedule mode.

OLT(config)#show qos queue-scheduler Queue scheduler mode:SP ----------------------------------------------------------------------------- Queue Scheduler Mode WRR Weight Bandwidth(kbps) ----------------------------------------------------------------------------- 0 SP-- 1 SP-- 2 SP-- 3 SP-- 4 SP-- 5 SP-- 6 SP-- 7 SP-- ----------------------------------------------------------------------------- Queue map mode:Cos ----------------------------------------------------------------------------- Priority Queue ----------------------------------------------------------------------------- 0 1 1 2 2 3 3 4 4 5 5 6 6 7 7 0 OLT(config)#

OLT Port QOS Queue Configuration

Config OLT Port QOS Queue Mapping Mode

| Command | OLT(config)#qos cosq-map port-based {cos¦diffserv¦tos} {epon¦ge¦xge} <port-id> <parameter> |
| --- | --- |
| View | Config view |
| Description | This command is used to set mapping mode of port-based queue |
| {cos¦diffserv¦tos} | Cos:The mapping mode is based on 802.1p diffserv:The mapping mode is based on diffserv Tos:The mapping mode is based on tos |

【Example】

Example 1：Set the mapping mode of pon1 queue as 802.1q.

OLT(config)#qos global mode port-based OLT(config)#qos cosq-map port-based cos epon 0/0 1 OLT(config)#

Config OLT Port QOS Queue Mapping Mode Based On 802.1p

| Command | OLT(config)#qos cosq-map port-based cos {epon¦ge¦xge} <port-id> cos0 <Queue id> cos1 <Queue id> cos2 <Queue id> cos3 <Queue id> cos4 <Queue id> cos5 <Queue id> cos6 <Queue id> cos7 <Queue id> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the mapping table between port-based queue and 802.1p |
| <Queue id> | Queue id,range for 0-7 |

【Example】

Example 1：Set the mapping table between pon1 queue and 802.1q

OLT(config)#qos cosq-map port-based cos epon 0/0 1 OLT(config)#qos cosq-map port-based cos epon 0/0 1 cos0 1 cos1 2 cos2 3 cos3 4 cos4 5 cos5 6 cos6 7 cos7 0 OLT(config)#

Config OLT Port QOS Queue Schedule Mode

| Command | OLT(config)#qos queue-scheduler port-based {epon¦ge¦xge} <port-id> strict-priority OLT(config)#qos queue-scheduler port-based {epon¦ge¦xge} <port-id> wrr <queue0-weight queue1-weight queue2-weight queue3-weight queue4-weight queue5-weight queue6-weigh tqueue7-weight> OLT(config)#qos queue-scheduler port-based {epon¦ge¦xge} <port-id> bandwidth cos0 <bandwidth> cos1 <bandwidth> cos2 <bandwidth> cos3 <bandwidth> cos4 <bandwidth> cos5 <bandwidth> cos6 <bandwidth> cos7 <bandwidth> |
| --- | --- |
| View | Config view |
| Description | This command is used to set system queue scheduled mode.The message which is sending from the same port is divided into several queue by Queue scheduling,and schedules them between queue and queue,it decides the sending sequence of queue.when the user needs to select different queue scheduling mode according to the importance of service,and ensure that the QoS guarantees are still available for important business when the network is blocked,using this command.After setting,the system will send the queue message according to new dispatching mode. System support PQ,WRR,WRR+PQ dispatching mode.when a queue is null,it can switch to next queue immediately and dispatch,which can make full use of bandwidth source. |
| strict-priority | Strict-priority scheduling,When this mode is applied,the system schedules strictly according to the priority of the queue.Only high priority queue is null,the low priority queue can be scheduled.The disadvantage of the PQ scheduling mode shows as follows: When the blocking is happening,if the high priority queue has some groups existing for a long time,corresponding apply will time out for the reason that the message with low priority can not be scheduled in time. |
| wrr | Weighted Round Robin.When this mode is applied,it needs to match a weight for each queue(weight means the obtained resource proportion),it takes turns to scheduling according to the weight in queue and assure each queue can obtain definite service.Each queue has the same priority but different weights,the bigger weight it is,the longer scheduling time obtained by this queue.In this way it can assure the lowest priority queue which can obtain definite service at least.avoiding that the message in low priority queue can not obtain service for a long time when adopting the PQ scheduling mode. queue0-weight/queue1-weight/queue2-weight/queue3-weight/queue4-weight/queue5-weight/queue6-weight/queue7-weight:the weight of each queue.system supports 8 queues,the weight’s sum of 8 queues is 100. |
| WRR+PQ | WRR+PQ scheduling mode is a mixture of WRR and PQ scheduling modes.When the scheduling mode is WRR,and the weight value of queue has 0,the queue scheduling mode is PQ+WRR.In this mode,system will schedule the queue with 0 weight first according to PQ mode,and then schedules the queue with non-zero weight according to WRR mode,the priority of PQ queue is higher than WRR queue in the meantime. System default scheduling mode is PQ mode. |

【Example】

Example 1：Set the scheduling mode of pon1 as WRR mode,let the messages with various kinds of priority can be scheduled.The weight of queue 0-7 is 15 15 20 10 10 10 10 10.

OLT(config)#qos queue-scheduler wrr 15 15 20 10 10 10 10 10 OLT(config)#

Show OLT Port QOS Queue Schedule Mode

| Command | OLT(config)#show qos queue-schedule port-based {epon¦ge¦xge} <port-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to show port-based queue-schedule mode |

【Example】

Example 1：Show pon1 queue-schedule mode

OLT(config)#show qos queue-scheduler port-based epon 0/0/1 Queue scheduler mode on epon 0/0/1:WRR ----------------------------------------------------------------------------- Queue Scheduler Mode WRR Weight Bandwidth(kbps) ----------------------------------------------------------------------------- 0 WRR 15- 1 WRR 15- 2 WRR 20- 3 WRR 10- 4 WRR 10- 5 WRR 10- 6 WRR 10- 7 WRR 10- ----------------------------------------------------------------------------- Queue map mode on epon 0/0/1:Cos ----------------------------------------------------------------------------- Priority Queue ----------------------------------------------------------------------------- 0 1 1 2 2 3 3 4 4 5 5 6 6 7 7 0 ----------------------------------------------------------------------------- OLT(config)#

OLT DHCP Function Configuration

OLT DHCP-Snooping Function Configuration

Enable or Disable DHCP-Snooping Function

| Command | OLT(config)#dhcp-Snooping {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to enable or disable dhcp-snooping function,after enabling this function,the functions showing as follows will be opened at the same time: Trust/un-trust port,MAC address detect,DHCP message rate limit in untrust port,port recovery,option82,dynamic ARP detect and ARP fast reply. |
| {enable¦disable} | Enable:enable dhcp-snooping function Disable:disable dhcp-snooping function |

【Example】

Example 1：Enable dhcp-snooping function

OLT(config)#dhcp-snooping enable OLT(config)#

Show DHCP-Snooping Configuration

| Command | OLT(config)#show dhcp-snooping configuration |
| --- | --- |
| View | Config view |
| Description | This command is used to show configuration of DHCP-Snooping |

【Example】

Example 1：Show configuration of DHCP-Snooping

OLT(config)#show dhcp-snooping configuration ----------------------------------------------------------------------------- DHCP Snooping Configurations ----------------------------------------------------------------------------- Switch DHCP Snooping status:Enable DHCP Snooping verification of hwaddr status:Disable DHCP Snooping option82 status:Disable DHCP Snooping option82 policy:Keep DHCP Snooping option82 format:Type1 DHCP Snooping database wirte-delay time:7200(s) Switch ARP dectection status:Disable Switch ARP reply-fast status:Disable DHCP Snooping is configured on following vlans: ----------------------------------------------------------------------------- - Port Trusted Rate-limit(pps) ----------------------------------------------------------------------------- ge0/0/1 No 300 ge0/0/2 No 300 ge0/0/3 No 300 ge0/0/4 No 300 ge0/0/5 No 300 ge0/0/6 No 300 ge0/0/7 No 300 ge0/0/8 No 300 xge0/0/1 No 300 xge0/0/2 No 300 pon0/0/1 No 300 pon0/0/2 No 300 pon0/0/3 No 300 pon0/0/4 No 300 pon0/0/5 No 300 pon0/0/6 No 300 pon0/0/7 No 300 pon0/0/8 No 300 pon0/0/9 No 300 pon0/0/10 No 300 pon0/0/11 No 300 pon0/0/12 No 300 pon0/0/13 No 300 pon0/0/14 No 300 pon0/0/15 No 300 pon0/0/16 No 300 lag1 No 300 lag2 No 300 lag3 No 300 lag4 No 300 lag5 No 300 lag6 No 300 lag7 No 300 lag8 No 300 lagL9 No 300 lagL10 No 300 lagL11 No 300 lagL12 No 300 lagL13 No 300 lagL14 No 300 lagL15 No 300 lagL16 No 300 ----------------------------------------------------------------------------- OLT(config)#

Config DHCP-Snooping Monitor VLAN

| Command | OLT(config)#dhcp-snooping vlan <vlan-list> |
| --- | --- |
| View | Config view |
| Description | This command is used to add specified monitoring vlan.DHCP message which includes into monitoring vlan will be monitored,DHCP message which does not include into monitoring vlan will be transferred with original shape. “no dhcp-snooping vlan <vlan-list>”is used to delete the specified monitoring vlan. |
| <vlan-list> | Vlan list |

【Example】

Example 1：Add monitoring vlan 100,200,300

OLT(config)#dhcp-snooping vlan 100,200,300 OLT(config)#

Config DHCP-Snooping Trust Port

| Command | OLT(config)#dhcp-snooping trust port {epon/ge/xge/lag} 0/0 <port id> |
| --- | --- |
| View | Config view |
| Description | This command is used to add dhcp-snooping trust port,trust port can receive all the DHCP message,untrust port can not receive DHCP reply message. “no dhcp-snooping ust port {epon/ge/xge/lag} 0/0 <port id>”is used to delete the trust port. |
| {epon/ge/xge/lag} | Port type Epon:Pon port Ge:Giga uplink port Xge:10 giga uplink port Lag:link aggregation group |
| 0/0 | Frame id/slot id,the default is 0/0 |
| <pord-id> | Port list,format for 1,2-3,4 |

【Example】

Example 1：Add GE1 to dhcp-snooping trust port.

OLT(config)#dhcp-snooping trust port ge 0/0 1 OLT(config)#

Enable or Disable OLT DHCP-Snooping Source MAC Address Detect

| Command | OLT(config)#dhcp-snooping chaddr-check {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to enable or disable dhcp-snooping chaddr-check,check whether the source mac address of dhcp request message received by untrust port is the same with CHADDR field,if yes,checking it,else discarding. |
| {enable¦disable} | Enable:open dhcp-snooping chaddr-check Disable:close dhcp-snooping chaddr-check |

【Example】

Example 1：Enable dhcp-snooping chaddr-check function

OLT(config)#dhcp-snooping chaddr-check enable OLT(config)#

Config DHCP-Snooping Request Message Rate Limit

| Command | OLT(config)#dhcp-snooping limit-rate <Rate> port {epon/ge/xge/lag} 0/0 <port-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to set rate limitation of dhcp request message received by untrust port,the message will be discard when it out of range.Rate limitation can be set in trust port but it will not take effect unless set this port as untrust port. |
| <Rate> | Rate of dhcp request message,range for 1-2048,unit is pps |
| {epon/ge/xge/lag} | Port type Epon:Pon port Ge:Giga uplink port Xge:10 giga uplink port Lag:link aggregation group |
| <port id> | Port list,format for 1,2-3,4 |

【Example】

Example 1：The rate limitation of DHCP request message received by GE6 is 20pps.

OLT(config)#dhcp-snooping limit-rate 20 port ge 0/0 6 OLT(config)#

Enable or Disable DHCP-Snooping Opton82 Function

| Command | OLT(config)#dhcp-snooping option82 {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to enable¦disable dhcp-snooping option82 function.This command inserts option82 information into dhcp request message received by untrust port and peels the option82 information from dhcp reply message received by trust port. |
| {enable¦disable} | Enable:Enable dhcp-snooping option82 function Disable:Disable dhcp-snooping option82 function |

【Example】

Example 1：Enable dhcp-snooping option82 function

OLT(config)#dhcp-snooping option82 enable OLT(config)#

Config DHCP-Snooping Option82 Forwarding Policy

| Command | OLT(config)#dhcp-snooping option82 policy {keep¦drop¦replace} |
| --- | --- |
| View | Config view |
| Description | This command is used to set the option82 forwarding policy of dhcp request message. |
| keep | Transmit option82 dhcp message according to the original shape |
| drop | Discard the option82 dhcp message |
| replace | Replace old option82 in the dhcp message and then transmit |

【Example】

Example 1：Set the dhcp option82 transmission policy as keep.

OLT(config)#dhcp-snooping option82 policy keep OLT(config)#

Config DHCP-Snooping option82 Format

| Command | OLT(config)#dhcp-snooping option82 format {type1¦type2¦type3¦type4¦type5} |
| --- | --- |
| View | Config view |
| Description | This command is used to set the format of dhcp-snooping option82 |
| <type1¦type2¦type3¦type4¦type5> | type1:UNI+ONU MAC type2:UNI+OLT MAC type3:ONU+ONU MAC type4:ONU+OLT MAC type5:PON+OLT MAC |

【Example】

Example 1：set the format of dhcp-snooping option82 as type1.

OLT(config)#dhcp-snooping option82 format type1 OLT(config)#

Config DHCP-Snooping Port Binding Policy

| Command | OLT(config)#dhcp-snooping binding<mac address><ip address> <vlan id> <port id> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the binding strategy of dhcp request message |
| <mac address> | MAC in static binding table,format for AA:BB:CC:DD:EE:FF |
| <ip address> | IP in static binding table,format for A.B.C.D |
| <vlan id> | vlan in static binding table,range for<1-4094> |
| <port id> | Port type Epon:Pon port Ge:Giga uplink port Xge:10 giga uplink port Lag:link aggregation group |

【Example】

Example 1：Add a static binding table entry,mac address is 00:0f:1f:c5:10:08,ip is 192.168.1.101,vlan is 100,port id is GE8.

OLT(config)#dhcp-snooping binding 00:0f:1f:c5:10:08 192.168.1.101 100 port ge 0/0/8 OLT(config)#

Delete DHCP-Snooping Bind-table

| Command | OLT(config)#dhcp-snooping bind-table clear {all¦static¦dynamic¦ip-address¦vlan} |
| --- | --- |
| View | Config view |
| Description | This command is used to clear dhcp-snooping bind-table. |
| all | Clear all the dhcp-snooping bind-table entry. |
| static | Clear the static dhcp-snooping bind-table entry. |
| dynamic | Clear the dynamic dhcp-snooping bind-table entry. |
| Ip-address | Delete the bind-table by specified ip |
| vlan | Delete the bind-table by specified vlan id |

【Example】

Example 1：Clear all the dhcp-snooping bind-table entry.

OLT(config)#dhcp-snooping bind-table clear all OLT(config)#

Config DHCP-Snooping Bind-table Write-delay Time

| Command | OLT(config)#dhcp-snooping bind-table write-delay <Delay time> |
| --- | --- |
| View | Config view |
| Description | This command is used to set dhcp-snooping bind-table write-delay time.After dhcp-snooping binding-table having been updated and waiting for write-delay time,the flash will be written |
| <Delay time> | Write-delay time,range for 240-86400,unit is second. |

【Example】

Example 1：When the dhcp-snooping has been updated,the flash will be written after 4min.

OLT(config)#dhcp-snooping bind-table write-delay 240 OLT(config)#

Config DHCP-Snooping Bind-table Delete-time

| Command | OLT(config)#dhcp-snooping bind-table delete-time <time> |
| --- | --- |
| View | Config view |
| Description | This command is used to set dhcp-snooping bind-table dynamic entry delete-time.dynamic table will not be deleted immediately when the lease time is over,but it will be deleted after waiting for the delete-time. |
| <time> | Dynamic table delay delete-time,range for 1-86400,unit is second |

【Example】

Example 1：When the lease time is expiration dynamic table will be deleted after 240s latter.

OLT(config)#dhcp-snooping bind-table delete-time 240 OLT(config)#

Config DHCP-Snooping Bind-table Write-to-flash

| Command | OLT(config)#dhcp-snooping bind-table write-to-flash |
| --- | --- |
| View | Config view |
| Description | This command is used to write the dhcp-snooping bind-table to the flash by manually. |

【Example】

Example 1：Write the dhcp-snooping bind-table to the flash.

OLT(config)#dhcp-snooping bind-table write-to-flash OLT(config)#

Save DHCP-Snooping Bind-table to Server

| Command | OLT(config)#dhcp-snooping bind-table save-to-tftp <IP address> |
| --- | --- |
| View | Config view |
| Description | This command is used to write the dhcp-snooping bind-table to the flash by manually and save it to the server |
| <IP address> | The ip address of the TFTP server |

【Example】

Example 1：Write the dhcp-snooping bind-table to the flash by manually and save it to the server 192.168.1.1

OLT(config)#dhcp-snooping bind-table save-to-tftp 192.168.1.1 OLT(config)#

Show DHCP-Snooping Bind-table Entry

| Command | OLT(config)#show dhcp-snooping bind-table {all¦static¦dynamic¦ip-address¦vlan} |
| --- | --- |
| View | Config view |
| Description | This command is used to show dhcp-snooping bind-table according to the type. |
| all | Show all the dhcp-snooping bind-table entry. |
| static | Show the static dhcp-snooping bind-table entry. |
| dynamic | Show the dynamic dhcp-snooping bind-table entry. |
| Ip-address | Show the bind-table by specified ip |
| vlan | Show the bind-table by specified vlan id |

【Example】

Example 1：Show all the dhcp-snooping bind-table entry.

OLT(config)#show dhcp-snooping bind-table all ----------------------------------------------------------------------------- database entries count:1 database entries delete time:240(s) ----------------------------------------------------------------------------- MacAddress IpAddress Vlan Port Lease(s)Type Status ----------------------------------------------------------------------------- 00:0F:1F:C5:10:08 192.168.1.101 100 ge0/0/8-Static Valid ----------------------------------------------------------------------------- OLT(config)#

Enable or Disable DHCP-Snooping arp-reply-fast

| Command | OLT(config)#dhcp-snooping arp-reply-fast {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to enable or disable arp-reply-fast function.After enabling this function,system accordings to dhcp-snooping bind-table to judge whether it will execute arp-reply-fast function.When this function is enabled,detecting the Arp message,if there is record in dhcp-snooping bind-table,arp request message will be replied fast rather than transmit to the upper network,thus reducing arp broadcast message. |
| {enable¦disable} | Enable:enable dhcp-snooping arp-reply-fast function Disable:disable dhcp-snooping arp-reply-fast function |

【Example】

Example 1：Enable dhcp-snooping arp-reply-fast function

OLT(config)#dhcp-snooping arp-reply-fast enable OLT(config)#

Enable or Disable DHCP-Snooping arp-detect Function

| Command | OLT(config)#dhcp-snooping arp-detect {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to enable or disable arp-detect function.After enabling this function,system judge whether the user of arp message is legal according to dhcp-snooping bind-table,thus preventing from the illegal arp attack. |
| {enable¦disable} | Enable:Enable dhcp-snooping arp-detect function Disable:Disable dhcp-snooping arp-detect function |

【Example】

Example 1：Disable dhcp-snooping arp-detect function

OLT(config)#dhcp-snooping arp-detect enable OLT(config)#

OLT DHCP-Relay Function Configuration

Enable or Disable DHCP-Relay Function

| Command | OLT(config)#dhcp-relay {enable¦disbale} |
| --- | --- |
| View | Config view |
| Description | This command is used to enable or disable dhcp-relay function. |
| {enable¦disable} | Enable:Enable dhcp-relay function Disable:Disable dhcp-relay function |

【Example】

Example 1：Enable dhcp-relay function

OLT(config)#dhcp-relay enable OLT(config)#

Config DHCP-Relay Vlanif Interface and Server

| Command | OLT(config)#dhcp-relay vlanif <vlan-id> server <IP address> |
| --- | --- |
| View | Config view |
| Description | This command is used to set the ip address of DHCP-Relay server |
| <IP address> | The ip address of dhcp-relay server in vlanif |

【Example】

Example 1：Set The ip address of dhcp-relay server in vlanif 100 as 192.168.100.1

OLT(config)#dhcp-relay vlanif 100 server 192.168.100.1 OLT(config)#

Show DHCP-Relay Configuration

| Command | OLT(config)#show dhcp-relay configuration |
| --- | --- |
| View | Config view |
| Description | This command is used to show configuration of dhcp-relay |

【Example】

Example 1：Show configuration of dhcp-relay.

OLT(config)#show dhcp-relay configuration ----------------------------------------------------------------------------- Switch DHCP Relay status:Enable ----------------------------------------------------------------------------- Vlanif Vlanif-Ip Vlanif-netmask Server-Ip ----------------------------------------------------------------------------- 100--192.168.100.1 ----------------------------------------------------------------------------- OLT(config)#

OLT DHCP-Client Function Configuration

Enable or Disable DHCP-Client Function

| Command | OLT(config-interface-vlanif-100)#dhcp-client {enable¦disable} |
| --- | --- |
| View | Vlanif view |
| Description | This command is used to enable or disable dhcp client function |
| {enable¦disable} | Enable:Enable dhcp client function Disable:Disable dhcp-rela function |

【Example】

Example 1：Enable dhcp client function in vlanif 100

OLT(config-interface-vlanif-100)#dhcp-client enable OLT(config-interface-vlanif-100)#

Config DHCP-Client Manual Renew IP Address

| Command | OLT(config-interface-vlanif-100)#dhcp-client renew |
| --- | --- |
| View | Vlanif view |
| Description | This command is used to enable the switch of renew in vlanif interface.It can be used when the vlanif interface needs to obtain the ip address actively,this command will trigger the system sending request message and asks for the dhcp server renewing the lease or renewing the ip address. |

【Example】

Example1:Enable the switch of renew in vlanif 100.

OLT(config-interface-vlanif-100)#dhcp-client renew OLT(config-interface-vlanif-100)#

Config DHCP-Client Manual Release IP Address

| Command | OLT(config-interface-vlanif-100)#dhcp-client release |
| --- | --- |
| View | Vlanif view |
| Description | This command is used to release the ip address of vlanif.Executing this command will trigger the system sending release message to inform the dhcp server to releasing the ip address of vlanif. |

【Example】

Example 1：Release the ip address of vlanif 100.

OLT(config-interface-vlanif-100)#dhcp-client release OLT(config-interface-vlanif-100)#

Config DHCP-Client Option60

| Command | OLT(config-interface-vlanif-100)#dhcp-client option60<option60> |
| --- | --- |
| View | Vlanif view |
| Description | This command is used to set option60 info carrying by the dhcp-client request message.When the uplink device had set a strategy that proceeds dhcp third layer relay according to option60,we can use this command to set option60 in vlanif interface to match the uplink device. “no dhcp-client option60”is used to delete the configuration of option60 and revert to default value. |
| <option60> | The info of option60 |

【Example】

Example1:Set option60 of dhcp client in vlanif 100 as“test”.

OLT(config-interface-vlanif-100)#dhcp-client option60 test OLT(config-interface-vlanif-100)#

Show DHCP-Client Configuration

| Command | OLT(config)#show dhcp-client |
| --- | --- |
| View | Config view |
| Description | This command is used to show the configuration of dhcp-client |

【Example】

Example 1：Show the configuration of dhcp-client.

OLT(config)#show dhcp-client --------------------------------------------------------------------------- Index Name FSM IP/MASK Leased Until Time --------------------------------------------------------------------------- 1 vlanif100 INIT-/-- --------------------------------------------------------------------------- OLT(config)#

Show DHCP-Client Option60 Configuration

| Command | OLT(config)#show dhcp-client option60 |
| --- | --- |
| View | Config view |
| Description | This command is used to show dhcp-client option60 info of each vlanif interface. |

【Example】

Example 1：Show dhcp-client option60 info of each vlanif interface.

OLT(config)#show dhcp-client option60 --------------------------------------------------------------------------- VLANIF OPTION60 --------------------------------------------------------------------------- 100 test --------------------------------------------------------------------------- OLT(config)#

OLT Link Aggregation Function Configuration

OLT Link-Aggregation Group Basic Function Config

Add or Delete Link-Aggregation Group Member

| Command | OLT(config-interface-aggregation)#member {add/delete} {ge¦xge} 0/0 <port-list> link-aggregation group <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | Device supports 16 aggregation group,this command is used to add or delete member port in corresponding aggregation group. |
| <port-list> | Port id |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Add ge1 and ge2 to link-aggregation group 1.

OLT(config-interface-aggregation)#member add ge 0/0 1,2 link-aggregation group 1 OLT(config-interface-aggregation)#

Example 2:Delete ge1 and ge2 from link-aggregation group 1.

OLT(config-interface-aggregation)#member delete ge 0/0 1,2 link-aggregation group 1 OLT(config-interface-aggregation)#

Enable or Disable Link-Aggregation Group Flow-control Function

| Command | OLT(config-interface-aggregation)#flow-control <group-id> {enable¦disable} |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to enable or disable aggregation group flow control function |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Enable the flow control function of aggregation group1

OLT(config-interface-aggregation)#flow-control 1 enable OLT(config-interface-aggregation)#

Config Link-Aggregation Group System Priority

| Command | OLT(config-interface-aggregation)#lacp set system priority <priority value> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set system priority |
| <priority value> | Priority value,range for<0-65535>/default |

【Example】

Example 1：Set system priority as 3000.

OLT(config-interface-aggregation)#lacp set system priority 3000 OLT(config-interface-aggregation)#

Set Link-Aggregation Group Port Priority

| Command | OLT(config-interface-aggregation)#link-aggregation port-priority <ge¦xge> 0/0 <port-list> <priority value> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set port priority |
| <port-list> | Port list to be set |
| <priority value> | Priority value,range for<0-65535> |

【Example】

Example 1：Set link-aggregation port priority of ge1 as 3000

OLT(config-interface-aggregation)#link-aggregation port-priority ge 0/0 1 3000 OLT(config-interface-aggregation)#

Show Link-Aggregation Group System Priority

| Command | OLT(config-interface-aggregation)#show lacp system priority |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to show lacp system priority |

【Example】

Example 1：Show lacp system priority

OLT(config-interface-aggregation)#show lacp system priority lacp system priority value:3000 OLT(config-interface-aggregation)#

Show Link-Aggregation Group Priority

| Command | OLT(config-interface-aggregation)#show link-aggregation port-priority <ge¦xge> 0/0 <port-list> <lacp/manual> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to show port priority of link-aggregation. |
| <port-list> | Port list to be set |

【Example】

Example 1：Show ge1 port priority of link aggregation

OLT(config-interface-aggregation)#show link-aggregation port-priority ge 0/0/1 lacp lacp port priority:32768 OLT(config-interface-aggregation)#

Config Link-Aggregation Group MTU

| Command | OLT(config-interface-aggregation)#mtu <group-id> <mtu value> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set mtu of link-aggregation |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <mtu value> | mtu value,range for 328-16356 |

【Example】

Example 1：Set the mtu of link-aggregation group1 as 2000

OLT(config-interface-aggregation)#mtu 1 2000 OLT(config-interface-aggregation)#

Config Link-Aggregation Group Unicast Load Balance Mode

| Command | OLT(config-interface-aggregation)#link-aggregation group <group-id> unicast balance {dest-ip¦dest-mac¦source-dest-ip¦source-dest-mac¦source-ip¦source-mac} |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set unicast data balance mode of link aggregation |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <balance> | dest-ip:Load sharing unicast according to destination ip address dest-mac:Load sharing unicast according to destination mac address source-dest-ip:Load sharing unicast according to source and destination ip address source-dest-mac:Load sharing unicast according to source and destination mac address source-ip:Load sharing unicast according to source ip address source-mac:Load sharing unicast according to source mac address(default) |

【Example】

Example 1：Set the unicast load balance mode of link-aggregation mode as dest-ip.

OLT(config-interface-aggregation)#link-aggregation group 1 unicast balance dest-ip OLT(config-interface-aggregation)#

Config Link-Aggregation Group Non-unicast Load BalanceMode

| Command | OLT(config-interface-aggregation)#link-aggregation group non-unicast balance (¦dest-mac¦source-dest-mac¦source-mac¦source-port) |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set unicast data balance mode of link aggregation |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <balance> | dest-mac:Load sharing non-unicast according to destination mac address source-dest-mac:Load sharing non-unicast according to source and destination mac address source-mac:Load sharing non-unicast according to source mac address(default) Source-port:Load sharing non-unicast according to source port(default) |

【Example】

Example 1：Set the unicast load balance mode of link-aggregation mode as source-port.

OLT(config-interface-aggregation)#link-aggregation group non-unicast balance source-port OLT(config-interface-aggregation)#

Config Link-Aggregation Group Name

| Command | OLT(config-interface-aggregation)#port-name <group-id> <port name> |
| --- | --- |
| View | link-aggregation view |
| Description | Set the name of link-aggregation group |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <port name> | Name of aggregation group,length for<1-17> |

【Example】

Example 1：Name link-aggregation group1 as test

OLT(config-interface-aggregation)#port-name 1 test OLT(config-interface-aggregation)#

Clear Link-Aggregation Group Statistics

| Command | OLT(config-interface-aggregation)#reset statistics port <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to clear the link-aggregation statistic data |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Clear the statistic data of link-aggregation group 1.

OLT(config-interface-aggregation)#reset statistics port 1 OLT(config-interface-aggregation)#

Enable or Disable Link-Aggregation Group

| Command | OLT(config-interface-aggregation)#{no} shutdown <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | Open or shutdown link-aggregation group,open by default. |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Shutdown link-aggregation group1.

OLT(config-interface-aggregation)#shutdown 1 OLT(config-interface-aggregation)#

Example 2:Open link-aggregation group1.

OLT(config-interface-aggregation)#no shutdown 1 OLT(config-interface-aggregation)#

Link-Aggregation Group STP Configuration

Config Link-Aggregation Group STP Cost

| Command | OLT(config-interface-aggregation))#spanning-tree cost <group-id> <cost> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set the spanning tree cost of link-aggregation group.The optimal path is determined by port cost when there are multi links between two device but nor root port in them. |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <cost> | Cost value,range for 1-200000000 |

【Example】

Example 1：Set the spanning tree cost of link-aggregation group1 as 2000.

OLT(config-interface-aggregation))#spanning-tree cost 1 2000 OLT(config-interface-aggregation))#

Enable or Disable Link-Aggregation Group STP Edged-port

| Command | OLT(config-interface-aggregation))#spanning-tree edged-port <group-id> {enable¦disable} |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set the edged-port of the link-aggregation group.If user specifies a port as edged-port,then when the port migrates forwarding status from congestion status,this port can migrate rapidly doing without waiting for delay time.the user can only set the port which is connected with the terminal as the edged-port.All ports are default to not edged-port. |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| {enable¦disable} | enable:Set this port as edged-port disable:Set this port as not edged-port |

【Example】

Example 1：Set the edged-port of link aggregation group1.

OLT(config-interface-aggregation))#spanning-tree edged-port 1 enable OLT(config-interface-aggregation))#

Config Link-Aggregation Group STP Mcheck Property

| Command | OLT(config-interface-aggregation))#spanning-tree mcheck <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set the RSTP mcheck property of link-aggregation group.Port mcheck property is used to detected whether the port which is running under STP compatible mode can migrate to RSTP mode.By setting mcheck,you can check whether there is a bridge running STP protocol within the network segment which is connected with current Ethernet port,If yes,RSTP protocol will migrate the protocol running mode of this port to STP mode. |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Set mcheck property of link-aggregation group1.

OLT(config-interface-aggregation))#spanning-tree mcheck 1 OLT(config-interface-aggregation))#

Config Link-Aggregation Group STP Point-to-Point Function

| Command | OLT(config-interface-aggregation))#spanning-tree point-to-point <group-id> {auto/true/false} |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set point-to-point link of link-aggregation group spanning tree.If bridge works in RSTP mode,two ports which is connected by p2p link can migrate to forwarding status by sending synchronization message,it reduces the needless transfer delay time,if set this parameter as auto-mode,RSTP protocol can detect whether current Ethernet port has connected with point-to-point link automatically.The user can set by manually whether current Ethernet port connects with the p2p link.The recommendation is auto-mode. |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| auto/true/false | auto:Set the point-to-point link as auto-mode true:Connect link-aggregation group to point-to-point link false:Disconnect link-aggregation group to point-to-point link |

【Example】

Example 1：Set spanning tree point-to-point of link-aggregation as true.

OLT(config-interface-aggregation))#spanning-tree point-to-point 1 true OLT(config-interface-aggregation))#

Config Link-Aggregation Group STP Priority

| Command | OLT(config-interface-aggregation))#spanning-tree priority <group-id> <port-priority> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set the RSTP priority of link-aggregation group.By setting the priority of the link-aggregation group,You can specify that a particular link-aggregation group is contained within the spanning tree.Generally,the smaller of the setting value is,the higher of the link-aggregation group priority,this link-aggregation group is likely to include in spanning tree.If all the link-aggregation group of the bridge adapt to the same index number,the priority of the link-aggregation group depends on the index number of the link-aggregation group. |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <port-priority> | Priority value,range for 0-240,step length for 16.The default value is 128. |

【Example】

Example 1：Set spanning tree priority of link-aggregation group 1 as 160.

OLT(config-interface-aggregation))#spanning-tree priority 1 160 OLT(config-interface-aggregation))#

Link-Aggregation Group VLAN Configuration

Config Link-Aggregation Group VLAN Mode

| Command | OLT(config-interface-aggregation)#vlan mode <group-id> {access/hybrid/trunk} |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set vlan mode of link-aggregation group.The default is access mode.In each vlan mode,the message processing way of the port is showed in Appendix1 |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| access/hybrid/trunk | Access:This kind of port only belongs to one vlan,generally it is used to connect to computer. Trunk:This kind of ports can allow multi vlan pass,can receive and transfer the message of different vlan.Usually,it is used to connect to the port between switches. Hybrid:This kind of port allows multiple vlan pass,can receive and transfer the message of different vlan.It can be used to connect the port between switch or connect to the PC. |

【Example】

Example 1：Set the vlan mode of link-aggregation group 1 as access.

OLT(config-interface-aggregation)#vlan mode 1 access OLT(config-interface-aggregation)#

Config Link-Aggregation Group Native-vlan(access)

| Command | OLT(config-interface-aggregation)#vlan native-vlan <group-id> <vlan-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set Native VLAN of link-aggregation group,the default is 1.In each vlan mode,the message processing way of the port is showed in Appendix1 |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <vlan-id> | VLAN ID,range for 1-4094. |

【Example】

Example 1：Set native vlan of link-aggregation group1 as 10.

OLT(config-interface-aggregation)#vlan native-vlan 1 10 OLT(config-interface-aggregation)#

Config Link-Aggregation Group Native-vlan Priority

| Command | OLT(config-interface-aggregation)#vlan native-vlan-priority <group-id> <priority> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set native vlan priority of link-aggregation group,the default is 0. |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <priority> | Range of priority for 0-7 |

【Example】

Example 1：Set native vlan priority of link-aggregation group1 as 1.

OLT(config-interface-aggregation)#vlan native-vlan-priority 1 1 OLT(config-interface-aggregation)#

Config Link-Aggregation Group Access Mode VLAN

| Command | OLT(config-interface-aggregation)#vlan access <group-id> <vlan-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set Access VLAN of link aggregation,the default is 1..In each vlan mode,the message processing way of the port is showed inAppendix1 |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <vlan-id> | Access VLAN ID,range for 1-4094 |

【Example】

Example 1：Set access vlan of link-aggregation group1 as 100.

OLT(config-interface-aggregation)#vlan access 1 100 OLT(config-interface-aggregation)#

Config Link-Aggregation Group Hybrid Mode VLAN

| Command | OLT(config-interface-aggregation)#vlan hybrid <group-id> {tagged¦untagged} <vlan-list> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to set hybrid VLAN of link aggregation,the default is 1..In each vlan mode,the message processing way of the port is showed inAppendix1 |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| {tagged¦untagged} | tagged:Add corresponding vlan tag for egress message untagged:Peel off corresponding vlan tag for egress message |
| <vlan-list> | VLAN ID,range for 1-4094.format is 1,11-27,100 |

【Example】

Example 1：Add untagged hybrid vlan 10-15 for link-aggregation group1.

OLT(config-interface-aggregation)#vlan hybrid 1 untagged 10-15 lag1:hybrid vlan added,failed:0,success:6 OLT(config-interface-aggregation)#

Example 2:Add tagged hybrid vlan 101 for link-aggregation group1.

OLT(config-interface-aggregation)#vlan hybrid 1 tagged 101 lag1:hybrid vlan added,failed:0,success:1 OLT(config-interface-aggregation)#

Delete Link-Aggregation Group Hybrid Mode VLAN

| Command | OLT(config-interface-aggregation)#no vlan hybrid <group-id> {tagged¦untagged} <vlan-list> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to delete hybrid VLAN of link aggregation,the default is 1..In each vlan mode,the message processing way of the port is showed inAppendix1 |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| {tagged¦untagged} | tagged:Add corresponding vlan tag for egress message untagged:Peel off corresponding vlan tag for egress message |
| <vlan-list> | VLAN ID,range for 1-4094.format is 1,11-27,100 |

【Example】

Example 1：Delete tagged hybrid vlan 10-15 for link-aggregation group1.

OLT(config-interface-ge)#no vlan hybrid 1 tagged 10-15 OLT(config-interface-ge)#

Config Link-Aggregation Group Trunk Mode VLAN

| Command | OLT(config-interface-aggregation)#vlan trunk <group-id> <vlan-list> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to delete trunk VLAN of link aggregation,the default is 1..In each vlan mode,the message processing way of the port is showed inAppendix1 |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <vlan-list> | VLAN ID,range for 1-4094.format is 1,11-27,100 |

【Example】

Example 1：Add trunk vlan 10-15 for link-aggregation group1.

OLT(config-interface-aggregation)#vlan trunk 1 10-15 lag1:trunk vlan allowed,failed:0,success:6 OLT(config-interface-aggregation)#

Delete Link-Aggregation Group Trunk Mode VLAN

| Command | OLT(config-interface-aggregation)#no vlan trunk <group-id> <vlan-list> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to delete trunk VLAN of link aggregation,the default is 1..In each vlan mode,the message processing way of the port is showed inAppendix1 |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |
| <vlan-list> | VLAN ID,range for 1-4094.format is 1,11-27,100 |

【Example】

Example 1：Delete trunk vlan 10 for link-aggregation group1.

OLT(config-interface-aggregation)#no vlan trunk 1 10 OLT(config-interface-aggregation)#

Show Link-Aggregation Group Information

Show Link-Aggregation Group VLAN Information

| Command | OLT(config-interface-aggregation)#show port vlan <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to show vlan info of link-aggregation group |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Show vlan info of link-aggregation group1

OLT(config-interface-aggregation)#show port vlan 1 ----------------------------------------------------------------------------- Port:lag1 Mode:Access Native-Vlan:1 Priority:0 ----------------------------------------------------------------------------- Tagged-Vlan: - ----------------------------------------------------------------------------- Untagged-Vlan: 1 ----------------------------------------------------------------------------- OLT(config-interface-aggregation)#

Show Link-Aggregation Group STP Information

| Command | OLT(config-interface-aggregation)#show port spanning-tree <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to show spanning tree info of link-aggregation group |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example1:Show spanning tree info of link-aggregation group 1.

OLT(config-interface-aggregation)#show port spanning-tree 1 -----------------------lag1 RSTP STATUS:-------------------------------- Port STP Mode:RSTP Port Priority:128 Port Path Cost:20000 Port Edge Admin:NON-Edge Port Edge Status:NEdge Port Link Type Admin:Auto Port Link Type Status:P2P Port Role:Unknown Port State:Down ----------------------------------------------------------------------------- OLT(config-interface-aggregation)#

Show Link-Aggregation Group Port Status

| Command | OLT(config-interface-aggregation)#show port state <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to show port state of link-aggregation group |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Show port state of link-aggregation group1.

OLT(config-interface-aggregation)#show port state 1 ----------------------------------------------------------------------------- Port name:test Current port state:enable Current link state:DOWN The maximum transmit unit:1500 Link speed:autonegotiation(1000 Mbps) Link duplex:autonegotiation(FULL) Flow-control:supported ----------------------------------------------------------------------------- Native-vlan:1 Link-type:Access Priority:0 Untagged VLAN ID: 1 ----------------------------------------------------------------------------- OLT(config-interface-aggregation)#

Show Link-Aggregation Group STP Information Statistics Information

| Command | OLT(config-interface-aggregation)#show statistics port <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to show statistic info of link-aggregation group |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Show statistic info of link-aggregation group1.

OLT(config-interface-aggregation)#show statistics port 1 member ge0/0/1 statistics: ----------------------------------------------------------------------------- Direction Total Uncast Bcast Mcast Err (bytes)(pkts)(pkts)(pkts)(pkts) ---------------------------------------------------------------------------- RX 0 0 0 0 0 TX 0 0 0 0 0 ----------------------------------------------------------------------------- link-aggregation group 1 statistics: ----------------------------------------------------------------------------- Direction Total Uncast Bcast Mcast Err (bytes)(pkts)(pkts)(pkts)(pkts) ---------------------------------------------------------------------------- RX 0 0 0 0 0 TX 0 0 0 0 0 ----------------------------------------------------------------------------- OLT(config-interface-aggregation)#

Show Link-Aggregation Group Config Information

| Command | OLT(config-interface-aggregation)#show link-aggregation group <group-id> |
| --- | --- |
| View | link-aggregation view |
| Description | This command is used to show link-aggregation group matching state |
| <group-id> | Group id,1-8 is static aggregation group,9-16 is dynamic aggregation group |

【Example】

Example 1：Show matching state of link aggregation group1.

OLT(config-interface-aggregation)#show link-aggregation group 1 Lag Lag Select Unselect Load Master ID Type Ports Ports Balance Port 1 Manual-ge0/0/1,dest-ip- OLT(config-interface-aggregation)#

OLT Routing Function

Add or Delete Static Route

| Command | OLT(config)#{no} ip route-static <Ip-addr> <IP address mask> <Gateway address> |
| --- | --- |
| View | Config view |
| Description | This command is used to add or delete static route of olt,it can set a strip of static route only. |
| <no> | Delete the command |
| <ip-addr> | IP address.IP address is divided into 5 types,user can select a suitable ip subnet according to the use case,the ip address with 32bits 0 or 32bits 1 has special function,so it can not use as general ip address,format for X.X.X.X |
| <IP address mask> | Subnet mask,format for X.X.X.X |
| <Gateway address> | Gateway address,format for X.X.X.X |

【Example】

Example 1：Set a strip of static route of olt,gateway is 192.168.2.253.

OLT(config)#ip route-static 0.0.0.0 0.0.0.0 192.168.5.253 Successfully add static routing entries! OLT(config)#

Example 2:Delete a static route.

OLT(config)#no ip route-static 192.168.3.0 255.255.255.0 192.168.5.1 Successfully Del static routing entries! OLT(config)#

Show IP Route Information

| Command | OLT(config)#show ip route |
| --- | --- |
| View | Config view |
| Description | This command is used to show ip route info |

【Example】

Example 1：Show ip route info

OLT(config)#show ip route Flags:U-use,M-Mutipath,E-ECMP Destination/Mask Proto Pre Cost NextHop Interface flag 192.168.1.0/24 Direct 1 0*vlanIf2 0.0.0.0/0 Static 2 2 192.168.5.253 vlanIf0 U 192.168.3.0/24 Static 2 2 192.168.5.1 vlanIf0 U OLT(config)#

Show IP Routing-table Information

| Command | OLT(config)#show ip routing-table all |
| --- | --- |
| View | Config view |
| Description | This command is used to show ip routing-table info |

【Example】

Example 1：Show ip routing-table info

OLT(config)#show ip routing-table all Flags:U-use,M-Mutipath,E-ECMP Destination/Mask Proto Pre Cost NextHop Interface flag 192.168.1.0/24 Direct 1 0*vlanIf2 U 0.0.0.0/0 Static 2 2 192.168.5.253 vlanIf0 U 192.168.3.0/24 Static 2 2 192.168.5.1 vlanIf0 U OLT(config)#

OLT Profile Configuration

OLT DBA Profile Configurations

Create DBA Profile

| Command | OLT(config)#dba-profile {profile-id <profile-id>¦profile-name <profile-name>} |
| --- | --- |
| View | Config view |
| Description | This command is used to add a dba-profile and enter dba-profile editing view.The deployment and control of uplink Bandwidth of ONU is realized through the Dynamic Bandwidth Allocation technology.dba-profile defines the uplink bandwidth of onu.Device adjusts the allocation of uplink bandwidth automatically according to the burst need of uplink services,which increases the uplink bandwidth service efficiency of pon system.When the default dba-profile can not satisfy the service needing and needs to create a new dba profile according to the real time service needing,using this command.“no”command is used to delete dba profile. |
| <profile-id> | dba profile id.System allocates a mini idle profile number if it is not specified by system.Proile 0 is the default profile.Onu with autoauth will match profile 0 automatically. |
| <profile-name> | DBA profile name.If it is not specified,system will adopt the default name“dba-profile_id”. |

【Example】

Example 1：Create a new dba profile 10.

OLT(config)#dba-profile profile-id 10 OLT(config-dba-profile-10)#

Example 2：Delete dba-profile id 10.

OLT(config)# no dba-profile profile-id 10 OLT(config)#

Config DBA Profile Bandwith

| Command | OLT(config-dba-profile-10)#type1 fix<fixed bandwidth> OLT(config-dba-profile-10)#type2 assure<assure bandwidth> OLT(config-dba-profile-10)#type3 assure<assure bandwidth> max <max bandwidth> OLT(config-dba-profile-10)#type4 max <max bandwidth> OLT(config-dba-profile-10)#type5 fix <fixed bandwidth> assure <assure bandwidth> max <max bandwidth> |
| --- | --- |
| View | DBA profile view |
| Description | This command is used to configure the type of DBA control bandwidth and the size of the bandwidth. |
| <type1> | Fix bandwidth.It is reserved to specified onu or the specific service of onu,this bandwidth can not be used by other onu even if the onu has not uplink service stream.It is mainly used in the service with hypersensitive qos,such as TDM,VoIP and etc. |
| <type2> | Assure bandwidth.It assures that onu can obtain a specified bandwidth when it is in need.Device’s dba mechanism could allocate the surplus bandwidth to other onu’s service when the real time service stream of onu can not reach the assure bandwidth.its realtime performance is worse than fixed bandwidth for the reason that it needs to control the allocation of bandwidth according to dba mechanism. |
| <type3> | Type3 includes assure bandwidth and max bandwidth.Type3 is a combination bandwidth type which assures the user has a certain bandwidth and it commits that the user has a certain bandwidth to preempt.But the sum of assure bandwidth can not exceed the max bandwidth.This kind of bandwidth is mainly used in voip and iptv service. |
| <type4> | Max bandwidth.The upper limit of bandwidth that the onu can obtain.It is mainly used in internet service. |
| <type5> | Type5 includes fixed bandwidth and assure bandwidth and max bandwidth..Type5 is a combination bandwidth type.The sum of fix bandwidth and assure bandwidth can not exceed the max bandwidth. |
| <fix> | Fix bandwidth.This part of bandwidth is allocated to the user firmly,other user can not preempt it even if it is in idle state. |
| <assure> | Assure bandwidth.If it is in idle other user can preempt it. |
| <max> | Max bandwidth.the max available bandwidth for the user. The sum of assure bandwidth in type3 can not exceed the max bandwidth. The sum of fix bandwidth and assure bandwidth in type5 can not exceed the max bandwidth. |

【Example】

Example 1：Set the type of profile 10 as type5,fix bandwidth is 5Mbps,assure bandwidth is 10Mbps,max bandwidth is 30Mbps.

OLT(config-dba-profile-10)#type5 fix 5120 assure 10240 max 30720 OLT(config-dba-profile-10)#

Commit DBA Profile Config

| Command | OLT(config-dba-profile-10)#commit |
| --- | --- |
| View | DBA pprofile view |
| Description | This command is used to commit the current dba profile setting.All the parameter will take effect only after the command is committed. |

【Example】

Example 1：Commit the current dba profile setting.

OLT(config-dba-profile-10)#commit OLT(config-dba-profile-10)#

Show OLT DBA Profile Information

| Command | OLT(config)#show dba-profile {all¦profile-id <profile-id>¦profile-name <profile-name>} |
| --- | --- |
| View | Config view |
| Description | This command is used to show dba profile info.dba profile describes the flow parameter of the link,onu allocates the bandwidth automatically by binding dba profile which can increase the bandwidth utilization. |
| <profile-id> | Profile id to be show. |
| <profile-name> | Profile name to be show. |

【Example】

Example 1：Show all the dba profile.

OLT(config)#show dba-profile all ----------------------------------------------------------------------------- Profile Profile Type Fix Assure Max Bind ID Name(kbps)(kbps)(kbps)times ---------------------------------------------------------------------------- 0 dba-profile_0 4 0 0 1000000 1 1 dba-profile_1 4 0 0 1000000 0 2 dba-profile_2 4 0 0 1000000 0 10 dba-profile_10 5 5120 10240 30720 0 ----------------------------------------------------------------------------- Total:4 OLT(config)#

ont-lineprofile Configuraton

Create ont-lineprofile

| Command | OLT(config)#ont-lineprofile epon {profile-id <profile-id> profile-name <profile-name>} |
| --- | --- |
| View | Config view |
| Description | This command is used to create a new ont-lineprofile and enter the correspo hip between llid and dba profile. |
| <profile-id> | Profile id of ont-lineprofile.When it is unspecified,the system will match a minimum idle number.ont-lineprofile 0 is default profile.Autoauth’s onu will match ont-lineprofile 0 automatically. |
| <profile-name> | Ont-lineprofile name.the default name is“ont-lineprofile_name” |

【Example】

Example 1：Create and enter ont-lineprofile 10.

OLT(config)#ont-lineprofile epon profile-id 10 OLT(config-epon-lineprofile-10)#

Bind DBA Profile in ont-lineprofile

| Command | OLT(config-epon-lineprofile-10)#llid <llid-id> dba-profile-id <profile-id> |
| --- | --- |
| View | ont-lineprofile view |
| Description | This command is used to bind dba profile. |
| <profile-id> | dba profile id.When it is unspecified,the system will match a minimum idle number.dba profile 0 is default profile.Autoauth’s onu will match dba profile 0 automatically. |

【Example】

Example 1：Bind dba profile 10 to ont-lineprofile 10.

OLT(config-epon-lineprofile-10)#llid 1 dba-profile-id 10 OLT(config-epon-lineprofile-10)#

Config FEC Function in ont-lineprofile

| Command | OLT(config-epon-lineprofile-10)#fec {enable¦disable} |
| --- | --- |
| View | ont-lineprofile view |
| Description | This command is used to enable or disable FEC. |

【Example】

Example 1：Enable FEC in ont-lineprofile 10.

OLT(config-epon-lineprofile-10)#fec enable OLT(config-epon-lineprofile-10)#

Config Data Encryption in ont-lineprofile

| Command | OLT(config-epon-lineprofile-10)#llid <llid-id> encrypt {enable¦disable} |
| --- | --- |
| View | ont-lineprofile view |
| Description | This command is used to enable or disable data encryption. |

【Example】

Example 1：Enable data encryption in ont-lineprofile 10.

OLT(config-epon-lineprofile-10)#llid 1 encrypt enable OLT(config-epon-lineprofile-10)#

Config ONU-Car in ont-lineprofile

| Command | OLT(config-epon-lineprofile-10)#llid <llid-id> ont-car <profile-id> |
| --- | --- |
| View | ont-lineprofile view |
| Description | This command is used to bind ont-car and the it is used in conjunction with traffic-profile command in config view. |
| <profile-id> | ont-car id,ont-car is created by traffic-profile command in config view. |

【Example】

Example 1：Bind ont-car 10 in ont-lineprfile 10.

OLT(config-epon-lineprofile-10)#llid 1 ont-car 10 OLT(config-epon-lineprofile-10)#

Commit ont-lineprofile Configuration

| Command | OLT(config-epon-lineprofile-10)#commit |
| --- | --- |
| View | ont-lineprofile view |
| Description | This command is used to commit the current ont-lineprofile setting.All the parameter will take effect only after the command is committed. |

【Example】

Example 1：Commit current ont-lineprofile setting.

OLT(config-epon-lineprofile-10)#commit OLT(config-epon-lineprofile-10)#

Show OLT ont-lineprofile Information

| Command | OLT(config)#show ont-lineprofile {all¦profile-id <profile-id> ¦profile-name <profile-name>} |
| --- | --- |
| View | Config view |
| Description | This command is used to show ont-lineprofile info. |
| <profile-id> | Ont-lineprofile id to be show. |
| <profile-name> | Ont-lineprofile name to be show. |

【Example】

Example 1：Show ont-lineprofile 10 info.

OLT(config)#show ont-lineprofile epon profile-id 10 ----------------------------------------------------------------------------- Profile-ID:10 Profile-name:lineprofile_10 Binding times:0 ---------------------------------------------------------------------------- FEC switch:Disable Encrypt type:Off DBA Profile-ID:0 Traffic profile ID:- DBA-threshold: ---------------------------------------------------------------------------- Queue-set-index Q1 Q2 Q3 Q4 Q5 Q6 Q7 Q8 --------------------------------------------------------------------------- 1-------- 2-------- 3-------- 4-------- ----------------------------------------------------------------------------- OLT(config)#

ont-srvprofile Configuration

Create ont-srvprofile

| Command | OLT(config)#ont-srvprofile epon {profile-id <profile-id> profile-name <profile-name>} |
| --- | --- |
| View | Config view |
| Description | This command is used to create ont-srvprofile or enter the created ont-srvprofile editing view.The association attribute of ont service can be set in ont-srvprofile.It needs to bind epon ont-srvprofile for ont when it is added to olt,if the ont-srvprofile is not specified,the system will bind the default ont-srvprofile to ont automatically. |
| <profile-id> | ont-srvprofile id.When it is not specified,system will allocate a minimum idle profile number automatically.the autoauth onu will be matched the ont-srvprofile 0. |
| <profile-name> | Ont-srvprofile name.The default name is ont-srvprofile_id. |

【Example】

Example 1：Create ont-srvprofile 10 and enter its editing view.

OLT(config)#ont-srvprofile epon profile-id 10 OLT(config-epon-srvprofile-10)#

Config ONU Capability in ont-srvprofile

| Command | OLT(config-epon-srvprofile-10)#ont-port eth <number> catv <number> pots <number> |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set the ont-srvprofile capability set.it can set the number of each kind of port.The config-capability of ont port must be the same with ont real capability. |
| <number> | The number of ont’s port. Eth:ethernet port Pots:voice port Catv:coaxial cable port |

【Example】

Example 1：Set ont capability of ont-profile 10,ethernet port is adaptive and 1 pots port.

OLT(config-epon-srvprofile-10)#ont-port eth adaptive pots 1 OLT(config-epon-srvprofile-10)#

Config ONU Port Native-vlan in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#port native-vlan eth <port-list> <vlan id> priority <value> |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set ONU eth port native-vlan |
| <port-list> | Port list to be set |
| <vlan id> | Native-vlan id |
| <value> | vlan priority,range for 0-7,the default is 0. |

【Example】

Example 1：Set the native-vlan of eth1 in ont-srvprofile 10 as 100 and its priority is 0.

OLT(config-epon-srvprofile-10)#port native-vlan eth 1 100 priority 0 OLT(config-epon-srvprofile-10)#

Config ONU Port VLAN in ont-srvprofile

| Command | OLT(config-epon-srvprofile-10)#port vlan eth <port-list> <vlan id> <priority value> |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to add ont port to the specified vlan in ont-srvprofile. |
| <port-list> | Port list to be set. |
| <vlan id> | Vlan id |
| <value> | vlan priority,range for 0-7,the default is 0. |

【Example】

Example 1：Add eth1 to vlan 100 in ont-srvprofile 10.

OLT(config-epon-srvprofile-10)#port vlan eth 1 100 1 OLT(config-epon-srvprofile-10)#

Config ONU VLAN Mode in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#port vlan eth <port-list> <vlan mode> |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set vlan mode of the ONU port. |
| <vlan mode> | The mode of vlan are as follows: Transparent,translation,vlan-pool |

【Example】

Example 1：Set vlan mode of eth 1 as vlan-pool 1 in ont-srvprofile 10,the vlan of eth 1 will be allocated by vlan-pool 1.

OLT(config-epon-srvprofile-10)#port vlan eth 1 vlan-pool 1 OLT(config-epon-srvprofile-10)#

Config ONU Port Multicast VLAN in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#port multicast-vlan eth <port-list> <vlan id> |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set multicast vlan of ont’s port |
| <vlan id> | Multicast valn id |

【Example】

Example 1：Set multicast-vlan of eth1 as 100.

OLT(config-epon-srvprofile-10)#port multicast-vlan eth 1 100 OLT(config-epon-srvprofile-10)#

Config ONU Port Downstream Rate limit in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#port eth 1 ds-policing {unconcern¦<profile id>} |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set downstream rate limitation of ont port |
| unconcern | There is no rate limitation with downstream |
| <Profile ID> | traffic-profile id,it must be created in config view before it is called. |

【Example】

Example 1：Set eth1 downstream rate limitation,match eth1 to traffic-profile 20.

OLT(config-epon-srvprofile-10)#port eth 1 ds-policing 20 OLT(config-epon-srvprofile-10)#

Config ONU Port Upstream Rate limit in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#port eth 1 up-policing {unconcern¦<profile id>} |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set upstream rate limitation of ont port |
| unconcern | There is no rate limitation with downstream |
| <Profile ID> | traffic-profile id,it must be created in config view before it is called. |

【Example】

Example 1：Set eth1 upstream rate limitation,match eth1 to traffic-profile 20.

OLT(config-epon-srvprofile-10)#port eth 1 up-policing 20 OLT(config-epon-srvprofile-10)#

Config ONU Port MAC Count Limit in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#port eth <port-list> max-mac-count {unlimited¦<max mac count>} |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set max-mac-count of ont port. |
| unlimited | There is no rate limitation with mac address. |
| <max mac count> | The max mac address number,range for 1-64. |

【Example】

Example 1：Set max-mac-count of eth1 as 10 entries.

OLT(config-epon-srvprofile-10)#port eth 1 max-mac-count 10 OLT(config-epon-srvprofile-10)#

Config ONU Port IGMP Number in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#port eth <port-list> group-num-max <max group num> |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set multicast group-num-max of ont port |
| <Max group num> | The max multicast group number,range for 0-255 |

【Example】

Example 1：Set the max multicast group number of eth1 as 20.

OLT(config-epon-srvprofile-10)#port eth 1 group-num-max 20 OLT(config-epon-srvprofile-10)#

Config ONU Port IGMP VLAN Process Mode in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#port eth <port-list> multicast-tagstrip {tag¦untag¦translation} |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set vlan processing mode of ont port’s multicast stream. |
| tag¦untag¦translation | Tag:Add VLAN tag Untag:Peel off VLAN tag Translation:translates multicast vlan to another vlan. |

【Example】

Example 1：Set multicast vlan processing mode of eth1 as tag mode in ont-srvprofile.

OLT(config-epon-srvprofile-10)#port eth 1 multicast-tagstrip tag OLT(config-epon-srvprofile-10)#

Config ONU IGMP Fast-leave Function in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#multicast fast-leave {enable¦disable} |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set ONU multicast fast-leave |
| {enable¦disable} | Enable:enable multicast fast-leave Disable:disable multicast fast-leave |

【Example】

Example 1：Enable multicast fast-leave

OLT(config-epon-srvprofile-10)#multicast fast-leave enable OLT(config-epon-srvprofile-10)#

Enable or Disable ONU Loop Detect Function in ont-lineprofile

| Command | OLT(config-epon-srvprofile-10)#ring check {enable¦disable} |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to set ring check function |
| {enable¦disable} | Enable:enable ring check function Disable:disable ring check function |

【Example】

Example 1：Enable ring check function in ont-srvprofile 10.

OLT(config-epon-srvprofile-10)#ring check enable OLT(config-epon-srvprofile-10)#

Show ont-srvprofile Current Configuration

| Command | OLT(config-epon-srvprofile-10)#show ont-srvprofile current |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to show current configuration of ont-srvprofile |

【Example】

Example 1：Show current configuration of ont-srvprofile

OLT(config-epon-srvprofile-10)#show ont-srvprofile current ----------------------------------------------------------------------------- Profile-ID:10 ---------------------------------------------------------------------------- Port-type Port-number --------------------------------------------------------------------------- ETH adaptive POTS 0 CATV 0 ---------------------------------------------------------------------------- Multicast fast leave switch:Enable Ring check switch:Enable ---------------------------------------------------------------------------- Port Port Up-traffic Down-traffic MAC-learn Classification type ID CAR-ID CAR-ID count --------------------------------------------------------------------------- ETH 1 20 20 10 None ETH 2 Unconcern Unconcern Unlimited None ETH 3 Unconcern Unconcern Unlimited None ETH 4 Unconcern Unconcern Unlimited None ETH 5 Unconcern Unconcern Unlimited None ETH 6 Unconcern Unconcern Unlimited None ETH 7 Unconcern Unconcern Unlimited None ETH 8 Unconcern Unconcern Unlimited None ---------------------------------------------------------------------------- Port Port Multicast Multicast Multicast Multicast type ID S-VLAN C-VLAN tag-strip group-num --------------------------------------------------------------------------- ETH 1--Tag 20 ETH 2--Tag 64 ETH 3--Tag 64 ETH 4--Tag 64 ETH 5--Tag 64 ETH 6--Tag 64 ETH 7--Tag 64 ETH 8--Tag 64 ---------------------------------------------------------------------------- Port Port Service-type Index N-VLAN N-PRI S-VLAN S-PRI C-VLAN C-PRI type ID --------------------------------------------------------------------------- ETH 1 Transparent------- ETH 2 Transparent------- ETH 3 Transparent------- ETH 4 Transparent------- ETH 5 Transparent------- ETH 6 Transparent-------

Commit ont-srvprofile Configuration

| Command | OLT(config-epon-srvprofile-10)#commit |
| --- | --- |
| View | Ont-srvprofile view |
| Description | This command is used to commit the current ont-srvprofile setting.All the parameter will take effect only after the command is committed. |

【Example】

Example 1：Commit current ont-srvprofile setting

OLT(config-epon-srvprofile-10)#commit OLT(config-epon-srvprofile-10)#

OLT traffic-profile Configuration

Create traffic-profile

| Command | OLT(config)#traffic-profile profile-id <profile id> profile-name <profile name> cir <parameter> pir <parameter> cbs <parameter> pbs <parameter> |
| --- | --- |
| View | Config view |
| Description | This command is used to create traffic-profile and set traffic-profile parameters to coordinate with rate limitation. |
| <Profile ID> | traffic-profile id,range for 1-256 |
| <profile name> | traffic-profile name,range for 1-16 |
| cir | Committed Information Rate,range for 64-10240000,unit is kbps |
| pir | Peek Information Rate,range for 64-10240000,unit is kbps |
| cbs | Committed Burst Size,range for 2000-10240000,unit is byte |
| pbs | Peek Burst Size,range for 2000-10240000,uni is byte |

【Example】

Example 1：Create traffic profile 10,name it as test1,set cir as 128,pir as 256,cbs as 2000,pbs as 3000.

OLT(config)#traffic-profile profile-id 10 profile-name test1 cir 128 pir 256 cbs 2000 pbs 3000 OLT(config)#

Modify traffic-profile

| Command | OLT(config)#traffic-profile modify {profile Id¦profile name} {¦cir¦pir¦cbs¦pbs} |
| --- | --- |
| View | Config view |
| Description | This command is used to modify the traffic-profile. |
| <Profile ID> | Profile id,range for 1-256 |
| <profile name> | Profile name,range for 1-16 |
| cir | Committed Information Rate,range for 64-10240000,unit is kbps |
| pir | Peek Information Rate,range for 64-10240000,unit is kbps |
| cbs | Committed Burst Size,range for 2000-10240000,unit is byte |
| pbs | Peek Burst Size,range for 2000-10240000,uni is byte |

【Example】

Example 1：Modify cir as 150 in traffic-profile 10.

OLT(config)#traffic-profile modify profile-id 10 cir 150 OLT(config)#

Show OLT traffic-profile Information

| Command | OLT(config)#show traffic-profile all¦profile id¦profile name |
| --- | --- |
| View | Config view |
| Description | This command is used to show traffic-profile info |
| <Profile ID> | Profile id,range for 1-256 |
| <Profile name> | Profile name,range for 1-16 |

【Example】

Example 1：Show all of traffic-profile info.

OLT(config)#show traffic-profile all ----------------------------------------------------------------------------- ID Profile-name CIR(kbps)PIR(kbps)CBS(bytes)PBS(bytes)Bind ---------------------------------------------------------------------------- 10 test1 150 256 2000 3000 0 20 test 222 222 2000 2000 2 ----------------------------------------------------------------------------- Total:2 OLT(config)#

OLT VLAN Pool Configuration

Create VLAN Pool

| Command | OLT(config)#vlan-pool add <vlan pool id> start-vid <vlan id> end-vid <vlan-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to create vlan-pool. |
| <VLAN pool ID> | Vlan-pool id,range for 1-128 |

【Example】

Example 1：Create vlan-pool 1,its range is start-vid 100 to end-vid 200.

OLT(config)#vlan-pool add 1 start-vid 100 end-vid 200 OLT(config)#

Modify VLAN Pool

| Command | OLT(config)#vlan-pool modify <vlan pool id> start-vid <vlan id> end-vid <vlan-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to modify the parameter of vlan-pool |
| <vlan pool id> | Vlan-pool id,range for 1-128 |

【Example】

Example 1：Modify vlan-pool 1,its range is start-vid 100 to end-vid 300.

OLT(config)#vlan-pool modify 1 start-vid 200 end-vid 300 OLT(config)#

Delete VLAN Pool

| Command | OLT(config)#no vlan-pool <vlan pool id> |
| --- | --- |
| View | Config view |
| Description | This command is used to delete vlan-pool. |
| <vlan pool id> | Vlan-pool id,range for 1-128 |

【Example】

Example 1：Delete vlan-pool 1.

OLT(config)#no vlan-pool 1 OLT(config)#

Show OLT VLAN Pool Information

| Command | OLT(config)#show vlan-pool {all¦<vlan pool id>} |
| --- | --- |
| View | Config view |
| Description | This command is used to show configuration info of vlan-pool. |
| <vlan pool id> | Vlan-pool id,range for 1-128 |

【Example】

Example 1：Show configuration info of all the vlan-pool.

OLT(config)#show vlan-pool all ----------------------------------------------------------------------------- VLAN-Pool Start-VID End-VID ---------------------------------------------------------------------------- 1 200 300 ----------------------------------------------------------------------------- OLT(config)#

ONU Management

ONU Authentication Management

Enable or Disable ONU Autofind Function

| Command | OLT(config-interface-epon-0/0)#ont autofind <port-id> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable ont autofind function in pon port |
| <port-id> | Pon port id,range for 1-16 |
| {enable¦disable} | Enable Disable |

【Example】

Example 1：Disable ont autofind function in pon 1.

OLT(config-interface-epon-0/0)#ont autofind 1 disable OLT(config-interface-epon-0/0)#

Show Autofind ONU

| Command | OLT(config-interface-epon-0/0)#show ont autofind <port-id> {all¦mac¦llid} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show basic info of autofind ont.when adding ont,it can show mac address,autofind time and etc info of unauthentication ont. |
| <port-id> | Pon port id,range for 1-16 |
| {all¦mac¦llid} | All:Show all the autofind ont in the pon port. Mac:Show the autofind ont according to the mac address in the pon port. Llid:Show the autofind ont according to the llid in the pon port. |

【Example】

Example 1：Show all the autofind ont in pon 1.

OLT(config-interface-epon-0/0)#show ont autofind 1 all ----------------------------------------------------------------------------- Index MAC Autofind-Time ---------------------------------------------------------------------------- 1 E0:67:B3:AA:BB:0C 2000-01-01 00:01:37 2 00:01:62:45:66:05 2000-01-01 08:02:24 3 00:01:62:45:99:07 2000-01-01 08:02:25 ----------------------------------------------------------------------------- Total:3 Total:3 ----------------------------------------------------------------------------- ----------------------------------------- OLT(config-interface-epon-0/0)#

MAC Address Method Authenticated ONU And Bind Profile

| Command | OLT(config-interface-epon-0/0)#ont add <port-id> <onu-id> mac-auth <mac-address> {ont-lineprofile-id<profile-id>¦ont-lineprofile-name<profile-name>} {ont-srvprofile-id <profile-id>¦ont-srvprofile-name<profile-name>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to authenticate onu by mac-auth and bind the ont-lineprofile and ont-srvprofile.olt will check whether the mac reported by ont is the same with the setting mac,if yes,ont will online normally. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <mac-address> | Mac address of ont,format for xx:xx:xx:xx:xx:xx |
| <profile-id> | Profile id,0-512 |
| <profile-name> | Profile name,it supports 1-16 strings. |

【Example】

Example 1：Add an ont in pon1 and bind it to ont-lineprofile 5 and ont-srvprofile 5.

OLT(config-interface-epon-0/0)#ont add 1 9 mac-auth 11:11:11:11:11:11 ont-lineprofile-id 5 ont-srvprofile-id 5 OLT(config-interface-epon-0/0)#

Loid Method Authenticated ONU and Bind Profile

| Command | OLT(config-interface-epon-0/0)#ont add <port-id> <onu-id> loid-auth <loid> { ont-lineprofile-id <profile-id>¦ ont-lineprofile-name } {<profile-name> ont-srvprofile-id <profile-id>¦ont-srvprofile-name <profile-name> } { always ¦ once-aging ¦ once-no-aging} |
| --- | --- |
| View | epon view |
| Description | This command is used to authenticate onu by loid-auth and bind the ont-lineprofile and ont-srvprofile.olt will check whether the loid reported by ont is the same with the setting loid,if yes,ont will online normally. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <loid> | Loid of ont,it supports 1-24 strings |
| <profile-id> | Profile id,0-512 |
| <profile-name> | Profile name,it supports 1-16 strings. |
| {always¦once-aging¦once-no-aging} | Always:ont can be authenticated at any time once-aging:ont only can be authenticated for once time within the allotted time,ont will be cleared once it is offline. once-no-aging:no limitation in authentication time for ont,but it will be cleared once it is offline. |

【Example】

Example 1：Add an ont in pon1 and bind it to ont-lineprofile 5 and ont-srvprofile 5.

OLT(config-interface-epon-0/0)#ont add 1 10 loid-auth loid ont-lineprofile-id 5 ont-srvprofile-id 5 always OLT(config-interface-epon-0/0)#

Loid+Password Method Authenticated ONU and Bind Profile

| Command | OLT(config-interface-epon-0/0)#ont add <port-id> <onu-id> loid-auth <loid> password <password> { ont-lineprofile-id <profile-id>¦ ont-lineprofile-name <profile-name> } {ont-srvprofile-id <profile-id>¦ont-srvprofile-name <profile-name> } { always ¦ once-aging ¦ once-no-aging} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to authenticate onu by loid-auth and password-auth and bind the ont-lineprofile and ont-srvprofile.olt will check whether the loid and password reported by ont is the same with the setting loid and password,if yes,ont will online normally. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <loid> | Loid of ont,it supports 1-24 strings |
| <password> | password of ont,it supports 1-12 strings |
| <profile-id> | Profile id,0-512 |
| <profile-name> | Profile name,it supports 1-16 strings. |
| always¦once-aging¦once-no-aging | Always:ont can be authenticated at any time once-aging:ont only can be authenticated for once time within the allotted time,ont will be cleared once it is offline. once-no-aging:no limitation in authentication time for ont,but it will be cleared once it is offline. |

【Example】

Example 1：Add an ont in pon1 and bind it to ont-lineprofile 5 and ont-srvprofile 5.

OLT(config-interface-epon-0/0)#ont add 1 10 loid-auth test password-auth test ont-lineprofile-id 5 ont-srvprofile-id 5 always OLT(config-interface-epon-0/0)#

Config ONU Authmode

| Command | OLT(config-interface-epon-0/0)#ont authmode <port-id><auth-mode> |
| --- | --- |
| View | epon view |
| Description | This command is used to set ont authmode in pon port. |
| <port-id> | Pon port id,range for 1-16 |
| <auth-mode> | Adaptive:Adaptive policy authenticated Auto:All ont will be authenticated by mac address unconditional Loid:authenticate ont by loid Loid-password:authenticate ont by loid+password Mac:authenticate ont by mac address Mac-or-loid:authenticate ont by mac address or loid Mac-or-loid-password:authenticate ont by mac addressor loid+password |

【Example】

Example 1：Set the authmode of pon 1 as mac.

OLT(config-interface-epon-0/0)#ont authmode 1 mac OLT(config-interface-epon-0/0)#

Change ONU Bind Profile

| Command | OLT(config-interface-epon-0/0)#ont modify <port-id> {<onu-id>¦<onu-list>} {ont-lineprofile-id <profile-id> ¦ont-lineprofile-name <profile-name> ¦ont-srvprofile-id <profile-id>¦ ont-srvprofile-name <profile-name>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to modify the bound ont-lineprofile and ont-srvprofile of ont. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id>¦<onu-llist> | Onu-id:ont id to be modified,range for 1-64 Onu-list:ont list to be modified,format for 1,3-5,8 |
| <profile-id> | profile id,range for 0-512 |
| <profile-name> | Profile name,it supports 1-16 strings. |

【Example】

Example 1：Modify the ont-lineprofile of ont 1 in pon 1 as ont-lineprofile 5

OLT(config-interface-epon-0/0)#ont modify 1 1 ont-lineprofile-id 5 OLT(config-interface-epon-0/0)#

Example 2:Modify the ont-lineprofile and ont-srvprofile of ont 1 in pon 1 as ont-lineprofile 5 and ont-srvprofile 5.

OLT(config-interface-epon-0/0)#ont modify 1 1 ont-lineprofile-id 5 ont-srvprofile-id 5 OLT(config-interface-epon-0/0)#

Config Pre-binding ONU Profile

| Command | OLT(config-interface-epon-0/0)ont predetermine <port-id> {< onu-id >¦<onu-list>} { ont-lineprofile-id <profile-id> ¦ont-lineprofile-name <profile-id>} {ont-srvprofile-id <profile-name>¦ ont-srvprofile-name <profile-name>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to predetermine the ont-lineprofile and ont-srvprofile for unregister ont in pon port,these profile will be applied to ont once it is online. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id>¦<onu-llist> | Onu-id:ont id,range for 1-64 Onu-list:ont list,format for 1,3-5,8 |
| <profile-id> | profile id,range for 0-512 |
| <profile-name> | Profile name,it supports 1-16 strings. |

【Example】

Example 1：Predetermine the ont-lineprofile 5 and ont-srvprofile 5 for ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont predetermine 1 1 ont-lineprofile-id 5 ont-srvprofile-id 5 OLT(config-interface-epon-0/0)#

Confirm Autofind ONU

| Command | OLT(config-interface-epon-0/0)#ont confirm <port-id> {all ¦mac-auth <mac-address> ¦ loid-auth <loid>} {password-auth <password>} {ont-lineprofile-id <profile-id> ¦ ont-lineprofile-name <profile-name> ont-srvprofile-id <profile-id> ¦ ont-srvprofile-name <profile-name>} |
| --- | --- |
| View | epon view |
| Description | This command is used to confirm the discovered ont.If ont autofind is enable,olt will obtain the register info of ont after accessing the ont to olt,at this time,ont is in‘autofind’state.ont will turn into working state and can be configured after confirming.Batch of ont registration is supported. |
| <port-id> | Pon port id,range for 1-16 |
| <mac-address> | Mac address of ont,format is xx:xx:xx:xx:xx:xx |
| <loid> | Loid of ont,it supports 1-24 strings |
| <password> | password of ont,it supports 1-12 strings |
| <profile-id> | Profile id,0-512 |
| <profile-name> | Profile name,it supports 1-16 strings. |

【Example】

Example 1：Authenticate all the autofind ont in pon1 according to mac address.

OLT(config-interface-epon-0/0)#ont confirm 1 all mac-auth Number of ONUs that can be added:0,success:0 OLT(config-interface-epon-0/0)#

Example 2:Authenticate all the autofind ont in pon1 according to loid“test”.

OLT(config-interface-epon-0/0)#ont confirm 1 loid-auth test Add port 1 ONU 1 successfully. OLT(config-interface-epon-0/0)#

Delete Authentication ONU

| Command | OLT(config-interface-epon-0/0)#ont delete <port-id> {<onu-id>¦all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to delete the ont in pon port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| all | Delete all ont in pon port |

【Example】

Example 1：Delete all ont in pon1.

OLT(config-interface-epon-0/0)#ont delete 1 all This command will delete all the ONUs in port.Are you sure to execute this command?(y/n):y 2000-01-04 11:32:30 ONU 0/0 1 1 is offline Number of ONUs that can be delete:1,success:1 OLT(config-interface-epon-0/0)#2000-01-04 11:32:30 PON 0/0/1 ONU 1 onu is offline

Cancel Autofind ONU

| Command | OLT(config-interface-epon-0/0)#ont cance l<port-id> {<mac-address>¦all} |
| --- | --- |
| View | epon view |
| Description | This command is used to cancel the autofind ont |
| <port-id> | Pon port id,range for 1-16 |
| <mac-address> | Mac saddress of ont,format is xx:xx:xx:xx:xx:xx |
| all | All the autofind ont in corresponding pon port |

【Example】

Example 1：Cancel the autofind ont in pon1,its mac address is E0:67:B3:12:11:8A.

OLT(config-interface-epon-0/0)#ont cancel 1 E0:67:B3:12:11:8A OLT(config-interface-epon-0/0)#

Re-register ONU

| Command | OLT(config-interface-epon-0/0)#ont re-register <port-id> {<onu-id>¦all} |
| --- | --- |
| View | epon view |
| Description | This command is used to re-register the specified ont or all the ont in corresponding pon port. |
| <port-id> | Pon port id,range for 1-16 |
| {<onu-id>¦all} | onu-id:ont id to be re-register,range for 1-64 All:re-register all the ont in pon port |

【Example】

Example 1：Re-register onu1 in pon 1.

OLT(config-interface-epon-0/0)#ont re-register 1 1 2000-01-08 02:02:49 PON 0/0/1 ONU 1 onu is offline 2000-01-08 02:02:49 PON 0/0/1 pon port link down 2000-01-08 02:02:56 PON 0/0/1 ONU 1 onu is online 2000-01-08 02:02:56 PON 0/0/1 pon port link up OLT(config-interface-epon-0/0)#

Modify ONU Authmode

| Command | OLT(config-interface-epon-0/0)#ont modify <port-id> <onu-id> auth-type {mac-auth <mac-address> ¦ loid-auth <loid>} {password-auth <password>} |
| --- | --- |
| View | Config view |
| Description | This command is used to modify ont authmode |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <mac-address> | Mac address of ont,format is xx:xx:xx:xx:xx:xx |
| <loid> | Loid of ont,it supports 1-24 strings |
| <password> | password of ont,it supports 1-12 strings |

【Example】

Example 1：Modify the auth-type of onu 1 as mac-auth in pon1,its mac is 11:11:11:11:11:11.

OLT(config-interface-epon-0/0)#ont modify 1 1 auth-type mac-auth 11:11:11:11:11:11 OLT(config-interface-epon-0/0)#

Example 2:Modify the auth-type of onu 1 as loid-auth in pon1,its loid is test.

OLT(config-interface-epon-0/0)#ont modify 1 1 auth-type loid-auth test OLT(config-interface-epon-0/0)#

Example 3:Modify the auth-type of onu 1 as loid-password in pon1,its loid is test,password is test1.

OLT(config-interface-epon-0/0)#ont modify 1 1 auth-type loid-auth test password-auth test1 OLT(config-interface-epon-0/0)#

Delete Blacklist Auth ONU

| Command | OLT(config-interface-epon-0/0)#ont blacklist del <port-id> {<mac-address>¦all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to delete the blacklist ont in specified pon port. |
| <port-id> | Pon port id,range for 1-16 |
| <mac-address> | Mac address of blacklist ont,format for xx:xx:xx:xx:xx:xx |
| all | All of the blacklist ont in the pon port |

【Example】

Example 1：Delete all the blacklist ont in pon 1.

OLT(config-interface-epon-0/0)#ont black-list del 1 all Number of ONUs that can be delete:1,success:1 OLT(config-interface-epon-0/0)#

Show ONU BlackList Authenticated Config

| Command | OLT(config-interface-epon-0/0)#show ont black-list {<port-id> all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show black-list ont in specified pon port |
| <port-id> | Pon port id,range for 1-16 |

【Example】

Example 1：Show all the black-list ont in pon 1.

OLT(config-interface-epon-0/0)#show ONU black-list 1 all ----------------------------------------------------------------------------- Index MAC Try-Count Last-Auth-Time ---------------------------------------------------------------------------- 1 12:12:12:12:12:12 0 1900-01-00 00:00:00 ----------------------------------------------------------------------------- Total:1 OLT(config-interface-epon)#

Show ONU Registered Status and Information

| Command | OLT(config-interface-epon-0/0)#show ont infor <port-id>< {onu-id>¦all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show ont state info(including current state,relative configuration of ont) Port:The pon port id that the ont had registered in ONU ID:the setting ont id MAC:Mac address of ont Control flag： Active:Ont is in active state.Only if the ont is in active state the ont can register in olt. Deactive:Ont is in deactive state.When ont is in deactive state,we can active the ont by“ont active”command. Run state:The running sigh of ont,it includes“online”,“offline”,“online”state indicates that ont has registered in olt normally. Config state:This state can describe whether the config state is in normal.it includes 3 kinds of state: Initial:Ont is in configuration issuing or recovering state Failed:Failed to issue or recover the configuration Success:Success to issue or recover the configuration |
| <port-id> | Pon port id,range for 1-16 |
| {<onu-id> ¦all} | onu-id:ont id to be show,range for 1-64。 All:Show all of the ont state in current pon port |

【Example】

Example1:Show all of the ont state in pon 1.

OLT(config-interface-epon-0/0)#show ont info 1 all ----------------------------------------------------------------------------- F/S P ONU MAC Control Run Config Match Desc ID flag state state state ---------------------------------------------------------------------------- 0/0 1 1 E0:67:B3:00:00:09 active online success match 0/0 1 2 00:13:25:00:00:01 active offline initial initial 0/0 1 3 E0:67:B3:1B:8F:35 active offline initial initial 0/0 1 4 00:13:25:01:02:01 active offline initial initial ----------------------------------------------------------------------------- Total:4,online 1 OLT(config-interface-epon-0/0)#

Policy-auth ONU and Batch Delivery Configuration Manage

Enable or Disable Global Policy-auth ONU

| Command | OLT(config)# ont policy-auth {enable¦disable} |
| --- | --- |
| View | config view |
| Description | This command is used to enable or disable the global ONU policy authentication function. The default policy adopted by the OLT is that the ONU defaults to match the line profile 0 and the service profile. The OLT supports the following methods to match different ONUs, and then match different configurations for different ONUs. |
| {enable¦disable} | Enable:Enable policy-auth of ont Disable:Disable policy-auth of ont |

【Example】

Example 1： Enable global ONU policy authentication.

OLT(config)# ont policy-auth enable OLT(config)#

Enable or Disable Policy-auth ONU Under PON Port

| Command | OLT(config-interface-epon-0/0)#ont policy-auth <port-id> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable policy-auth of ont under OLT PON port. |
| <port-id> | Pon port id,range for 1-16 |
| {enable¦disable} | Enable:Enable policy-auth of ont Disable:Disable policy-auth of ont |

【Example】

Example 1：Enable policy-auth of ont in pon1

OLT(config-interface-epon-0/0)#ont policy-auth 1 enable OLT(config-interface-epon-0/0)#

Config Policy-auth ONU Mode

| Command | OLT(config)# ont policy-auth mode {all ¦model-auth ¦model-swver-auth ¦model-vendor-hwver-auth¦vendor-auth} to {loid-auth ¦loid-password-auth ¦mac-auth} {always¦once-no-aging} |
| --- | --- |
| View | Config view |
| Description | This command is used to configure the ONU policy authentication mode. Mainly configures what the ONU is based on to match the policy. The ONU is based on what is registered, and the number of times the policy action is executed. |
| <all ¦model-auth ¦model-swver-auth ¦model-vendor-hwver-auth¦vendor-auth > | All: The object that executes the policy is all online ONUs. Model-auth : Match different strategies model-swver-auth according to different model ids: match different strategies according to different model id and software version Model-vendor-hwver-auth: Match different strategies according to different model id, software version and hardware version Vendor-auth : match different strategies based on different vendor ids |
| <loid-auth ¦loid-password-auth ¦mac-auth> <always¦once-no-aging> | Mac-auth: onu is registered based on mac address Loid-auth: onu is registered based on loid id Loid-password-auth:onu is registered based on loid id and password Always: the action of executing the policy is always executed Once-no-aging: the action of executing the policy is executed only once |

【Example】

Example 1： Configure the ONU to match the policy by model id, register based on mac address, and always execute the policy.

OLT(config)# ont policy-auth mode model-auth to mac-auth always OLT(config)#

Config Policy-auth ONU Mode and Batch Delivery Configuration Match Mode

| Command | OLT(config)# ont policy-auth {all ¦model-auth ¦model-swver-auth ¦model-vendor-hwver-auth¦vendor-auth } <parameter> { ont-lineprofile-id <profile-id> ¦ont-lineprofile-name <profile-id>} {ont-srvprofile-id <profile-name>¦ ont-srvprofile-name <profile-name>} |
| --- | --- |
| View | Config view |
| Description | This command is used to configure the matching policy of the ONU, configure specific parameters to match the policy, and configure line profiles and service templates that match the ONUs of the policy. |
| <all ¦model-auth ¦model-swver-auth ¦model-vendor-hwver-auth¦vendor-auth > | All: The object that executes the policy is all online ONUs. Model-auth : Match different strategies model-swver-auth according to different model ids: match different strategies according to different model id and software version Model-vendor-hwver-auth: Match different strategies according to different model id, software version and hardware version Vendor-auth : match different strategies based on different vendor ids |
| <ont-lineprofile-id¦ont-lineprofile-name> | Configure the line profile id or name matching the online ONU that matches the policy. |
| <ont-srvprofile-id¦ont-srvprofile-name> | Configure the service template id or name matching the online ONU that matches the policy. |

【Example】

Example 1： Configure the ONU on the line that has the model id of 0x31303053 to automatically match the line profile 10 and the service profile 10.

OLT(config)# ont policy-auth policy model-auth 0x31303053 ont-lineprofile-id 10 ont-srvprofile-id 10 OLT(config)#

Example 2: The ONU of the PON1 with the model id of 0x31303053 is configured based on the mac address authentication. The line-up always matches the line profile 10 and the service profile 10 automatically. All the configurations required are as follows:

OLT(config)# ont policy-auth enable OLT(config)# ont policy-auth mode model-auth to mac-auth always OLT(config)# ont policy-auth policy model-auth 0x31303053 ont-lineprofile-id 10 ont-srvprofile-id 10 OLT(config)# interface epon 0/0 OLT(config-interface-epon-0/0)# ont policy-auth 1 enable OLT(config-interface-epon-0/0)# ont authmode 1 adaptive OLT(config-interface-epon-0/0)# ont autofind 1 enable

Show Policy-auth ONT Configuration Information

| Command | OLT(config)# show ont policy-auth |
| --- | --- |
| View | Config view |
| Description | This command is used to view the configuration of the current ONU policy authentication of the OLT. |

【Example】

Example 1： Check the configuration of the current ONU policy authentication of the OLT.

OLT(config)# show ont policy-auth ----------------------------------------------------------------------------- Policy-auth Switch : enable Policy-auth Mode : model-auth Target auth Mode : mac-auth Time Mode : always ----------------------------------------------------------------------------- OLT(config)#

ONU Basic Function Management

Active ONU

| Command | OLT(config-interface-epon-0/0)#ont activate <port-id> {<onu-id>¦all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to active the ont with disactive state.Ont will work in normal only when it is in active state.ont is in active state by default. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id>¦all | Ont id,range for 1-64 All:all of the ont |

【Example】

Example 1：Active the ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont activate 1 1 OLT(config-interface-epon-0/0)#

Deactive ONU

| Command | OLT(config-interface-epon-0/0)#ont deactivate <port-id> {<onu-id>¦all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to disactive the ont with active state.Ont will work in normal only when it is in active state.ont is in active state by default. |
| <port-id> | Pon port id,range for 1-16 |
| {<onu-id>¦all} | Ont id,range for 1-64 All:all of the ont |

【Example】

Example 1：Disactive the ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont deactivate 1 1 OLT(config-interface-epon-0/0)#

Reboot ONU

| Command | OLT(config-interface-epon-0/0)#ont reboot <port-id> {<onu-id>¦all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to reboot the specified ont or all of the ont. |
| <port-id> | Pon port id,range for 1-16 |
| {<onu-id>¦all} | Ont id,range for 1-64 All:all of the ont |

【Example】

Example 1：Reboot the ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont reboot 1 1 OLT(config-interface-epon-0/0)#

ONU Remote Manage IP Function Config

| Command | OLT(config-interface-epon-0/0)#ont ipconfig <port-id> < onu-id > ip-address <ip-address> mask <mask> gateway <gateway-ip> manage-vlan <vlan-id> priority <priority> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the management ip,netmask,gateway,manage vlan and priority and etc. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <ip-address> | Manage ip,format for xx:xx:xx:xx:xx:xx |
| <mask> | Netmask,format for xx:xx:xx:xx:xx:xx |
| <gateway-ip> | Gateway ip,format for xx:xx:xx:xx:xx:xx |
| <vlan-id> | vlan id.range for 1-4094 |
| <priority> | Priority,range for 0-7 |

【Example】

Example 1：Set the parameters of ont 1 in pon 1,set its management ip as 192.168.101.1,netmask as 255.255.255.0,gateway ip is 192.168.101.254,management vlan as 101,priority as 0.

OLT(config-interface-epon-0/0)#ont ipconfig 1 1 ip-address 192.168.101.1 mask 255.255.255.0 gateway 192.168.101.254 manage-vlan 101 priority 0 OLT(config-interface-epon-0/0)#

ONU Upstream Rate Limit Function Config

| Command | OLT(config-interface-epon-0/0)#ont sla <port-id> < onu-id > upstream {assure <assured-bandwidth> ¦ fix <fixed-bandwidth> ¦ max <max-bandwidth>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set upstream rate limitation of ont |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <assured-bandwidth> | Assure-bandwidth,range for 1-1000000,unit is kpbs |
| <fixed-bandwidth> | Fixed-bandwidth,range for 1-1000000,unit is kpbs |
| <max-bandwidth> | Max-bandwidth,range for 512-1000000,unit is kpbs |

【Example】

Example 1：Set the fixed-bandwidth of ont 1 in pon1 as 100000kbps

OLT(config-interface-epon-0/0)#ont sla 1 1 upstream fix 100000 OLT(config-interface-epon-0/0)#

ONU Downstream Rate Limit Function Config

| Command | OLT(config-interface-epon-0/0)#ont sla <port-id> < onu-id > downstream max <max-bandwidth> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set downstream rate limitation of ont |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <max-bandwidth> | Max-bandwidth,range for 64-1000000,unit is kpbs |

【Example】

Example 1：Set the max-bandwidth of ont 1 in pon1 as 100000kbps

OLT(config-interface-epon-0/0)#ont sla 1 1 downstream max 100000 OLT(config-interface-epon-0/0)#

ONU MAC Aging Time Config

| Command | OLT(config-interface-epon-0/0)#nt mac-aging <port-id> <onu-id> aging-time <aging-time> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set mac-aging time of specified ont |
| <port-id> | Pon port id,range for 1-16 |
| <ont-id> | Ont id,range fro 1-64 |
| <aging-time> | Mac-aging time,range for 0-36000.”0”means without aging. |

【Example】

Example 1：Set the mac-aging time of onu 4 in pon 1 as 12s.

OLT(config-interface-epon-0/0)#ont mac-aging 1 4 aging-time 12 OLT(config-interface-epon-0/0)#

Add ONU Description Information

| Command | OLT(config-interface-epon-0/0)#ont description <port-id> <onu-id> <description> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to add description for ontt |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <description> | Description info of ont,it supports 1-64 strings. |

【Example】

Example 1：Add description“test”for ont 1 in pon1.

OLT(config-interface-epon-0/0)#ont description 1 1 test OLT(config-interface-epon-0/0)#

ONU Encryption Function Config

| Command | OLT(config-interface-epon-0/0)#ont encrypt <port-id> <onu-id> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set encryption function of ont.the message between specified pon port and specified ont will be encrypted after executing this command. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| {enable¦disable} | Enable:enable the encryption function Disable:disable the encryption function |

【Example】

Example 1：Enable the encryption function of onu 1 in pon 1.

OLT(config-interface-epon-0/0)#ont encrypt 1 1 enable OLT(config-interface-epon-0/0)#

Enable or Disable ONU FEC Function

| Command | OLT(config-interface-epon-0/0)#ont fec <port-id> <onu-id> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable FEC function of specified ont in pon port. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| {enable¦disable} | Enable:enable FEC function Disable:disable FEC function |

【Example】

Example 1：enable FEC function of ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont fec 1 1 enable OLT(config-interface-epon-0/0)#

ONU Service-sla Function Config

| Command | OLT(config-interface-epon-0/0)#ont service-sla <port-id> {all ¦ <onu-id>} {profile-id <profile-id>¦profile-name <profile-name>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to bind ont-slaprofile to ont.Firstly,it needs to create a n ont-slaprofile in config view. |
| <port-id> | Pon port id,range for 1-16 |
| {all¦<onu-id>} | Ont id,range for 1-64 All:all of the ont |
| <profile-id> | Profile id,range for 0-256 |
| <profile-name> | profile name,it supports 1-16 strings |

【Example】

Example 1：Bind ont-slaprofile 5 to ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont service-sla 1 1 profile-id 5 OLT(config-interface-epon-0/0)#

ONU SNMP Function Config

| Command | OLT(config-interface-epon-0/0)#ont snmp-config <port-id> < onu-id > v2c <readgroup> <writegroup> <trap-ip> <snmp-port> { <trap-port> <security-name> } |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set snmp parameters of ONU |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <readgroup> | read group,it supports 1-32 strings |
| <writegroup> | write group,it supports 1-32 strings |
| <trap-ip> | Trap ip,format for xx:xx:xx:xx:xx:xx |
| <snmp-port> | Snmp port,range for 1-65535 |
| <trap-port> | Trap port,range for 1-65535 |
| <security-name> | Security-name,it supports 1-32 strings |

【Example】

Example 1：Set snmp parameters of ONU,Set read group as public,write group as private,trap ip as 192.168.5.200,snmp port is 23,trap port is 161,security name is test.

OLT(config-interface-epon-0/0)#ont snmp-config 1 1 v2c public private 192.168.5.200 23 161 test OLT(config-interface-epon-0/0)#

Enable or Disable ONU Tx Optical Power Supply

| Command | OLT(config-interface-epon-0/0)#ont tx-power-supply <port-id> <onu-id> {enable/disable} {<disable-time>¦forever} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the tx-power-supply limitation of specified ont and its shining time. |
| <port-id> | Pon port id,range for 1-16 |
| <ont-id> | Ont id,range fro 1-64 |
| {enable¦disable} | Enable:ont shine in normal Disable:ont unshining |
| <disable-time>¦forever | <disable-time>:unshining time of ont,range for 1-65534s Forever:make onu unshining forever |

【Example】

Example 1：Set ont 3 in pon 1 un-shining for 3s.

OLT(config-interface-epon-0/0)#ont tx-power-supply 1 3 disable 3 OLT(config-interface-epon-0/0)# 2000-01-01 00:07:06 PON 0/0/1 ONU 3 onu is offline 2000-01-01 00:07:10 PON 0/0/1 ONU 3 onu is online

Show ONU ipconfig(Remote Manage IP Address)

| Command | OLT(config-interface-epon-0/0)#show ont ipconfig <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show ont ipconfig,it includes manage ip,gateway,manage vlan and etc. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show ipconfig of ont 1 in pon 1.

OLT(config-interface-epon-0/0)#show ont ipconfig 1 1 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:1 ONU-ID:1 IP:192.168.1.1 Subnet mask:255.255.255.0 Gateway:192.168.1.100 Manage VLAN:100 Manage priority:5 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU SNMP Configuration

| Command | OLT(config-interface-epon-0/0)#show ont snmp-config <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show snmp configuration of ont |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show snmp configuration of ont 1 in pon 1.

OLT(config-interface-epon-0/0)#show ont snmp-config 1 1 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:1 ONU-ID:1 Version:v2c Read group name:public Write group name:private Destination IP:192.168.5.200 Snmp port:162 Trap port:162 SNMP body name: ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU Capability Information

| Command | OLT(config-interface-epon-0/0)#show ont capability <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show ont capability info,including ont port type,number and etc. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show ont 3 capability info in pon 1.

OLT(config-interface-epon-0/0)#show ont capability 1 3 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:1 ONU-ID:3 Type:SFU Number of uplink PON ports:1 Number of ETH ports:4(4FE+0GE) Number of POTS ports:0 Number of CATV ports:1 Number of uplink queues:8 MAX number of uplink queues:8 Number of downlink queues:8 MAX number of downlink queues:8 Number of LLID:1 Fast leave ability:non-fast-leave in snooping mode fast-leave in snooping mode non-fast-leave in CTC mode fast-leave in CTC mode non-fast-leave in MLD snooping mode fast-leave in MLD snooping mode ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU Configured Capability

| Command | OLT(config-interface-epon-0/0)#show ont config-capability <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show ont config-capability.It can check whether the ability is matched according to compare config-capability with capability. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show config-capability of ont 3 in pon 1.

OLT(config-interface-epon-0/0)#show ont config-capability 1 3 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:1 ONU-ID:3 Number of ETH ports:adaptive Number of POTS ports:adaptive Number of CATV ports:adaptive ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU Opitcal Power Information

| Command | OLT(config-interface-epon-0/0)#show ont optical-info <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show optical info of ont in pon port.Generally,it is used to routine maintenance and troubleshooting of ont,it can show optical info of ont to check whether the optical module is in normal. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show optical info of ont 3 in pon 3.

OLT(config-interface-epon-0/0)#show ont optical-info 1 3 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:1 ONU-ID:3 ONU-MAC:E0:67:B3:31:85:8A Voltage(V):3.34 Tx optical power(dBm):1.72 Rx optical power(dBm):-3.73 Laser bias current(mA):18.20 Temperature(C):45.07 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU Firmware Version

| Command | OLT(config-interface-epon-0/0)#show ont version <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show ont version info,it can show software,hardware,vendor and etc info of ont. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show version info of ont 3 in pon 1.

OLT(config-interface-epon-0/0)#show ont version 1 3 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:1 ONU-ID:3 Vendor-ID:PON OUI Version:CTC3.0 ONU model:572R(0x35373252) Extended model:GT832T_R ONU mac address:E0:67:B3:31:85:8A ONU hardware version:V1.2 ONU software version:V2.1.5.b3 ONU chipset vendor ID:RL ONU chipset model:9603 ONU chipset revision:01 ONU chipset version/date:00.00.07 ONU firmware version:0x56322e312e352e623320202d -----------------------------------------------------------------------------

ONU Port VLAN Configuration(Non-template Mode)

ONU Port Native-vlan(access) Configs

| Command | OLT(config-interface-epon-0/0)#ont port native-vlan <port-id> <onu-id> eth <eth-port-id> vlan <vlan-id> priority <priority> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set native vlan of ont port.Caution:a corresponding vlan must be set in ont port firstly by the command of“ont port vlan”. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| <vlan-id> | Vlan id,range for 1-4094 |
| <priority> | Priority,range for 0-7 |

【Example】

Example1:Set the native vlan of ont 1 eth 1 in pon 1 as 100,and set its priority as 3.

OLT(config-interface-epon-0/0)#ont port native-vlan 1 1 eth 1 vlan 100 priority 3 OLT(config-interface-epon-0/0)#

ONU Port Vlan Config

| Command | OLT(config-interface-epon-0/0)#ont port vlan <port-id> <onu-id> eth <eth-port-id> <vlan-id> priority <priority> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set ont port vlan. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| <vlan-id> | Vlan id,range for 1-4094 |
| <priority> | Priority,range for 0-7 |

【Example】

Example 1：Set the port vlan of ont 1 eth 1 in pon 1 as 100,and set its priority as 5.

OLT(config-interface-epon-0/0)#ont port vlan 1 1 eth 1 100 5 OLT(config-interface-epon-0/0)#

ONU Port Translation Mode VLAN Config

| Command | OLT(config-interface-epon-0/0)#ont port vlan <port-id> <onu-id> eth <eth-port-id> translation <service-vlan-id> <priority> <customer-vlan-id> <priority> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set vlan translation of ont port,it translates the service vlan to customer vlan in downstream direction. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| <service-vlan-id> | service vlan id,range for 1-4094 |
| <customer-vlan-id> | Customer vlan id,range for 1-4094 |
| <priority> | Priority,range for 0-7 |

【Example】

Example 1：Set vlan mode of ont 1 eth 1 in pon 1 as translation and translate its service vlan 12 to customer vlan 13.

OLT(config-interface-epon-0/0)#ont port vlan 1 1 eth 1 translation 12 user-vlan 13 OLT(config-interface-epon-0/0)#

ONU Port Transparent Mode VLAN Config

| Command | OLT(config-interface-epon-0/0)#ont port vlan <port-id> <onu-id> eth <eth-port-id> transparent |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set vlan transparent mode of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |

【Example】

Example 1：Set vlan mode of ont 1 eth 1 in pon 1 as transparent.

OLT(config-interface-epon-0/0)#ont port vlan 1 1 eth 1 transparent OLT(config-interface-epon-0/0)#

Show ONU Port VLAN Configuration

| Command | OLT(config-interface-epon-0/0)#show ont port vlan <port-id> <onu-id> eth <onu-port id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show specified eth port vlan of ont |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <onu-port id> | Eth port id |

【Example】

Example 1：Show eth 1 port vlan of ont 2 in pon 1.

OLT(config-interface-epon-0/0)#show ont port vlan 1 2 eth 1 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:1 ONU ID:2 Port ID:1 ---------------------------------------------------------------------------- Service Profile ID:0 Service Profile name:srvprofile_0 ---------------------------------------------------------------------------- Service-type Index N-VLAN N-PRI S-VLAN S-PRI C-VLAN C-PRI --------------------------------------------------------------------------- Transparent------- ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

ONU Port Management

Enable or Disable ONU Port

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> operational-state {on/off} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable ont port.ont port can communicate normally when it is in enabling state,else it can’t. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| {on/off} | On:enable eth port Off:disable eth port |

【Example】

Example 1：Disable ont 1 eth 1 in pon 1.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 operational-state off OLT(config-interface-epon-0/0)#

ONU Port Isolate Function Config

| Command | OLT(config-interface-epon-0/0)#ont port-isolate <port-id> <onu-id> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable ont port isolation function.If it is enabled,port to port of ont can not communicate,else they can communicate with each other. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| {enable¦disable} | Enable:enable ont port isolation function Disable:disable ont port isolation function |

【Example】

Example 1：enable ont 1 port isolation function in pon 1.

OLT(config-interface-epon-0/0)#ont port-isolate 1 1 enable OLT(config-interface-epon-0/0)#

ONU PON Port Performance Statistics Config

| Command | OLT(config-interface-epon-0/0)#ont statistics <port-id> <onu-id> pon {enable¦disable} period<period> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set performance statistics of ont’s pon port and set its statistic period. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| {enable¦disable} | Enable:enable ont’s pon port performance statistic function Disable:disable ont’s pon port performance statistic function |
| <period> | The period of performance statistic,range for 1-864000,unit is second. |

【Example】

Example 1：Enable the performance statistic of ont1’s pon port in pon 1 and set its period as 100s.

OLT(config-interface-epon-0/0)#ont statistics 1 1 pon enable period 100 OLT(config-interface-epon-0/0)#

ONU ETH Port Performance Statistics Config

| Command | OLT(config-interface-epon-0/0)#ont statistics <port-id> <onu-id> eth <eth-port-id> {enable¦disable} period<period> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set performance statistics of ont’s eth port and set its statistic period. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| {enable¦disable} | Enable:enable ont’s eth port performance statistic function Disable:disable ont’s eth port performance statistic function |
| <period> | The period of performance statistic,range for 1-864000,unit is second. |

【Example】

Example 1：Enable the performance statistic of ont1’s eth 1 in pon 1 and set its period as 100s.

OLT(config-interface-epon-0/0)#ont statistics 1 1 eth 1 enable period 100 OLT(config-interface-epon-0/0)#

ONU CATV Port Enable or Disable

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> catv <catv-port-id> operational-status {on/off} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable CATV port of ont |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <catv-port-id> | CATV port id,value is 1 |
| {on/off} | On:enable CATV port Off:disable CATV port |

【Example】

Example1:Enable CATV port of ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 catv 1 operational-state on OLT(config-interface-epon-0/0)#

ONU Port Auto-negotiation Function Config

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> auto-neg {restart} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| restart | Optional parameter,it is used to reset the autonegotiation function |

【Example】

Example 1：Reset the autonegotiation function of ont 1 eth 1 in pon 1.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 auto-neg restart OLT(config-interface-epon-0/0)#

ONU Port Downstream Rate Limit Config

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> ds-policing {unconcern¦cir <cir-bandwidth> pir <pir-bandwidth>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set downstream rate limitation of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| Unconcern | It means that there is no limitation in this eth port. |
| <cir-bandwidth> | Committed Information Rate,range for 64-1024000,unit is kbps |
| <pir-bandwidth> | Peak Information Rate,range for 64-1024000,unit is kbps |

【Example】

Example 1：Set downstream rate limitation of ont 1 eth 1 in pon 1,cir is 10000kpbs,pir is 100000kpbs.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 ds-policing cir 10000 pir 100000 OLT(config-interface-epon-0/0)#

ONU Port Upstream Rate Limit Config

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> up-policing {unconcern¦cir <cir-bandwidth> cbs <cbs> ebs <ebs>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set upstream rate limitation of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| Unconcern | It means that there is no limitation in this eth port. |
| <cir-bandwidth> | Committed Information Rate,range for 64-1024000,unit is kbps |
| <cbs> | Committed Burst Size,range for 64-1024000,unit is kbps |
| <ebs> | Excess Burst Size,range for 2000-1024000,unit is kbps |

【Example】

Example 1：Set upstream rate limitation of ont 1 eth 1 in pon 1,cir is 10000kpbs,cbs is 1234kpbs,ebs is 2345kpbs.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 up-policing cir 10000 cbs 1234 ebs 2345 OLT(config-interface-epon-0/0)#

Enable or Disable ONU Port Flow Control Function

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> flow-control {on/off} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable flow control of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| {on/off} | On:enable flow control of ont port Off:disable flow control of ont port |

【Example】

Example 1：enable flow control of ont 1 eth 1 in pon 1.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 flow-control on OLT(config-interface-epon-0/0)#

Clear ONU Port Learned MAC

| Command | OLT(config-interface-epon-0/0)#ont port learned-mac-clear <port-id> <onu-id> eth <eth-port-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to clear the learned mac of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |

【Example】

Example 1：Clear the learned mac of ont 1 eth 1 in pon 1.

OLT(config-interface-epon-0/0)#ont port learned-mac-clear 1 1 eth 1 OLT(config-interface-epon-0/0)#

ONU Port MAC Count Limit Function Config

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> max-mac-count {<max-mac-count> ¦no-learning¦unlimited} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to limit the learned mac count of ont port. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| {<max-mac-count>¦no-learning¦unlimited} | Max-mac-count:range for 1-254 No-learning:no learning mac address Unlimited:unlimit the count of learned mac address |

【Example】

Example 1：Limit the learned mac address count of ont 1 eth 1 in pon 1 as 100 entries.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 max-mac-count 100 OLT(config-interface-epon-0/0)#

ONU Port Multicast Group Number Limit Config

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> multicast-max-group-num <number> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to limit the multicast group number of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| <number> | Max multicast group number,range for 0-255 |

【Example】

Example 1：Limit the multicast group number of ont 1 eth 1 in pon 1 as 5 entries.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 multicast-max-group-num 5 OLT(config-interface-epon-0/0)#

ONU Port Multicast VLAN Process Mode Config

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> multicast-tagstrip {tag¦untag¦translation} {<service-vlan><customer-vlan>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the multicast vlan mode of ont eth port. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| {tag¦untag¦translation} | Tag:no peeling off multicast VLAN tag Untag:peel off multicast VLAN tag Translation:translates the multicast vlan |
| <service-vlan> | Server-side VLAN,range for 1-4094 |
| <customer-vlan> | Customer VLAN,range for 1-4094 |

【Example】

Example 1：Set the multicast vlan mode of ont 1 eth 1 in pon 1 as tag.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 multicast-tagstrip untag OLT(config-interface-epon-0/0)#

Example 2:Set the multicast vlan mode of ont 1 eth 1 in pon 1 as translation,translate the server-side vlan 10 to customer vlan 11.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 multicast-tagstrip translation 10 11 OLT(config-interface-epon-0/0)#

ONU Port Speed and Duplex Mode Config

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> speed <speed> dulepx {full/half} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set the speed and duplex mode of ont port. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| <speed> | 10：10Mb/s 100：100Mb/s 1000：1000Mb/s |
| full/half | Full:full duplex Half:half duplex |

【Example】

Example 1：Set the speed of ont 1 eth 1 in pon 1 as 1000Mbps,and set its duplex mode as full.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 speed 1000 dulepx full OLT(config-interface-epon-0/0)#

ONU Port Storm-ctrl Function Config

| Command | OLT(config-interface-epon-0/0)#ont port attribute <port-id> <onu-id> eth <eth-port-id> xstorm-ctrl admin {disable/enable} type {all¦bcmc¦bcuc¦broadcast¦mcuc¦multicast¦unicast} rate<rate> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set strom-ctrl funcction of ont port and set its rate limitation |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| {disable¦enable} | Disable:disable storm-ctrl function Enable:enable storm-ctrl function |
| all¦bcmc¦bcuc¦broadcast¦mcuc¦multicast¦unicast | All:all of the broadcast message,multicast message,and unicast message Bcmc:broadcast message and multicast message Bcuc:broadcast message and unicast message Broadcast:broadcast message Mcuc:multicast message and unicast message Multicast:multicast message Unicast:unknown unicast message |
| <rate> | Rate,range for 8-16777215,unit is kbps |

【Example】

Example 1：Enable the storm-ctrl function of ont 1 eth 1 in pon 1,and set its unknown unicast rate limitation as 5000kbps.

OLT(config-interface-epon-0/0)#ont port attribute 1 1 eth 1 storm-ctrl admin enable type unicast rate 5000 OLT(config-interface-epon-0/0)#

Enable or Disable ONU Port Loop Detect Function

| Command | OLT(config-interface-epon-0/0)#ont ring check <port-id> <onu-id> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable the ring check of specified ont in pon port.After enabling this function,the olt will alarm if there is a ring in ont. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| {enable¦disable} | Enable Disable |

【Example】

Example 1：Enable ring check of ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont ring check 1 1 enable OLT(config-interface-epon-0/0)#

Enable or Disable ONU Port Loop Detected

Auto-shutdown Function

| Command | OLT(config-interface-epon-0/0)#ont ring check <port-id> <onu-id> auto-shutdown {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable auto-shutdown eth port when ring occurs in ont.After executing this command,if ont has a ring,ont will shutdown the port to prevent the ring from affecting the upper network.the default is“enable” |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| {enable¦disable} | Enable Disable |

【Example】

Example 1：Enable auto-shutdown eth port of ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont ring check 1 1 auto-shutdown enable OLT(config-interface-epon-0/0)#

ONU Port Upstream classification/mapping/priority-mark Rule Config

| Command | OLT(config-interface-epon-0/0)#ont port classification <port-id> < onu-id > eth <eth-port-id> acl <acl-id> rule <rule-id> { precedence <precedence> queue-mapped <queue-id> priority-mark <priority>¦no-priority-mark } |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set upstream classification/mapping/priority-mark rule of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| <acl-id> | Acl id,range for 9000-9499 |
| <rule-id> | Rule id,range for 1-8 |
| <precedence> | Priority,range for 0-255 |
| <queue-id> | Queue id,range for 0-7 |
| <priority>¦no-priority-mark | <priority>:priority mark,range for 0-7 no-priority-mark |

【Example】

Example 1：Map the ont 1 eth 1 in pon 1 to acl 9000 and rule 1.

OLT(config-interface-epon-0/0)#ont port classification 1 1 eth 1 acl 9000 rule 1 OLT(config-interface-epon-0/0)#

Show ONU Port Performance Statistics

| Command | OLT(config-interface-epon-0/0)#show ont statistics <port-id> <onu-id> eth <eth-port-id> status |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show performance statistics of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |

【Example】

Example 1：Show performance statistics of ont 1 eth 1 in pon 1.

OLT(config-interface-epon-0/0)#show ont statistics 1 1 eth 1 status ----------------------------------------------------------------------------- S/P:0/1 ONU ID:1 Port ID:UNI1 Statistics status:Disable Statistics period:0 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU Port Attribute(Configuration)

| Command | OLT(config-interface-epon-0/0)#show ont port attribute <port-id> <onu-id> eth all |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show ont port attribute |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| all | All the eth port |

【Example】

Example 1：Show all the eth port attribute of ont 3 in pon 3.

OLT(config-interface-epon-0/0)#show ont port attribute 3 3 eth all ----------------------------------------------------------------------------- ONU ONU Auto-neg Speed Duplex Port Flow Mac port(Mbps)switch control Limit ---------------------------------------------------------------------------- 3 1 Enable Auto Auto on off unlimited 3 2 Enable Auto Auto on off unlimited 3 3 Enable Auto Auto on off unlimited 3 4 Enable Auto Auto on off unlimited 3 5 Enable Auto Auto on off unlimited 3 6 Enable Auto Auto on off unlimited 3 7 Enable Auto Auto on off unlimited 3 8 Enable Auto Auto on off unlimited ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU CATV Port Status

| Command | OLT(config-interface-epon-0/0)#show ont port state <port-id> <onu-id> {eth¦catv<onu-portid>} |
| --- | --- |
| View | EPON interface view |
| Description | Ths command is used to show state of ont’s eth port and catv port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <ONU-portid> | Port id of eth or catv |

【Example】

Example 1：Show all eth port state of ont 2 in pon 1.

OLT(config-interface-epon-0/0)#show ont port state 1 2 eth all ----------------------------------------------------------------------------- Port Type Port-switch Link Flow-cONUrol Auto-neg Max-MAC ---------------------------------------------------------------------------- 1 eth on on off enable unlimit 2 eth on off off enable unlimit 3 eth on off off enable unlimit 4 eth on off off enable unlimit ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU Port Learned MAC Address Information

| Command | OLT(config-interface-epon-0/0)#show ont port learned-mac <port-id> <onu-id> eth <eth-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show learned-mac address of ont port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| all | All the eth port |

【Example】

Example 1：Show learned-mac address of ont3’s all eth port in pon 3.

OLT(config-interface-epon-0/0)#show ont port learned-mac 1 5 eth 1 ----------------------------------------------------------------------------- Index MAC ---------------------------------------------------------------------------- 1 F0:DE:F1:62:C5:97 2 74:D0:2B:A1:F1:84 3 E0:67:B3:00:57:3E 4 5C:FF:35:0D:D8:C5 5 EC:6C:9F:05:49:79 6 20:6A:8A:54:6C:7D ----------------------------------------------------------------------------- OLT(config-interface-epon)#

Show ONU PON Port Performance Statistics

| Command | OLT(config-interface-epon-0/0)#show ont statistics <port-id> <onu-id> pon status |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show performance statistics of ont’s pon port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show performance statistics of ont 1’s pon port in pon 1.

OLT(config-interface-epon-0/0)#show ont statistics 1 1 pon status ----------------------------------------------------------------------------- S/P:0/1 ONU ID:1 Port ID:PON1 Statistics status:Disable Statistics period:0 ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

ONU IGMP Function Configuration(Non-template Mode)

ONU IGMP Mode Config

| Command | OLT(config-interface-epon-0/0)#ont multicast-mode <port-id> <onu-id> <mode> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set multicast mode of ont |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <mode> | Ctc:controllable multicast mode transparent igmp-snooping |

【Example】

Example 1：Set the multicast mode of ont 4 in pon 1 as snooping mode.

OLT(config-interface-epon-0/0)#ont multicast-mode 1 4 igmp-snooping OLT(config-interface-epon-0/0)#

ONU Fsast-leave Function Config

| Command | OLT(config-interface-epon-0/0)#ont multicast fast-leave <port-id> <onu-id> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set ont multicast fast-leave. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| {enable¦disable} | Enable:enable multicast fast-leave Disable:disable multicast fast-leave |

【Example】

Example 1：Enable multicast fast-leave in pon 1 onu 4.

OLT(config-interface-epon-0/0)#ont multicast fast-leave 1 4 enable OLT(config-interface-epon-0/0)#

Show ONU Multicast-group Record

| Command | OLT(config-interface-epon-0/0)#show ont multicast-group <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show multicast-group of ont. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show multicast-group of ont 1 in pon 1.

OLT(config-interface-epon-0/0)#show ont multicast-group 1 1 ERROR:There is not any onu group record OLT(config-interface-epon-0/0)#

Show ONU IGMP Mode

| Command | OLT(config-interface-epon-0/0)#show ont multicast-mode <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show multicast-mode of ont. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show multicast-mode of ont 3 in pon 3.

OLT(config-interface-epon-0/0)#show ont multicast-mode 3 3 ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:3 ONU-ID:3 Multicast mode:Transparent ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU Port IGMP VLAN Tagstrip Mode

| Command | OLT(config-interface-epon-0/0)#show ont port attribute <port-id> <onu-id> eth <eth-id> multicast-tagstrip |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show multicast-tagstrip mode of ont eth port |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-id> | Eth port id,range for 1-24 |

【Example】

Example 1：Show multicast-tagstrip mode of ont 3 eth 1 in pon 3.

OLT(config-interface-epon-0/0)#show ont port attribute 3 3 eth 1 multicast-tagstrip ----------------------------------------------------------------------------- Frame/Slot:0/0 Port:3 ONU ID:3 Port ID:1 ---------------------------------------------------------------------------- Service Profile ID:0 Service Profile name:srvprofile_0 ---------------------------------------------------------------------------- Tag strip mode:tag ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

ONU Upgrade Management

Transfer ONU Firmware to OLT

| Command | OLT(config)# load file {ftp <ip-address> <ftp-user-name> < ftp-user-password> < FILE-NAME>} ¦ {tftp <ip-address> < FILE-NAME>} |
| --- | --- |
| View | Config view |
| Description | This command is used to transfer the upgrade file of the ONU to the OLT. |
| ftp | Use the ftp protocol to transfer ONU upgrade files to the OLT. |
| <ip-address> | IP address of ftp or tftp server, format X.X.X.X |
| <ftp-user-name> | User name of the ftp server, ranging from 1 to 32 characters. |
| <ftp-user-password> | Access password of the ftp server, ranging from 1 to 32 characters. |
| < FILE-NAME> | The name of the upgrade file of the ONU, ranging from 1 to 64 characters. The extension of the ONT upgrade file is required. |
| tftp | Transfer the ONT upgrade file to the OLT using the tftp protocol. |

【Example】

Example 1： Use the ftp method to transfer the ONT upgrade file 111.tar to the OLT.

OLT(config)# load file ftp 192.168.5.111 test test 111.tar OLT(config)#

Batch Upgrade ONU

| Command | OLT(config)# ont load select {all ¦ model < MODEL-ID >} |
| --- | --- |
| View | Config view |
| Description | This command is used to configure the OLT to upgrade the ONUs in batches. |
| all | Configure to upgrade all ONUs |
| < MODEL-ID > | Configure batch upgrade of ONUs based on the model id of the ONU. |

【Example】

Example 1： Configure the ONUs in batches according to the model id 0x31303053 of

the ONU.

OLT(config)# ont load select model 0x31303053 Number of ONTs that can be selected: 0, success: 0 OLT(config)#

Start or Stop Batch Upgrade ONU

| Command | OLT(config)# ont load start <FILENAME >commit-mode {auto ¦ manual} OLT(config)# ont load stop |
| --- | --- |
| View | Config view |
| Description | This command is used to start or stop the ONU batch upgrade. |
| load start | Configure to upgrade all ONUs |
| <FILENAME > | Configure the ONU upgrade file name for starting batch upgrade. |
| commit-mode | Select the new upgrade file to take effect mode. When not input, the default is ONU auto take effect in next time reboot. |
| auto | The effective mode of the ONU loading policy is that the graceful reset takes effect. That is, after loading the file to the ONU according to the loading policy, the ONU decides whether to restart immediately according to its own related settings to make the loading take effect (for example, whether there is an emergency call). The ONU waits for up to four hours, and when it is exceeded, it is forced to restart. |
| manaul | After the loading is completed, the ONU is manually restarted to make the loading take effect. |
| load stop | Delete the ONU load data. This parameter is enabled when you want to delete all load tasks and data on the ONU immediately. |

【Example】

Example 1：Start the ONU to use the 111.tar upgrade file for batch upgrade. After the upgrade, it will automatically restart to make the upgrade take effect.

OLT(config)# ont load start 111.tar commit-mode auto OLT(config)

Show ONU Upgrade Configuration Information

| Command | OLT(config)# show ont load info |
| --- | --- |
| View | Config view |
| Description | This command is used to view the configuration information of the ONU upgrade that needs to be upgraded. |

【Example】

Example 1：Check the ONU upgrade information configured on the OLT.

OLT(config)# show ont load info ----------------------------------------------------------------------------- File name : Load state : stop Commit mode : auto ----------------------------------------------------------------------------- OLT(config)#

Show ONU Upgrade Progress

| Command | OLT(config)# show ont load select |
| --- | --- |
| View | Config view |
| Description | This command is used to check the progress of the ONU upgrade. |

【Example】

Example 1：Check the progress of the ONU upgrade under the OLT.

OLT(config)# show ont load select ----------------------------------------------------------------------------- F/S PON ONT ID State Progress ---------------------------------------------------------------------------- 0/0 3 1 waiting 0% ----------------------------------------------------------------------------- Total: 1, waiting: 1, fail: 0, success: 0, loading: 0, cancel: 0 OLT(config)#

Separately Upgrade ONU Under PON Port

| Command | OLT(config)# show ont load select |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to select an ONU that needs to be upgraded separately under the PON port. |
| <port-id> | Specifies the PON port number where the ONU is located. The value is 1-16. |
| <onu-id> | The ONU id to be upgraded, in the range of 1-64. |
| all | Configure all ONU upgrades to the specified PON port. |
| < MODEL-ID > | Configure the ONU for the specified PON port to upgrade the ONU according to the model id. |

【Example】

Example 1：Select the first ONU under the PON1 port to be upgraded.

OLT(config-interface-epon-0/0)# ont load select 1 1 Number of ONUs that can be added: 1, success: 1 OLT(config-interface-epon-0/0)#

Active and Commit ONU New Firmware

| Command | OLT(config-interface-epon-0/0)#ont load commit <port-id> {<onu-id> ¦ all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to activate the image of the ONU. After the ONU is upgraded, the command will be executed. After the ONU restarts, the new mirror software will be run. Otherwise, the ONU will only run the software image before the upgrade. |
| <port-id> | Specifies the PON port number where the ONU is located. The value is 1-16 |
| <onu-id> | The ONU id of the new software to be activated, in the range of 1-64. |
| all | Activate new software for all ONUs under the specified PON port. |

【Example】

Example 1：Activate the software image of the first ONU under the PON1 port.

OLT(config-interface-epon-0/0)# ont load commit 1 1 Number of ONUs that can be commit: 1, success: 1 OLT(config-interface-epon-0/0)#

ONU Alarm Management

ONU Opitcal Bias Current Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm optical <port-id> <onu-id> {bias-high-alarm¦bias-high-warning¦bias-low-alarm¦bias-low-warning} {enable¦disable} <threshold><restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable bias current alarm of specified ont and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| bias-high-alarm bias-high-warning bias-low-alarm bias-low-warning | bias-high-alarm:bias current overtops the highest alarm threshold. bias-high-warning:warning the high bias current bias-low-alarm:alarming the bias current lower than threshold bias-low-warning:warning the low bias current |
| {enable¦disable} | Enable Disable |
| <threshold> | The alarm threshold,range for 0-10000,unit is mA |
| <restore-threshold> | The alarm restored threshold,range for 0-10000,unit is mA |

【Example】

Example 1：Enable the bias current alarm of ont 1 in pon 1,set its alarm threshold as 50,restore-threshold as 45.

OLT(config-interface-epon-0/0)#ont alarm optical 1 1 bias-high-alarm enable 50 45 OLT(config-interface-epon-0/0)#

ONU Optical Voltage Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm optical <port-id> <onu-id> {voltage-high-alarm¦voltage-high-warning¦voltage-low-alarm¦voltage-low-warning{enable¦disable} <threshold><restore-threshold>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable voltage alarm of specified ont and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| voltage-high-alarm voltage-high-warning voltage-low-alarm voltage-low-warning | voltage-high-alarm:voltage overtops the alarm threshold voltage-high-warning:warning the higher voltage voltage-low-alarm:alarming the voltage is lower than threshold voltage-low-warning:warning the lower voltage |
| {enable¦disable} | Enable Disable |
| <threshold> | The alarm threshold,range for 0.00-100.00,unit is V |
| <restore-threshold> | The alarm restored threshold,range for 0.00-100.00,unit is V |

【Example】

Example 1：Enable the voltage alarm of ont 1 in pon 1,set its range of threshold as 1-5V.

OLT(config-interface-epon-0/0)#ont alarm optical 1 1 voltage-high-alarm enable 1 5 OLT(config-interface-epon-0/0)#

ONU Opitcal Power Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm optical <port-id> <onu-id> {rx-power-high-alarm¦rx-power-high-warning¦rx-power-low-alarm¦rx-power-low-warning¦tx-power-high-alarm¦tx-power-high-warning¦tx-power-low-alarm¦tx-power-low-warning} {enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable optical power alarm of specified ont and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| rx-power-high-alarm rx-power-high-warning rx-power-low-alarm rx-power-low-warning tx-power-high-alarm tx-power-high-warning tx-power-low-alarm tx-power-low-warning | rx-power-high-alarm:rx-power overtops the alarm threshold rx-power-high-warning:warning the high rx-power rx-power-low-alarm:rx-power is lower than alarm threshold rx-power-low-warning:warning the low rx-power tx-power-high-alarm:tx-power overtops the alarm threshold tx-power-high-warning:warning the high tx-power tx-power-low-alarm:tx-power is lower than alarm threshold tx-power-low-warning:warning the low tx-power |
| {enable¦disable} | Enable Disable |
| <threshold> | The alarm threshold,range for(-99)-100.00,unit is dbm |
| <restore-threshold> | The alarm restored threshold,range for(-99)-100.00,unit is dbm |

【Example】

Example 1：Enable the rx-power alarm of ont 1 in pon 1,set itsalarm threshold as-3,restore-threshold as-4.

OLT(config-interface-epon-0/0)#ont alarm optical 1 1 rx-power-high-alarm enable-3-4 OLT(config-interface-epon-0/0)#

ONU Opitcal Temperature Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm optical <port-id> <onu-id> {temperature-high-alarm¦temperature-high-warning¦temperature-low-alarm¦temperature-low-warning{enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable temperature alarm of specified ont and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| temperature-high-alarm temperature-high-warning temperature-low-alarm temperature-low-warning | temperature-high-alarm:temperature overtops the alarm threshold temperature-high-warning:warning high temperature but not reaching the alarm threshold temperature-low-alarm:temperature is lower than alarm threshold temperature-low-warning:warning low temperature but not reaching the alarm threshold |
| {enable¦disable} | Enable Disable |
| <threshold> | The alarm threshold,range for(-99)-300,unit is℃ |
| <restore-threshold> | The alarm restored threshold,range for(-99)-300,unit is℃ |

【Example】

Example 1：Enable the temperature alarm of ont 1 in pon 1,set its alarm threshold as 55℃,restore-threshold as 45℃.

OLT(config-interface-epon-0/0)#ont alarm optical 1 1 temperature-high-alarm enable 55 45 OLT(config-interface-epon-0/0)#

ONU Port CRC Checkout Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm port <port-id> <onu-id> {pon¦eth} <eth-port-id> {rx-crcerrors-alarm¦rx-crcerrors-warning¦tx-crcerrors-alarm¦tx-crcerrors-warning} {enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable error message CRC checkout alarm of specified ont port and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| {pon¦eth} | Pon:ont’s pon port Eth:ont’s ethernet port |
| <eth-port-id> | Ont’s ethernet port id,range for 1-24 |
| rx-crcerrors-alarm¦rx-crcerrors-warning¦tx-crcerrors-alarm¦tx-crcerrors-warning | rx-crcerrors-alarm:receive the crc checkout message error alarm rx-crcerrors-warning:receive the crc checkout message error warning tx-crcerrors-alarm:transmit the crc checkout message error alarm tx-crcerrors-warning:transmit the crc checkout message error warning |
| {enable¦disable} | enable disable |
| <threshold> | The alarm threshold,range for 0-4294967294 |
| <restore-threshold> | The alarm restore threshold,range for 0-4294967294 |

【Example】

Example 1：Enable the crc checkout alarm of eth1 in pon 1 ont 1,set its alarm threshold as 234,restore-threshold as 150.

OLT(config-interface-epon-0/0)#ont alarm port 1 1 pon rx-crcerrors-alarm enable 234 150 OLT(config-interface-epon-0/0)#

ONU Port Dropping Message Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm port <port-id> <onu-id> {pon¦eth} <eth-port-id> {rx-discards-alarm¦rx-discards-warning¦tx-discards-alarm¦tx-discards-warning} {enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable dropping message alarm of specified ont port and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| {pon¦eth} | Pon:ont’s pon port Eth:ont’s ethernet port |
| <eth-port-id> | Ont’s ethernet port id,range for 1-24 |
| rx-discards-alarm¦rx-discards-warning¦tx-discards-alarm¦tx-discards-warning | rx-discards-alarm:receive the alarm of discarded message rx-discards-warning:receive the warning of discarded message tx-discards-alarm:transmit the alarm of discarded message tx-discards-warning:transmit the warning of discarded message |
| {enable¦disable} | enable disable |
| <threshold> | The alarm threshold,range for 0-4294967294 |
| <restore-threshold> | The alarm restored threshold,range for 0-4294967294 |

【Example】

Example 1：Enable and receive the alarm of discarded message of eth1 in pon 1 ont 1,set its alarm threshold as 200,restore-threshold as 150.

OLT(config-interface-epon-0/0)#ont alarm port 1 1 eth 1 rx-discards-alarm enable 200 150 OLT(config-interface-epon-0/0)#

ONU Port Dropevents Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm port <port-id> <onu-id> {pon¦eth} <eth-port-id> {rx-dropevents-alarm ¦rx-dropevents-warning¦tx-dropevents-alarm¦tx-dropevents-warning} {enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable droppevents alarm of specified ont port and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| {pon¦eth} | Pon:ont’s pon port Eth:ont’s ethernet port |
| <eth-port-id> | Ont’s ethernet port id,range for 1-24 |
| rx-dropevents-alarm¦rx-dropevents-warning¦tx-dropevents-alarm¦tx-dropevents-warning | rx-dropevents-alarm:receive the alarm of drop event rx-dropevents-warning:receive the warning of drop event tx-dropevents-alarm:transmit the alarm of drop event tx-dropevents-alarm:transmit the warning of drop event |
| {enable¦disable} | enable disable |
| <threshold> | The alarm threshold,range for 0-4294967294 |
| <restore-threshold> | The alarm stored threshold,range for 0-4294967294 |

【Example】

Example 1：Enable and receive the alarm of dropevents of eth1 in pon 1 ont 1,set its alarm threshold as 123,restore-threshold as 100.

OLT(config-interface-epon-0/0)#ont alarm port 1 1 eth 1 rx-dropevents-alarm enable 123 100 OLT(config-interface-epon-0/0)#

ONU Port Error Message Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm port <port-id> <onu-id> {pon¦eth} <eth-port-id> {rx-errors-alarm¦rx-errors-warning¦tx-errors-alarm¦tx-errors-warning} {enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable error message alarm of specified ont port and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| {pon¦eth} | Pon:ont’s pon port Eth:ont’s ethernet port |
| <eth-port-id> | Ont’s ethernet port id,range for 1-24 |
| rx-errors-alarm¦rx-errors-warning¦tx-errors-alarm¦tx-errors-warning | rx-errors-alarm:receive the alarm of error message rx-errors-warning:receive the warning of error message tx-errors-alarm:transmit the alarm of error message tx-errors-alarm:transmit the warning of error message |
| {enable¦disable} | enable disable |
| <threshold> | The alarm threshold,range for 0-4294967294 |
| <restore-threshold> | The alarm restored threshold,range for 0-4294967294 |

【Example】

Example 1：Enable and receive the alarm of error message of eth1 in pon 1 ont 1,set its alarm threshold as 123,restore-threshold as 100.

OLT(config-interface-epon-0/0)#ont alarm port 1 1 eth 1 rx-errors-alarm enable 123 100 OLT(config-interface-epon-0/0)#

ONU Port Message Fragment Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm port <port-id> <onu-id> {pon¦eth} <eth-port-id> {rx-fragments-alarm ¦rx-fragments-warning ¦tx-fragments-alarm¦tx-fragments-warning} {enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable message fragment alarm of specified ont port and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| {pon¦eth} | Pon:ont’s pon port Eth:ont’s ethernet port |
| <eth-port-id> | Ont’s ethernet port id,range for 1-24 |
| rx-fragments-alarm¦rx-fragments-warning¦tx-fragments-alarm¦tx-fragments-warning | rx-fragments-alarm:receive the alarm of message fragment rx-fragments-warning:receive the warning of message fragment tx-fragments-alarm:transmit the alarm of message fragment tx-fragments-alarm:transmit the warning of error message |
| {enable¦disable} | enable disable |
| <threshold> | The alarm threshold,range for 0-4294967294 |
| <restore-threshold> | The alarm restored threshold,range for 0-4294967294 |

【Example】

Example 1：Enable and receive the alarm of message fragment of eth1 in pon 1 ont 1,set its alarm threshold as 123,restore-threshold as 100.

OLT(config-interface-epon-0/0)#ont alarm port 1 1 eth 1 rx-fragments-alarm enable 123 100 OLT(config-interface-epon-0/0)#

ONU Port Message Jabber(time out) Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm port <port-id> <onu-id> {pon¦eth} <eth-port-id> {rx-jabbers-alarm ¦rx-jabbers-warning ¦tx-jabbers-alarm¦tx-jabbers-warning} {enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable message jabber alarm of specified ont port and set its alarm threshold,it means when the message has crc error and it is out of the range,it will alarm. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| {pon¦eth} | Pon:ont’s pon port Eth:ont’s ethernet port |
| <eth-port-id> | Ont’s ethernet port id,range for 1-24 |
| rx-jabbers-alarm¦rx-jabbers-warning¦tx-jabbers-alarm¦tx-jabbers-warning | rx-jabbers-alarm:receive the alarm of message jabber rx-jabbers-warning:receive the warning of message jabber tx-jabbers-alarm:transmit the alarm of message jabber tx-jabbers-alarm:transmit the warning of message jabber |
| {enable¦disable} | enable disable |
| <threshold> | The alarm threshold,range for 0-4294967294 |
| <restore-threshold> | The alarm restored threshold,range for 0-4294967294 |

【Example】

Example 1：Enable and receive the alarm of message jabber of eth1 in pon 1 ont 1,set its alarm threshold as 1518,restore-threshold as 1500.

OLT(config-interface-epon-0/0)#ont alarm port 1 1 eth 1 rx-jabbers-alarm enable 1518 1500 OLT(config-interface-epon-0/0)#

ONU Port Oversize Message Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm port <port-id> <onu-id> {pon¦eth} <eth-port-id> {rx-oversizes-alarm ¦rx-oversizes-warning ¦tx-oversizes-alarm¦tx-oversizes-warning} {enable¦disable} <threshold> <restore-threshold> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable oversize message alarm of specified ont port and set its alarm threshold,it means crc of message is correct but the size of it is over range,it will alarm. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| {pon¦eth} | Pon:ont’s pon port Eth:ont’s ethernet port |
| <eth-port-id> | Ont’s ethernet port id,range for 1-24 |
| rx-oversizes-alarm¦rx-oversizes-warning¦tx-oversizes-alarm¦tx-oversizes-warning | rx-oversizes-alarm:receive the alarm of oversize message rx-oversizes-warning:receive the warning of oversize message tx-oversizes-alarm:transmit the alarm of oversize message tx-oversizes-alarm:transmit the warning of oversize message |
| {enable¦disable} | enable disable |
| <threshold> | The lowest alarm threshold,range for 0-4294967294 |
| <restore-threshold> | The highest alarm threshold,range for 0-4294967294 |

【Example】

Example 1：Enable and receive the alarm of oversize message of eth1 in pon 1 ont 1,set its alarm threshold as 1518,restore-threshold as 1500.

OLT(config-interface-epon-0/0)#ont alarm port 1 1 eth 1 rx-oversizes-alarm enable 1518 1500 OLT(config-interface-epon-0/0)#

ONU Port Ultra Short Message Alarm Function Config

| Command | OLT(config-interface-epon-0/0)#ont alarm port <port-id> <onu-id> {pon¦eth} <eth-port-id> {rx-undersizes-alarm ¦ rx-undersizes-warning¦tx-undersizes-alarm¦tx-undersizes-warning} {enable¦disable} <threshold> <restore-threshold>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable ultra short message alarm of specified ont port and set its alarm threshold |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Onu-id:ont id,range for 1-64 |
| {pon¦eth} | Pon:ont’s pon port Eth:ont’s ethernet port |
| <eth-port-id> | Ont’s ethernet port id,range for 1-24 |
| rx-undersizes-alarm¦rx-undersizes-warning¦tx-undersizes-alarm¦tx-undersizes-warning | rx-undersizes-alarm:receive the alarm of ultra short message rx-undersizes-warning:receive the warning of ultra short message tx-undersizes-alarm:transmit the alarm of ultra short message tx-undersizes-alarm:transmit the warning of ultra short message |
| {enable¦disable} | enable disable |
| <threshold> | The alarm threshold,range for 0-4294967294 |
| <restore-threshold> | The alarm restored threshold,range for 0-4294967294 |

【Example】

Example 1：Enable and receive the alarm of ultra short message of eth1 in pon 1 ont 1,set its alarm threshold as 60,restore-threshold as 61.

OLT(config-interface-epon-0/0)#ont alarm port 1 1 eth 1 rx-undersizes-alarm enable 60 61 OLT(config-interface-epon-0/0)#

Enable or Disable ONU Dying-gasp Alarm Function

| Command | OLT(config-interface-epon-0/0)#ont alarm dying-gasp <port-id> {enable¦disable} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to enable or disable dying-gasp alarm of specified ont |
| <port-id> | Pon port id,range for 1-16 |
| {enable¦disable} | Enable:enable dying-gasp alarm of onu Disable:disable dying-gasp alarm of onu |

【Example】

Example 1：Enable dying-gasp alarm of onu 1 in pon 1.

OLT(config-interface-epon-0/0)#ont alarm dying-gasp 1 enable OLT(config-interface-epon-0/0)#

Bind Alarm-config to ONU

| Command | OLT(config-interface-epon-0/0)#ont alarm-config <port-id> {all¦<onu-id>} {profile-id <profile-id>¦profile-name<profile-name>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to bind the alarm-config to ont.but first of all,the alarm-profile should be created in config view. |
| <port-id> | Pon port id,range for 1-16 |
| all¦<onu-id> | All:all of the ont onu-id:Onu-id:ont id,range for 1-64 |
| <profile-id> | Profile id,range for 1-50 |
| <profile-name> | Profile name,it supports 1-16 strings |

【Example】

Example 1：Bind alarm-profile 5 to ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont alarm-config 1 1 profile-id 5 OLT(config-interface-epon-0/0)#

Bind Optical-alarm-config to ONU

| Command | OLT(config-interface-epon-0/0)#ont optical-alarm-config <port-id> {all¦<onu-id>} {profile-id <profile-id>¦profile-name<profile-name>} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to bind the optical-alarm-config to ont.but first of all,the alarm-profile should be created in config view. |
| <port-id> | Pon port id,range for 1-16 |
| all¦<onu-id> | All:all of the ont onu-id:Onu-id:ont id,range for 1-64 |
| <profile-id> | Profile id,range for 1-50 |
| <profile-name> | Profile name,it supports 1-24 strings |

【Example】

Example 1：Bind optical-alarm-profile 5 to ont 1 in pon 1.

OLT(config-interface-epon-0/0)#ont optical-alarm-profile 1 1 profile-id 5 OLT(config-interface-epon-0/0)#

Show ONU Dying-gasp Alarm Config Status

| Command | OLT(config-interface-epon-0/0)#show ont alarm dying-gasp {<port-id> ¦all} |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show configured state of ont dying-gasp alarm |
| {<port-id>¦all} | port-id:Pon port id,range for 1-16 All:all the pon ports |

【Example】

Example 1：Show ont alarm dying-gasp of all the pon port

OLT(config-interface-epon-0/0)#show ont alarm dying-gasp all ----------------------------------------------------------------------------- F/S P State ---------------------------------------------------------------------------- 0/0 1 enable 0/0 2 enable 0/0 3 enable 0/0 4 enable 0/0 5 enable 0/0 6 enable 0/0 7 enable 0/0 8 enable 0/0 9 enable 0/0 10 enable 0/0 11 enable 0/0 12 enable 0/0 13 enable 0/0 14 enable 0/0 15 enable 0/0 16 enable ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU Optical power Alarm Config Status

| Command | OLT(config-interface-epon-0/0)#show ont alarm optical <port-id> <onu-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show the configured state of optical alarm and temperature alarm and voltage alarm and etc. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show the configured state of optical alarm in pon 1 ont 1.

OLT(config-interface-epon-0/0)#show ont alarm optical 1 1 ----------------------------------------------------------------------------- Rx optical power high alarm:disable Rx optical power low alarm:disable Rx optical power high warning:disable Rx optical power low warning:disable Tx optical power high alarm:disable Tx optical power low alarm:disable Tx optical power high warning:disable Tx optical power low warning:disable Bias current high alarm:disable Bias current low alarm:disable Bias current high warning:disable Bias current low warning:disable Supply voltage high alarm:disable Supply voltage low alarm:disable Supply voltage high warning:disable Supply voltage low warning:disable Temperature high alarm:disable Temperature low alarm:disable Temperature high warning:disable Temperature low warning:disable ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU ETH Port Alarm Config Status

| Command | OLT(config-interface-epon-0/0)#show ont alarm port <port-id> <onu-id> eth <eth-port-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show configured state of ont eth port alarm |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | Eth port id,range for 1-24 |

【Example】

Example 1：Show eth port alarm’s configured state of ont 1 eth 1 in pon 1.

OLT(config-interface-epon-0/0)#show ont alarm port 1 1 eth 1 ----------------------------------------------------------------------------- F/S P ONU-ID ONU-Port Alarm/Warning State Threshold Restore-Threshold ---------------------------------------------------------------------------- 0/0 1 1 1 tx-dropevents-alarm disable-- 0/0 1 1 1 rx-dropevents-alarm disable-- 0/0 1 1 1 tx-crcerrors-alarm disable-- 0/0 1 1 1 rx-crcerrors-alarm disable-- 0/0 1 1 1 tx-undersizes-alarm disable-- 0/0 1 1 1 rx-undersizes-alarm disable-- 0/0 1 1 1 tx-oversizes-alarm disable-- 0/0 1 1 1 rx-oversizes-alarm disable-- 0/0 1 1 1 tx-fragments-alarm disable-- 0/0 1 1 1 rx-fragments-alarm disable-- 0/0 1 1 1 tx-jabbers-alarm disable-- 0/0 1 1 1 rx-jabbers-alarm disable-- 0/0 1 1 1 tx-discards-alarm disable-- 0/0 1 1 1 rx-discards-alarm disable-- 0/0 1 1 1 tx-errors-alarm disable-- 0/0 1 1 1 rx-errors-alarm disable-- 0/0 1 1 1 tx-dropevents-warning disable-- 0/0 1 1 1 rx-dropevents-warning disable-- 0/0 1 1 1 tx-crcerrors-warning disable-- 0/0 1 1 1 rx-crcerrors-warning disable-- 0/0 1 1 1 tx-undersizes-warning disable-- 0/0 1 1 1 rx-undersizes-warning disable-- 0/0 1 1 1 tx-oversizes-warning disable-- 0/0 1 1 1 rx-oversizes-warning disable-- 0/0 1 1 1 tx-fragments-warning disable-- 0/0 1 1 1 rx-fragments-warning disable-- 0/0 1 1 1 tx-jabbers-warning disable-- 0/0 1 1 1 rx-jabbers-warning disable-- 0/0 1 1 1 tx-discards-warning disable-- 0/0 1 1 1 rx-discards-warning disable-- 0/0 1 1 1 tx-errors-warning disable-- 0/0 1 1 1 rx-errors-warning disable-- ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

Show ONU PON Port Alarm Config Status

| Command | OLT(config-interface-epon-0/0)#show ont alarm port <port-id> <onu-id> pon |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to show configured state of ont pon port alarm |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |

【Example】

Example 1：Show configured state of ont 1’s pon port alarm in pon 1.

OLT(config-interface-epon-0/0)#show ont alarm port 1 1 pon ----------------------------------------------------------------------------- F/S P ONU-ID Alarm/Warning State Threshold Restore-Threshold ---------------------------------------------------------------------------- 0/0 1 1 rx-dropevents-alarm disable-- 0/0 1 1 tx-dropevents-alarm disable-- 0/0 1 1 rx-crcerrors-alarm disable-- 0/0 1 1 tx-crcerrors-alarm disable-- 0/0 1 1 rx-undersizes-alarm disable-- 0/0 1 1 tx-undersizes-alarm disable-- 0/0 1 1 rx-oversizes-alarm disable-- 0/0 1 1 tx-oversizes-alarm disable-- 0/0 1 1 rx-fragments-alarm disable-- 0/0 1 1 tx-fragments-alarm disable-- 0/0 1 1 rx-jabbers-alarm disable-- 0/0 1 1 tx-jabbers-alarm disable-- 0/0 1 1 rx-discards-alarm disable-- 0/0 1 1 tx-discards-alarm disable-- 0/0 1 1 rx-errors-alarm disable-- 0/0 1 1 tx-errors-alarm disable-- 0/0 1 1 rx-dropevents-warning disable-- 0/0 1 1 tx-dropevents-warning disable-- 0/0 1 1 rx-crcerrors-warning disable-- 0/0 1 1 tx-crcerrors-warning disable-- 0/0 1 1 rx-undersizes-warning disable-- 0/0 1 1 tx-undersizes-warning disable-- 0/0 1 1 rx-ossversizes-warning disable-- 0/0 1 1 tx-oversizes-warning disable-- 0/0 1 1 rx-fragments-warning disable-- 0/0 1 1 tx-fragments-warning disable-- 0/0 1 1 rx-jabbers-warning disable-- 0/0 1 1 tx-jabbers-warning disable-- 0/0 1 1 rx-discards-warning disable-- 0/0 1 1 tx-discards-warning disable-- 0/0 1 1 rx-errors-warning disable-- 0/0 1 1 tx-errors-warning disable-- ----------------------------------------------------------------------------- OLT(config-interface-epon-0/0)#

ONU Port VLAN Pool Function Config

| Command | OLT(config-interface-epon-0/0)#ont port vlan <port-id> <onu-id> eth <eth-port-id> vlan-pool <vlan-pool-id> |
| --- | --- |
| View | EPON interface view |
| Description | This command is used to set vlan-pool of ont port.it needs to create a vlan-pool in config view firstly. |
| <port-id> | Pon port id,range for 1-16 |
| <onu-id> | Ont id,range for 1-64 |
| <eth-port-id> | eth port id,range for 1-24 |
| <vlan-pool-id> | Vlan-pool id,range for 1-128 |

【Example】

Example 1：Bind vlan-pool 2 to ont 1 eth 1 in pon 1.

OLT(config-interface-epon-0/0)#ont port vlan 1 1 eth 1 vlan-pool 2 OLT(config-interface-epon-0/0)#

OLT Log Management and Query

Add Syslog Record Server

| Command | OLT(config)#loghost add <ip-address> <hostname> |
| --- | --- |
| View | Config view |
| Description | This command is used to add log server.Device will generate lots of log info when it is in running,but the storage space of device is limited,when it needs to set log server to collect the log info,using this command.After successful adding the log server,some of important log info of device will be recorded in this host by Syslog mechanism |
| <ip-address> | IP address of syslog server |
| <Hostname> | Name of syslog server.It used to distinct with other syslog server and uniquely identify the syslog server. |

【Example】

Example 1：Add syslog server,its ip is 192.168.1.223,server name is log.

OLT(config)#loghost add 192.168.1.223 log OLT(config)#

Delete Syslog Record Server

| Command | OLT(config)#loghost delete ip <ip-address> name <hostname> |
| --- | --- |
| View | Config view |
| Description | This command is used to delete syslog server.when the syslog server is unnecessary or its ip address has been changed,this command can delete the log server.after that,we can add new log server or reset the old log server’s ip. |
| <ip-address> | IP address of syslog server |
| <Hostname> | Name of syslog server.It used to distinct with other syslog server and uniquely identify the syslog server. |

【Example】

Example 1：Delete the syslog server,its ip is 192.168.2.245,server name is test.

OLT(config)#loghost delete ip 192.168.2.245 name test Delete syslog host successed! OLT(config)#

Enable or Disable Syslog Record to Server

| Command | OLT(config)#loghost operlog {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to set the switch of whether the olt log will transmit to log server. |
| {enable¦disable} | Enable:olt log will transmit to log server Disable:olt log will not transmit to log server |

【Example】

Example 1：olt log won’t transmit to log server

OLT(config)#loghost operlog disable OLT(config)#

Enable or Disable Alarmlog Record to Server

| Command | OLT(config)#loghost alarmlog {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to set the switch of whether the alarmlog of olt will transmit to log server. |
| {enable¦disable} | Enable:alarmlog of olt will transmit to log server. Disable:alarmlog of olt won’t transmit to log server. |

【Example】

Example 1：Set alarmlog of olt won’t transmit to log server.

OLT(config)#loghost alarmlog disable OLT(config)#

Active Syslog Record server

| Command | OLT(config)#loghost activate ip<ip-address> name <hostname> |
| --- | --- |
| View | Config view |
| Description | This command is used to active the host of log server.When setting the control level of log host info output or it needs to active the switch of log output,using this command.System will report the log to the corresponding host only after the log host is active successfully. |
| <ip-address> | IP address of syslog server |
| <Hostname> | Name of syslog server.It used to distinct with other syslog server and uniquely identify the syslog server. |

【Example】

Example 1：Active the syslog server,its ip is 192.168.2.223,server name is loghost.

OLT(config)#loghost activate ip 192.168.2.223 name loghost Activate syslog host successed! OLT(config)#

Deactive Syslog Record Server

| Command | OLT(config)#loghost deactivate ip <ip-address> name <hostname> |
| --- | --- |
| View | Config view |
| Description | This command is used to deactive the host of log server.When an active log host is standing off and it needs to change the state of this log host,using this command.After setting,system won’t report the log info to the log host. |
| <ip-address> | IP address of syslog server |
| <Hostname> | Name of syslog server.It used to distinct with other syslog server and uniquely identify the syslog server. |

【Example】

Example 1：Deactive the syslog server,its ip is 192.168.2.223,server name is loghost.

OLT(config)#loghost deactivate ip 192.168.2.223 name loghost Deactivate syslog host successed! OLT(config)#

Show Syslog Record Server Config Status

| Command | OLT(config)#show loghost list |
| --- | --- |
| View | Config view |
| Description | This command is used to show the configuration info of loghost.including host ip address,host name,host state and etc. |

【Example】

Example 1：Show the configuration info of loghost

OLT(config)#show loghost list ---------------------------------------------------------------------------- IP address Host name Terminal state 192.168.2.223 loghost active ---------------------------------------------------------------------------- OLT(config)#

Backup OLT Log

| Command | OLT(config)#backup log ftp <server-ip-address> <user-name> <user-password> <filename> |
| --- | --- |
| View | Enable view,config view |
| Description | This command is used to save the log to ftp server by manually. |
| <server-ip-address> | IP address of ftp server |
| <user-name> | User name of ftp server |
| <user-password> | Password of ftp server |
| <filename> | The name of the backup log |

【Example】

Example 1：Save the log to ftp server 192.168.1.223,user name is admin,password is admin,file name is log.

OLT(config)#backup log ftp 192.168.1.223 admin admin logback Start backup log files The backup is successful

Erase OLT Log

| Command | OLT(config)#erase log |
| --- | --- |
| View | Config view |
| Description | This command is used to delete the log of olt |

【Example】

Example 1：Erase the log of olt.

OLT(config)#erase log OLT(config)#

22.10 Show OLT Log

| Command | OLT(config)#show log |
| --- | --- |
| View | enable view,config view |
| Description | Show all the log of olt |

【Example】

Example 1：Show all the log of olt

OLT(config)#show log 2000/01/03 11:39:16[root@Console:13]logoff 2000/01/03 11:44:05[root@192.168.5.70:43]logoff 2000/01/03 12:33:20[root@192.168.5.70:43]logon via Telnet successfully 2000/01/03 12:33:21[192.168.5.70@root]cmd:enable 2000/01/03 12:33:22[192.168.5.70@root]cmd:config 2000/01/03 12:33:31[192.168.5.70@root]cmd:interface link-aggregation 2000/01/03 12:38:54[root@192.168.5.70:43]logoff OLT(config)#

OLT Alarm Management and Query

Clear Specified Active Alarm Entry

| Command | OLT(config)#alarm active clear <alarm-raising-number> |
| --- | --- |
| View | Config view |
| Description | This command is used to clear the specified active alarm entry |
| <alarm-raising-number> | Clear the active alarm according to alarm-raising-number.range for 1-4294967295.alarm-raising-number needs to use the follow command to show its detail info: alarm output detail on show alarm active all |

【Example】

Example 1：Clear active alarm 2.

OLT(config)#alarm active clear 2 OLT(config)#

Clear Active Alarm by Specified Alarmlevel

| Command | OLT(config)#alarm active clear alarmlevel<1-4> |
| --- | --- |
| View | Config view |
| Description | This command is used to clear active alarm by specified alarmlevel |
| <1-4> | - Critical - major - minor - warning |

【Example】

Example 1：Clear the warning active alarm

OLT(config)#alarm active clear alarmlevel 4 OLT(config)#

Clear Active Alarm by Specified Board

| Command | OLT(config)#alarm active clear alarmparameter board <F/S> |
| --- | --- |
| View | Config view |
| Description | This command is used to clear active alarm by specified board. |
| <F/S> | Frame/slot:the default is 0/0 |

【Example】

Example 1：Clear active alarm in 0/0.

OLT(config)#alarm active clear alarmparameter board 0/0 OLT(config)#

Clear Active Alarm by Specified GE Port

| Command | OLT(config)#alarm active clear alarmparameter ge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to clear active alarm by specified GE port |
| <F/S/P> | Frame/slot/ge port id,range for 0/0/1–0/0/8 |

【Example】

Example 1：Clear the active alarm of ge1.

OLT(config)#alarm active clear alarmparameter ge 0/0/1 OLT(config)#

Clear Active Alarm by Specified PON Port

| Command | OLT(config)#alarm active clear alarmparameter pon <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to clear active alarm by specified pon port |
| <F/S/P> | Frame/slot/pon port id,range for 0/0/1–0/0/16 |

【Example】

Example 1：Clear the active alarm of pon 1.

OLT(config)#alarm active clear alarmparameter pon 0/0/1 OLT(config)#

Clear Active Alarm by Specified XGE Port

| Command | OLT(config)#alarm active clear alarmparameter xge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to clear active alarm by specified XGE port |
| <F/S/P> | Frame/slot/xge port id,range for 0/0/1–0/0/2 |

【Example】

Example 1：Clear the active alarm of xge 1.

OLT(config)#alarm active clear alarmparameter xge 0/0/1 OLT(config)#

Config Alarmlevel for Specified Alarm

| Command | OLT(config)#alarm alarmlevel<alarm-id> <1-4> |
| --- | --- |
| View | Config view |
| Description | This command is used to set alarmlevel for specified alarm |
| <alarm-id> | alarm-raising-number.range for 1-4294967295 |
| <0-4> | - Default - critical - major - minor - warning |

【Example】

Example 1：Set the alarmlevel of 102th alarm as 1(critical).

OLT(config)#alarm alarmlevel 102 1 OLT(config)#

Enable and Config Alarm Jitter-interval Time

| Command | OLT(config)#alarm jitter-interval<interval> |
| --- | --- |
| View | Config view |
| Description | This command is used to enable and set alarm jitter-interval.When this command is executed,alarm of the system will wait for a jitter-interval and then report it to network management,if the alarm state has recover during a jitter-interval,this alarm won’t be reported to network management. |
| <interval> | alarm interval,range for 1-60,unit is second. |

【Example】

Example 1：Set alarm jitter-interval as 3s.

OLT(config)#alarm jitter-interval 3 OLT(config)#

Disable Alarm Jitter-proof Function

| Command | OLT(config)#alarm jitter-proof disable |
| --- | --- |
| View | Config view |
| Description | This command is used to turn off alarm jitter-proof function. |

【Example】

Example 1：Turn off alarm jitter-proof function

OLT(config)#alarm jitter-proof disable OLT(config)#

Enable or Disable Specified Alarm Record Output

| Command | OLT(config)#alarm output alarmid <alarm-id> {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is enable and disable the specified alarm record output.When state is“enable”,permitting the specified alarm record reporting to EMS.When state is“disable”,denying the specified alarm record reporting to EMS. |
| <alarm-id> | alarm ID,the value range is 1-4294967294. |
| {enable¦disable} | Enable:turn on function Disable：turn off function |

【Example】

Example 1：Disable the 102 alarm record output.

OLT(config)#alarm output alarmid 102 disable OLT(config)#

Enable or Disable Specified Level Alarm Output

| Command | OLT(config)#alarm output alarmlevel <alarmlevel> {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is enable or disable the specified level alarm record output.When state is“enable”,permitting the specified level alarm record reporting to EMS.When state is“disable”,denying the specified level alarm record reporting to EMS. |
| <alarm-level> | - Critical - Major - Minor - Warning |
| {enable¦disable} | Enable：turn on function Disable：turn off function |

【Example】

Example 1：Disable level 4 alarm output.

OLT(config)#alarm output alarmlevel 4 disable OLT(config)#

Enable or Disable All Alarms Output

| Command | OLT(config)#alarm output all {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is enable or disable all alarm output.When state is“enable”,permitting all alarms reporting to EMS.When state is“disable”,denying all alarms reporting to EMS. |
| {enable¦disable} | Enable：turn on function Disable：turn off function |

【Example】

Example 1：Disable all alarms output.

OLT(config)#alarm output all disable OLT(config)#

Enable or Disable Detail Alarm Output

| Command | OLT(config)#alarm output detail {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to turn on or off alarm detail input function.When state is“on”,outputting detail alarm information.When state is“off”,outputting simple alarm information. |
| {enable¦disable} | on：enable off：disable |

【Example】

Example 1：Turn off alarm output detail information function.

OLT(config)#alarm output detail disable OLT(config)#

Show Specified Active Alarm Record

| Command | OLT(config)#show alarm active alarmid <alarm-id> |
| --- | --- |
| View | Config view |
| Description | This command is use to view the specified active alarm record. |
| <alarm-id> | Alarm-ID,the value range is 1-4294967294. |

【Example】

Example 1：View active alarm record of alarm-id 204.

OLT(config)#show alarm active alarmid 204 ALARM 15 Major 204 2000-01-02 02:22:51 ALARM NAME:pon port link down INSTANCE:PON FrameID:0,SlotID:0,PortID:3 REPEAT TIME:1 FIRST OCCUR:2000-01-02 02:22:51 LAST OCCUR:2000-01-02 02:22:51 DESCRIPTION: total number:1 OLT(config)#

Show Active Alarm Logs by Specified Level

| Command | OLT(config)#show alarm active alarmlevel <1-4> |
| --- | --- |
| View | Config view |
| Description | This command is used to view active alarm logs of the specified level. |
| <1-4> | - Critical - major - minor - warning |

【Example】

Example 1：View active alarm logs of level 2.

OLT(config)#show alarm active alarmlevel 2 ALARM 15 Major 204 2000-01-02 02:22:51 ALARM NAME:pon port link down INSTANCE:PON FrameID:0,SlotID:0,PortID:3 REPEAT TIME:1 FIRST OCCUR:2000-01-02 02:22:51 LAST OCCUR:2000-01-02 02:22:51 DESCRIPTION: total number:1 OLT(config)#

Show Active Alarm Logs by Specified Board

| Command | OLT(config)#show alarm active alarmparameter board <F/S> |
| --- | --- |
| View | Config view |
| Description | This command is used to view active alarm logs of the specified board. |
| <F/S> | Card slot number,the value is 0/0. |

【Example】

Example 1：View active alarm logs of board 0/0.

OLT(config)#show alarm active alarmparameter board 0/0 ALARM 15 Major 204 2000-01-02 02:22:51 ALARM NAME:pon port link down INSTANCE:PON FrameID:0,SlotID:0,PortID:3 REPEAT TIME:1 FIRST OCCUR:2000-01-02 02:22:51 LAST OCCUR:2000-01-02 02:22:51 DESCRIPTION: total number:1 OLT(config)#

Show Active Alarm Logs by Specified GE Port

| Command | OLT(config)#show alarm active alarmparameter ge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is use to view active alarm logs of the specified GE port. |
| <F/S/P> | GE port number,the value range is 0/0/1–0/0/8. |

【Example】

Example 1：View active alarm logs of ge8 port.

OLT(config)#show alarm active alarmparameter ge 0/0/8 ALARM 18 Critical 203 2000-01-02 02:48:48 ALARM NAME:sni port link down INSTANCE:GE FrameID:0,SlotID:0,PortID:8 REPEAT TIME:1 FIRST OCCUR:2000-01-02 02:48:48 LAST OCCUR:2000-01-02 02:48:48 DESCRIPTION: total number:1 OLT(config)#

Show Active Alarm Logs by Specified PON Port

| Command | OLT(config)#show alarm active alarmparameter pon <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to view active alarm logs of the specified PON port. |
| <F/S/P> | Pon port number,the value range is 0/0/1–0/0/16. |

【Example】

Example 1：View active alarm logs of pon 3 port

OLT(config)#show alarm active alarmparameter pon 0/0/3 ALARM 15 Major 204 2000-01-02 02:22:51 ALARM NAME:pon port link down INSTANCE:PON FrameID:0,SlotID:0,PortID:3 REPEAT TIME:1 FIRST OCCUR:2000-01-02 02:22:51 LAST OCCUR:2000-01-02 02:22:51 DESCRIPTION: total number:1 OLT(config)#

Show Active Alarm Logs by Specified XGE Port

| Command | OLT(config)#show alarm active alarmparameter xge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to view active alarm logs of the specified XGE port. |
| <F/S/P> | Card slot number,the value range is 0/0/1–0/0/2. |

【Example】

Example 1：View active alarm logs of xge1 port.

OLT(config)#show alarm active alarmparameter xge 0/0/1 ALARM 26 Critical 131082 2000-01-02 04:18:33 ALARM NAME:The sni port is unpluged INSTANCE:XGE FrameID:0,SlotID:0,PortID:1 REPEAT TIME:1 FIRST OCCUR:2000-01-02 04:18:33 LAST OCCUR:2000-01-02 04:18:33 DESCRIPTION: total number:1 OLT(config)#

Show All Active Alarm Log

| Command | OLT(config)#show alarm active all |
| --- | --- |
| View | Config view |
| Description | This command is used to view all alarm active logs. |

【Example】

Example 1：View all alarm active logs.

OLT(config)#show alarm active all ALARM 28 Critical 131082 2000-01-02 04:18:53 ALARM NAME:The sni port is unpluged INSTANCE:XGE FrameID:0,SlotID:0,PortID:2 DESCRIPTION: ALARM 26 Critical 131082 2000-01-02 04:18:33 ALARM NAME:The sni port is unpluged INSTANCE:XGE FrameID:0,SlotID:0,PortID:1 DESCRIPTION: ALARM 23 Critical 203 2000-01-02 04:17:40 ALARM NAME:sni port link down INSTANCE:GE FrameID:0,SlotID:0,PortID:5 DESCRIPTION: ALARM 18 Critical 203 2000-01-02 02:48:48 ALARM NAME:sni port link down INSTANCE:GE FrameID:0,SlotID:0,PortID:8 DESCRIPTION: ALARM 15 Major 204 2000-01-02 02:22:51 ALARM NAME:pon port link down INSTANCE:PON FrameID:0,SlotID:0,PortID:3 DESCRIPTION: total number:5 OLT(config)#

Show Specified Alarm History Record

| Command | OLT(config)#show alarm history alarmid<alarm-id> |
| --- | --- |
| View | Config view |
| Description | This command is used to view the specified alarm history records |
| <alarm-id> | Alarm ID,the value range is 1-4294967294. |

【Example】

Example 1：View alarm history records of alarm-ID 204.

OLT(config)#show alarm history alarmid 204 ALARM 35 Cleared 204 2000-01-02 07:05:07 ALARM NAME:pon port link up INSTANCE:PON FrameID:0,SlotID:0,PortID:1 DESCRIPTION: ALARM 34 Major 204 2000-01-02 07:02:33 ALARM NAME:pon port link down INSTANCE:PON FrameID:0,SlotID:0,PortID:1 DESCRIPTION: ALARM 32 Cleared 204 2000-01-02 07:01:39 ALARM NAME:pon port link up INSTANCE:PON FrameID:0,SlotID:0,PortID:1 DESCRIPTION:

Show Alarm History Record by Specified Level

| Command | OLT(config)#show alarm history alarmlevel<1-4> |
| --- | --- |
| View | Config view |
| Description | This command is used to view alarm history records of the specified level. |
| <1-4> | - Critical - major - minor - warning |

【Example】

Example 1：View alarm history records of level 4.

OLT(config)#show alarm history alarmlevel 4 ALARM 38 Cleared 401 2000-01-02 07:07:06 ALARM NAME:uni link up INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4,SlotID:0,Uni:1 DESCRIPTION: ALARM 37 Warning 401 2000-01-02 07:06:57 ALARM NAME:uni link down INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4,SlotID:0,Uni:1 DESCRIPTION: ALARM 30 Cleared 401 2000-01-02 05:57:49 ALARM NAME:uni link up INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:3,SlotID:0,Uni:1 DESCRIPTION:

Show Alarm History Record by Specified Board

| Command | OLT(config)#show alarm history alarmparameter board <F/S> |
| --- | --- |
| View | Config view |
| Description | This command is used to view alarm history records of the specified board. |
| <F/S> | Card slot number,the value is 0/0. |

【Example】

Example 1：View alarm history records of board 0/0.

OLT(config)#show alarm history alarmparameter board 0/0 ALARM 38 Cleared 401 2000-01-02 07:07:06 ALARM NAME:uni link up INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4,SlotID:0,Uni:1 DESCRIPTION: ALARM 37 Warning 401 2000-01-02 07:06:57 ALARM NAME:uni link down INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4,SlotID:0,Uni:1 DESCRIPTION: ALARM 36 Critical 403 2000-01-02 07:05:47 ALARM NAME:Onu ethernet port autoNegotiation failure INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:5,SlotID:0,Uni:1 DESCRIPTION:

Show Alarm History Record by Specified GE Port

| Command | OLT(config)#show alarm history alarmparameter ge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to view alarm history records of the specified GE port |
| <F/S/P> | GE port number,the value range is 0/0/1–0/0/8. |

【Example】

Example 1：View alarm history records of ge8 port.

OLT(config)#show alarm history alarmparameter ge 0/0/8 ALARM 18 Critical 203 2000-01-02 02:48:48 ALARM NAME:sni port link down INSTANCE:GE FrameID:0,SlotID:0,PortID:8 DESCRIPTION: ALARM 17 Cleared 203 2000-01-02 02:48:05 ALARM NAME:sni port link up INSTANCE:GE FrameID:0,SlotID:0,PortID:8 DESCRIPTION:

Show Alarm History Record by Specified PON Port

| Command | OLT(config)#show alarm history alarmparameter pon <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to view alarm history records of the specified PON port. |
| <F/S/P> | Pon port number,the value range is 0/0/1–0/0/16. |

【Example】

Example 1：View alarm history records of pon1 port.

OLT(config)#show alarm history alarmparameter pon 0/0/1 ALARM 40 Cleared 401 2000-01-02 23:42:34 ALARM NAME:uni link up INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4,SlotID:0,Uni:1 DESCRIPTION: ALARM 39 Warning 401 2000-01-02 23:42:31 ALARM NAME:uni link down INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4,SlotID:0,Uni:1 DESCRIPTION:

Show Alarm History Record by Specified XGE Port

| Command | OLT(config)#show alarm history alarmparameter xge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to view alarm history records of the specified XGE port. |
| <F/S/P> | Card slot number,the value range is 0/0/1–0/0/2. |

【Example】

Example 1：View alarm history records of xge1 port.

OLT(config)#show alarm history alarmparameter xge 0/0/1 ALARM 26 Critical 131082 2000-01-02 04:18:33 ALARM NAME:The sni port is unpluged INSTANCE:XGE FrameID:0,SlotID:0,PortID:1 DESCRIPTION: ALARM 25 Cleared 131082 2000-01-02 04:18:15 ALARM NAME:The sni port is pluged INSTANCE:XGE FrameID:0,SlotID:0,PortID:1 DESCRIPTION:

Show All Alarm History Record

| Command | OLT(config)#show alarm history all |
| --- | --- |
| View | Config view |
| Description | This command is use to view all alarm history records. |

【Example】

Example 1：View all alarm history records.

OLT(config)#show alarm history all ALARM 40 Cleared 401 2000-01-02 23:42:34 ALARM NAME:uni link up INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4,SlotID:0,Uni:1 DESCRIPTION: ALARM 39 Warning 401 2000-01-02 23:42:31 ALARM NAME:uni link down INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4,SlotID:0,Uni:1 DESCRIPTION:

Show Alarm Jitter-proof Interval Time

| Command | OLT(config)#show alarm jitter |
| --- | --- |
| View | Config view |
| Description | This command is used to view alarm jitter-proof interval. |

【Example】

Example 1：View OLT’s alarm jitter-proof interval

OLT(config)#show alarm jitter Jitter-Interval:5s OLT(config)#

Show Alarm Basic Information

| Command | OLT(config)#show alarm list |
| --- | --- |
| View | Config view |
| Description | This command is used to view alarm basic information. |

【Example】

Example 1：View OLT’s alarm basic information.

OLT(config)#show alarm list ---------------------------------------------------------------------------- AlarmId Output Level Def Level Name 102 Yes Major Major The board reset 104 Yes Warning Warning The temperature is abnormal 105 Yes Major Major The fan is abnormal 107 Yes Major Major The device power fault 201 Yes Critical Critical Pon port loopback link 202 Yes Major Major Pon los alarm raise 203 Yes Critical Critical sni port link down 204 Yes Major Major pon port link down 205 Yes Warning Warning The number of register llid is exceeded 206 Yes Major Major long luminescence 301 Yes Major Major onu critical event 302 Yes Warning Warning onu exchange key fails 303 Yes Critical Critical onu oam timeout 304 Yes Major Major onu mac auth fails 305 Yes Minor Minor the RX received power of the epon optical port is lower than the lower threshold 306 Yes Minor Minor the RX received power of the epon optical port is higher than the higher threshold 307 Yes Minor Minor the TX output power of the epon optical port is lower than the lower threshold 308 Yes Minor Minor the TX output power of the epon optical port is higher than the higher threshold 310 Yes Warning Warning onu power down 311 Yes Minor Minor the downstream BER is higher than threshold 312 Yes Minor Minor the downstream FER is higher than threshold 313 Yes Minor Minor the upstream BER is higher than threshold 314 Yes Minor Minor the upstream FER is higher than threshold 315 Yes Major Major The performance statistics upper crossed 316 Yes Major Major The performance statistics lower crossed 317 Yes Minor Minor the temperature of the optical module is higher than the higher threshold 318 Yes Minor Minor the temperature of the optical module is lower than the lower threshold 319 Yes Minor Minor the voltage of the optical module is higher than the higher threshold 320 Yes Minor Minor the voltage of the optical module is lower than the lower threshold 321 Yes Warning Warning onu optical down 401 Yes Warning Warning uni link down 402 Yes Minor Minor loopback of onu port is detected 403 Yes Critical Critical Onu ethernet port autoNegotiation failure 131082 Yes Critical Critical The sni port is unpluged 131083 Yes Critical Critical Sni port loopback link ---------------------------------------------------------------------------- OLT(config)#

OLT Event Management and Query

Config Event Level

| Command | OLT(config)#event eventlevel <event-id> <0-4> |
| --- | --- |
| View | Config view |
| Description | This command is used to configure the specified event level. |
| <event-id> | Event ID,only on behalf of one event alarm |
| <0-4> | - Default - critical - major - Minor - Warning |

【Example】

Example 1：Configure event level 3 for event ID 10001

OLT(config)#event eventlevel 10001 3 OLT(config)#

Enable or Disable All Events Output

| Command | OLT(config)#event output all {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to configure all event output in the CLI.When state is“enable”,all events can output in the CLI,or it can’t be |
| {enable¦disable} | Enable:turn on function Disable:turn off function |

【Example】

Example 1：Turn off all events output function.

OLT(config)#event output all disable OLT(config)#

Enable or Disable Detail Event Output

| Command | OLT(config)#event output detail {on¦off} |
| --- | --- |
| View | Config view |
| Description | This command is used to set the switch of event output detail function.When state is“on”,events can all output detail in the CLI.When state is“off”,thus outputting simple event information. |
| {on¦off} | on：enable off：disable |

【Example】

Example 1：Turn on event output detail function.

OLT(config)#event output detail on OLT(config)#

Enable or Disable Specified Event Output

| Command | OLT(config)#event output eventid <eventid> {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to permit or deny the output of the specified event.When state is“enable”,permitting the output of the specified event in the terminal;when state is“disable”,denying the output of the specified event in the terminal. |
| <eventid> | Event ID,the value range is 1-4294967294. |
| {enable¦disable} | Enable:turn on function Disable:turn off function |

【Example】

Example 1：Permit event 10001 output in the terminal.

OLT(config)#event output eventid 10001 enable OLT(config)#

Enable or Disable Specified Level Event Output

| Command | OLT(config)#event output eventlevel <eventlevel> {enable¦disable} |
| --- | --- |
| View | Config view |
| Description | This command is used to permit or deny the output of the specified level event.When state is“enable”,permitting the specified level event output in the terminal;when state is“disable”,denying the specified level event output in the terminal. |
| <eventlevel> | - Critical - Major - Minor - Warning |
| {enable¦disable} | Enable：turn on function Disable：turn off function |

【Example】

Example 1：Permit the event output of the level 3.

OLT(config)#event output eventlevel 3 enable OLT(config)#

Show All Event History Record

| Command | OLT(config)#show event history all |
| --- | --- |
| View | Config view |
| Description | This command is used to view all event history records. |

【Example】

Example 1：View all event history event records.

OLT(config)#show event history all EVENT 13 Warning 13002 2000-01-02 07:05:56 EVENT NAME:onu is offline INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:5 EVENT 12 Warning 13001 2000-01-02 07:05:43 EVENT NAME:onu is online INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:5

Show Specified Event History Record

| Command | OLT(config)#show event history eventid <eventid> |
| --- | --- |
| View | Config view |
| Description | This command is used to view history record of the specified event. |
| <eventid> | Event ID,the value range is 1-4294967294. |

【Example】

Example 1：View history record of event 13002.

OLT(config)#show event history eventid 13002 EVENT 14 Warning 13002 2000-01-03 05:27:27 EVENT NAME:onu is offline INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4 DESCRIPTION:E067B312118A00012770 EVENT 13 Warning 13002 2000-01-02 07:05:56 EVENT NAME:onu is offline INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:5 DESCRIPTION:E067B300000100012770

Show Specified Level Event History Record

| Command | OLT(config)#show event history eventlevel <eventlevel> |
| --- | --- |
| View | Config view |
| Description | This command is used to view history record of the specified level event. |
| <eventlevel> | - Critical - Major - Minor - Warning |

【Example】

Example 1：View history records of event level 4.

OLT(config)#show event history event level 4 EVENT 14 Warning 13002 2000-01-03 05:27:27 EVENT NAME:onu is offline INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4 DESCRIPTION:E067B312118A00012770 EVENT 13 Warning 13002 2000-01-02 07:05:56 EVENT NAME:onu is offline INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:5 DESCRIPTION:E067B300000100012770

Show Event History Record by Specified Board

| Command | OLT(config)#show event history eventparameter board <F/S> |
| --- | --- |
| View | Config view |
| Description | This command is used to view the history records of the specified board |
| <F/S> | Board number,the value is 0/0. |

【Example】

Example 1：View the event history records of the specified board 0/0.

OLT(config)#show event history eventparameter board 0/0 EVENT 14 Warning 13002 2000-01-03 05:27:27 EVENT NAME:onu is offline INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:4 DESCRIPTION:E067B312118A00012770 EVENT 13 Warning 13002 2000-01-02 07:05:56 EVENT NAME:onu is offline INSTANCE:FrameID:0,SlotID:0,PortID:1,OnuID:5 DESCRIPTION:E067B300000100012770

Show Event History Record by Specified GE Port

| Command | OLT(config)#show event history eventparameter ge <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to view the event history record of the specified GE port. |
| <F/S/P> | GE port number,the value range is 0/0/1–0/0/8. |

【Example】

Example 1：View the event history record of the specified ge8 port.

OLT(config)#show event history eventparameter ge 0/0/8 total number:0 OLT(config)#

Show Event History Record by Specified PON Port

| Command | OLT(config)#show event history eventparameter pon <F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to view the event history records of the specified PON port |
| <F/S/P> | Pon port number,the value range is 0/0/1–0/0/16. |

【Example】

Example 1：View the event history records of the pon1 port.

OLT(config)#show event history eventparameter pon 0/0/3 EVENT 7 Warning 13002 2000-01-02 02:22:51 EVENT NAME:onu is offline INSTANCE:FrameID:0,SlotID:0,PortID:3,OnuID:2 DESCRIPTION:E067B301010100012770 EVENT 6 Warning 13001 2000-01-02 02:22:40 EVENT NAME:onu is online INSTANCE:FrameID:0,SlotID:0,PortID:3,OnuID:2 DESCRIPTION:E067B301010100012708

Show Event History Record by Specified XGE Port

| Command | OLT(config)#show event history eventparameter xge<F/S/P> |
| --- | --- |
| View | Config view |
| Description | This command is used to view the event history records of the specified XGE port |
| <F/S/P> | Slot number,the value range is 0/0/1-0/0/2. |

【Example】

Example 1：View the event history records of xge1 port.

OLT(config)#show event history eventparameter xge 0/0/2 total number:0 OLT(config)#

Show Event Basic Information

| Command | OLT(config)#show event list |
| --- | --- |
| View | Config view |
| Description | This command is used to view the basic information of the event. |

【Example】

Example 1：view the basic information of the event

OLT(config)#show event list ---------------------------------------------------------------------------- EventId Output Level Def Level Name 10001 Yes Minor Warning The device reset 13001 Yes Warning Warning onu is online 13002 Yes Warning Warning onu is offline ---------------------------------------------------------------------------- OLT(config)#

Device Diagnostic Management

Ping Diagnostic Test

| Command | OLT(config)#ping {<destination-ip>¦<hostname>} |
| --- | --- |
| View | Any view |
| Description | This command is used for testing network accessibility between device and target host. |
| <destination-ip> | Destination IP address.There are five kinds of IP address,user can choose suitable IP address according to factual circumstance.Host IP address is not all 0 or 1,format for x.x.x.x. |
| <hostname> | Destination hostname |

【Example】

Example 1：Ping destination IP address 192.168.5.50

OLT(config)#ping 192.168.5.50 PING 192.168.5.50(192.168.5.50):56 data bytes 64 bytes from 192.168.5.50:seq=0 ttl=64 time=0.449 ms 64 bytes from 192.168.5.50:seq=1 ttl=64 time=0.379 ms 64 bytes from 192.168.5.50:seq=2 ttl=64 time=0.365 ms 64 bytes from 192.168.5.50:seq=3 ttl=64 time=0.612 ms ---192.168.5.50 ping statistics--- 4 packets transmitted,4 packets received,0%packet loss round-trip min/avg/max=0.365/0.451/0.612 ms OLT(config)#

  - 25.2 Traceroute Diagnostic Test

| Command | OLT(config)#traceroute {<destination-ip>¦<hostname>} hops <hops-id> timeout <time-id> ttl <ttl-id> |
| --- | --- |
| View | Any view |
| Description | Through traceroute,you can know about data packet transmission path from this host to the other end host |
| <Destination-IP> | Destination IP address.There are five kinds of IP address,user can choose suitable IP address according to factual circumstance.Host IP address is not all 0 or 1,format for x.x.x.x. |
| <Hostname> | Destination hostname |
| <hops-id> | Passed max router numbers that data messages arrive final host |
| <time-id> | Wait for the time(unit millisecond)specified by timeout for each response |
| <ttl-id> | Data packet Time-To-Live |

【Example】

Example 1：traceroute destination IP address 192.168.5.50

OLT(config)#traceroute 192.168.5.50 traceroute to 192.168.5.50(192.168.5.50),30 hops max,38 byte packets 1 192.168.5.50 0.954 ms 0.473 ms 0.189 ms OLT(config)#

- Appendix 1

The processing of messages in different VLAN modes is as follows:

| VLAN mode | Actions(in the inbound direction) |  | Actions(in the outbound direction) |
| --- | --- | --- | --- |
|  | Untagged frame | Tagged frame |  |
| Access | Tag the frame with the native VLAN tag. | - Drop the frame if its VLAN id is the same as the native VLAN id. Drop the frame if its VLAN id is different from the native VLAN id | Remove the native VLAN tag and send the frame |
| Trunk | Tag the frame with native VLAN tag. | - Receive the frame if its VLAN is carried on the port Drop the frame if its VLAN is not carried on the port. | - Send the frame and removing the tag if the frame is the same as native VLAN id. - Send the frame without removing the tag if its VLAN is carried on the port but is different from the native VLAN. |
| Hybrid |  |  | Send the frame if its VLAN is carried on the port.The frame is sent with the VLAN tag removed or intact depending on your configuration with the VLAN hybrid command. |

- Concluding Remarks

Thanks for using products of Shenzhen C-Data Technology Co. Ltd.

Contact Information:

Company Address: Room 601, Floor 6, Building F, Songbai Road 1008, Sunshine Community，Xili Street, Nanshan District, Shenzhen, China

Factory Address: 1st floor, Building B, Wentao Industrial Park, Yingrenshi Community, Shiyan Avenue, Baoan District, Shenzhen, China

Telephone: 0755-26014509/26014710/26014711

Fax: 0755-26014506

Email: Marketing@cdatatec.com

Website: www.cdatatec.com

www.cdatatec.com.cn